*********** Eplay ***********

Version 2.0 07/22/90

Public Domain

The STe audio sample player

(via the STe DMA soundchip)

By: AAron nAAs

AAron@sun.soe.clarkson.edu

*****************************

To use Eplay effectively you should understand at least "a little"

about how the STe DMA sound chip works.  The info given to it stays in

it until it is intentionally changed.  Among the information the chip

can be given, are the playback RATE (6.25/12.5/25/50 kHz.), Mono/Stereo

MODE and the location of the SAMPLE.  And of course how to PLAY the

sample, Once/Loop/Stop.  Master Volume, Left/Right Channel Volume,

Treble, Bass and Mix are also settable.  So if you play a sample with

certain parameters, then play another sample without specifying new

parameters, the old settings will still be used ("remembered").

The sample memory is a collection of contiguous Words (two bytes) where

each byte is a Signed number (from -128 to 127) which represents the

speaker displacement, where 0 is "neutral."  If the chip is in stereo

mode, the upper byte of each word is played through the left speaker,

and the lower byte through the right speaker.  So, to play a sample only

through one speaker, the sample must be loaded into every other byte.

**********

There are many ways a sample can be stored, one way (a very popular

format for ST audio samples), is that each byte is an UNSIGNED number.

Eplay can translate these bytes to Signed numbers while loading, so that

they playback correctly.

Another problem arises when you try to play a sound that wasn't sampled

in one of the four playback rates the chip supports.  There is more to

the sample conversion than just skipping or stuffing bytes to adjust

the playback rate (It wouldn't sound right).  I will be working on a

progam that recalculates samples from one sample rate to another sample

rate.  So, until then, if you want to play a sound that was sampled at

the popular 10kHz or 15kHz rates, you have to live with chosing a

sample playback rate that is "close" to the rate you want.

**********

ALSO remember that each time you run a program, it is most likely that

all free memory is cleared.  So if you start a sample looping, it will

continue playing correctly until you run another program (where the

sample will be blanked, and it will end up playing whatever is put

there), unless you protect the sample when you load it.  Of course if

you protect a sample you loose some free memory (more precisely the

amount just about equals the size of the sample plus the size of the

player, even though its only the sample you wanted to protect).

**********

Usage:

**********

(Rename eplay.tos to eplay.ttp if you want to pass parameters from the desktop)

Examples (all examples are to be considered independant of each other,

and note that the input is not case sensative, and the order

of the parameters does not matter):

1)       eplay sample1.spl

(will play sample1.spl at 12kHz, in mono

mode, once, which are defaults)

2)       eplay -lu sample1.spl -r0 -ms -pl

(will play sample1.spl at 6.25kHz, in stereo

mode, looping continuously.  It will continue

playing the memory where the sample is/was,

and if something happens to the memory where

the sample is/was, it ends up playing whatever

happens to end up in that location.  This won't

hurt anything, it will just end up sounding

really wierd.

The 'u' after the "-l" causes the sample to be

translating from Unsigned bytes.)

eplay -ps

(Stops the sample playing)

3)       eplay -lu sample1.spl -r3 -ms -pl -k

(will do what the above did, but the "-k"

informs the computer to "Keep" the memory

so it is not corrupted in the future.

Sample1.spl will continue to play again, and

again, and again, ...)

eplay -ps

(Stops the sample playing)

eplay -pl

or               (Starts it looping again)

eplay -po

(Plays it one more time)

4)       eplay -lu sample1.spl -r3 -mm -pl -k

eplay -r2

(slows loaded sample to 24.5 kHz)

eplay -ms

(switches play Mode to Stereo, which since it

is playing two bytes at a time, effectively

doubles the playback rate)

eplay -po

or                (Lets the sample play to completion then stops)

eplay -ps

(Stops the sample immediately)

5)       eplay -lru sample1.spl -r0 -pl -k

(Loads sample1.spl into only the Right speaker,

translating Unsigned, and plays at 6.25 kHz.,

and "Keeps" it's memory.  We note the locations

that eplay loaded the sample into, and refer

to them as 0xStart and 0xEnd in the next line.

The program saw "-lr", and assumed stereo Mode)

eplay -llu sample2.spl -s 0xStart 0xEnd

(Loads sample2.spl into only the Left speaker,

translating Unsigned, and loads as much as it

can into the memory protected by the above line.

Note the memory has already been protected, so

there is no need to protect it again.)

(Result: sample1.spl playing from right speaker,

sample2.spl playing from left speaker, over,

and over, and over ... at 6.25 kHz.)

eplay -ps

(Stops the sample playing)

(Try something like this!!)

6)       eplay -lru sample1.spl -r1 -k -pl

(Noting 0xStart and 0xEnd again, now add 0x1000

to 0xStart and call it 0xStart2 and put our

values into the next line.)

eplay -llu sample1.spl -s 0xStart2 0xEnd

(You should end up with the whole sample playing

in the right ear, and in the left ear, the same

sample lagging behind by 0x1000 bytes, and

missing the last 0x1000 bytes, pretty weird

sounding!)

eplay -ps

(Stops the sample playing)

**********

Final words:

Have fun, and remember that if you use the "-k" option, you don't get your

free memory back!!!

And, you can play memory ANYWHERE in the computer with the "-s" option,

but if you start loading samples into memory that you haven't protected,

(Kept with "-k"), you could crash your computer or corrupt other programs!

But hey, just turn the computer off, and start playing again!

**********

Possession stuff:

As stated previously, I am releasing this into the public domain, though I

reserve all rights to the executable and the sourcecode.  Copying and

distributing this package AS A WHOLE is not only permitted, but encouraged.

If profit can be made using this package, or ANY part of it, an agreement

must be made with me before any profit can be collected.

Address:                       E-Mail:

AAron nAAs                    AAron@sun.soe.clarkson.edu

8 W. Elizabeth St.

Skaneateles, NY (USA)

13152

**********