How to Implement Fine Scrolling on the STE.

The purpose of this document is to describe how to use the capabilities of the STE to achieve bit-wise fine-scrolling and vertical split screens. Horizontal and vertical scrolling are discussed and an example program is provided. Split screen effects are discussed and an example program with multiple independent scrolling regions is provided.

Three new registers are provided to implement fine-scrolling and split screen displays:

1) HSCROLL - This register contains the pixel scroll offset. If it is zero, this is the same as an ordinary ST. If it is non-zero, it indicates which data bits constitute the first pixel from the first word of data. That is, the leftmost displayed pixel is selected from the first data word(s) of a given line by this register.

2) LINEWID - This register indicates the number of extra words of data (beyond that required by an ordinary ST at the same resolution) which represent a single display line. If it is zero, this is the same as an ordinary ST. If it is non-zero, that many additonal words of data will constitute a single video line (thus allowing virtual screens wider than the displayed screen). CAUTION - In fact, this register contains the word offset which the display processor will add to the  video display address to point to the next line. If you are actively scrolling (HSCROLL <> 0), this register should contain the additional width of a display line minus one data fetch (in low resolution one data fetch would be four words, one word for monochrome, etc.).

3) VBASELO - This register contains the low-order byte of the video display base address. It can be altered at any time and will affect the next display processor data fetch. It is recommended that the video display address be altered only during vertical and horizontal blanking or display garbage may result.

These registers, when used in combination, can provide several video effects. In this document we will discuss only fine-scrolling and split-screen displays.

Fine Scrolling:

Many games use horizontal and vertical scrolling techniques to provide virtual playfields which are larger than a single screen. We will first discuss vertical scrolling (line-wise), then horizontal scrolling (pixel-wise) and finally the example program "neowall.s" which combines both.

Vertical Scrolling: To scroll line-wise, we simply alter the video display address by one line each time we wish to scroll one line. This is done at vertical blank interrupt time by writing to the three eight-bit video display address registers to define a twenty-four-bit pointer into memory. Naturally, additional data must be available to be displayed. We might imagine this as a tall, skinny screen which we are opening a window onto for the user. The video display address registers define where this window will start.

Horizontal Scrolling: To scroll horizontally we might also adjust the video display address. If that was all we did, we would find that the screen would jump sideways in sixteen pixel increments. To achieve smooth pixel-wise scrolling we must use the HSCROLL register to select where within each sixteen pixel block we wish to start displaying data to the screen. Finally, we must adjust the LINEWID register to reflect both the fact that each line of video data is wider than a single display line and any display processor fetch incurred by a non-zero value of HSCROLL. All this is done at vertical blank interrupt time. Naturally, additional data must be available to be

displayed. We might imagine this as an extremely wide screen which we are opening a window onto for the user. These registers define where this window will start.

For Example: The program "neowall.s" reads in nine NEOchromeø picture files, organizes them into a three by three grid and allows the user to scroll both horizontally and vertically over the images. The heart of this program (the only interesting thing about it actually) is the vertical blank interrupt server. This routine first determines the pixel offset and loads it into HSCROLL. The LINEWID register is now set to indicate that each virtual line is three times longer than the actual display width. If we are actively scrolling, this amount is reduced to reflect the additional four-plane data fetch which will be caused by the scrolling. Finally the video display address is computed to designate a window onto the grid of pictures. This twenty-four-bit address determines where the upper-left corner of the displayed region begins in memory. Thus, every frame an arbitrary portion of the total image is selected for display. The speed and resolution of this scrolling technique is limited only by the dexterity of the user.

Split Screen:

In many applications it is desirable to subdivide the screen into several independent regions. On the STE you may reload some video registers on a line-by-line basis (using horizontal blanking interrupts) to split the screen vertically into multiple independent regions. A single screen no longer need be a contiguous block of storage, but could be composed of dozens of strips which might reside in memory in any order. The same data could be repeated on one or more display lines. Individual regions might each have their own individual data and scrolling directions.

For Example: The program "hscroll.s" reads in a NEOchromeø picture file and duplicates each line of the image. This, combined with the proper use of LINEWID, effectively places two copies of the same picture side-by-side. Next, both vertical and horizontal blanking interrupt vectors are captured and the horizontal blanking interrupt is enabled in counter mode. To prevent flicker caused by keyboard input, the IKBD/MIDI interrupt priority is lowered below that of the HBL interrupt. Note that the program 'main loop' doesn't even call the BIOS to check the keyboard, since the BIOS sets the IPL up and causes flicker by locking out horizontal interrupts - this may cause trouble for programs in the real world. The screen is effectively divided into ten regions which scroll independently of one another. There are two ten-element arrays which contain the base address of each region and its current scroll offset. At vertical blank interrupt time we compute the final display values for each region in advance and store them into a third array. We then initialize the display processor for the first region and request an interrupt every twenty lines (actually every twenty horizontal blankings). During each horizontal interrupt service, we quickly reload the video display address registers and the HSCROLL register. This must be done immediately - before the display processor has time to start the current line or garbage may result. Note that horizontal blank interrupts are triggered by the display processor having finished reading the previous data line. You have approximately 144 machine cycles to reload the HSCROLL and video display registers before they will be used again by the display processor. Finally, the LINEWID register is set, this need only be done before the processor finishes reading the data for the current display line. We then pre-compute the data we will need for the next horizontal interrupt to shave few more cycles off the critical path and exit.