The purpose of this document is to describe how to use the capabilities

of the STE to achieve bit-wise fine-scrolling and vertical split screens. Horizontal and vertical scrolling are discussed and an example program is provided. Split screen effects

are discussed and an example program with multiple independent

scrolling regions is provided.

Three new registers are provided to implement fine-scrolling and split

screen displays:

1) HSCROLL - This register contains the pixel scroll offset.

If it is zero, this is the same as an ordinary ST.

If it is non-zero, it indicates which data bits constitute the first

pixel from the first word of data. That is, the leftmost displayed p

xel is selected from the first data word(s) of a given line by this

register.

2) LINEWID - This register indicates the number of extra words

of data (beyond that required by an ordinary ST at the same resolution)

which represent a single display line. If it is zero, this is the same

as an ordinary ST. If it is non-zero, that

any additonal words of data will constitute a single video line

(thus allowing virtual screens wider than the displayed screen).

CAUTION - In fact, this register contains the word offset which the

display processor will add to the  video display address

to point to the next line. If you are actively scrolling (HSCROLL <> 0)

this register should contain the additional width of a display line

minus one data fetch (in low resolution one data fetch would be four

words, one word for monochrome, etc.)

3) VBASELO - This register contains the low-order byte of the

video display base address. It can be altered at any time and will

affect the next display processor data fetch. It is recommended that

the video display address be altered only during ve

tical and horizontal blanking or display garbage may result.

These registers, when used in combination, can provide several

video effects. In this document we will discuss only fine-scrolling

and split-screen displays.

Fine Scrolling:

Many games use horizontal and vertical scrolling techniques to provide

virtual playfields which are larger than a single screen.

We will first discuss vertical scrolling (line-wise),

then horizontal scrolling (pixel-wise) and finally the example prog

am "neowall.s" which combines both.

Vertical Scrolling: To scroll line-wise, we simply alter the video

display address by one line each time we wish to scroll one line.

This is done at vertical blank interrupt time by writing to the three

eight-bit video display address registers to d

fine a twenty-four-bit pointer into memory. Naturally, additional

data must be available to be displayed. We might imagine this as a tall,

skinny screen which we are opening a window onto for the user. The video

display address registers define where

this window will start.

Horizontal Scrolling: To scroll horizontally we might also adjust the

video display address. If that was all we did, we would find that the

screen would jump sideways in sixteen pixel increments. To achieve smooth

pixel-wise scrolling we must use th

HSCROLL register to select where within each sixteen pixel block we wish

to start displaying data to the screen. Finally, we must adjust the

LINEWID register to reflect both the fact that each line of video data

is wider than a single display line a

d any display processor fetch incurred by a non-zero value of HSCROLL.

All this is done at vertical blank interrupt time. Naturally, additional

data must be available to be displayed. We might imagine this as an

extremely wide screen which we are ope

ing a window onto for the user. These registers define where this window

will start.

For Example: The program "neowall.s" reads in nine NEOchrome picture

files, organizes them into a three by three grid and allows the user to

scroll both horizontally and vertically over the images. The heart of this

program (the only interesting th

ng about it actually) is the vertical blank interrupt server.

This routine first determines the pixel offset and loads it into HSCROLL.

The LINEWID register is now set to indicate that each virtual line is three

times longer than the actual display width

If we are actively scrolling, this amount is reduced to reflect the

additional four-plane data fetch which will be caused by the scrolling.

Finally the video display address is computed to designate a window

onto the grid of pictures. This twent

-four-bit address determines where the upper-left corner of the displayed

region begins in memory. Thus, every frame an arbitrary portion of the

total image is selected for display. The speed and resolution of this

scrolling technique is limited only

by the dexterity of the user.

Split Screen:

In many applications it is desirable to subdivide the screen into

several independent regions. On the STE you may reload some video

registers on a line-by-line basis (using horizontal blanking interrupts) to

split the screen vertically into multiple

ndependent regions. A single screen no longer need be a contiguous block of

storage, but could be composed of dozens of strips which might reside in

memory in any order. The same data could be repeated on one or more display

lines. Individual regions

might each have their own individual data and scrolling directions.

For Example: The program "hscroll.s" reads in a NEOchrome picture file

and duplicates each line of the image. This, combined with the proper use

of LINEWID, effectively places two copies of the same picture side-by-side.

Next, both vertical and hor

zontal blanking interrupt vectors are captured and the horizontal blanking

interrupt is enabled in counter mode. To prevent flicker caused by keyboard

input, the IKBD/MIDI interrupt priority is lowered below that of the HBL

interrupt. Note that the p

ogram 'main loop' doesn't even call the BIOS to check the keyboard, since

the BIOS sets the IPL up and causes flicker by locking out horizontal

interrupts - this may cause trouble for programs in the real world.

The screen is effectively divided into

ten regions which scroll independently of one another. There are two

ten-element arrays which contain the base address of each region and its

current scroll offset. At vertical blank interrupt time we compute the

final display values for each region

n advance and store them into a third array. We then initialize the display

processor for the first region and request an interrupt every twenty lines

(actually every twenty horizontal blankings). During each horizontal

interrupt service, we quickly

eload the video display address registers and the HSCROLL register.

This must be done immediately - before the display processor has time to

start the current line or garbage may result. Note that horizontal blank

interrupts are triggered by the disp

ay processor having finished reading the previous data line. You have

approximately 144 machine cycles to reload the HSCROLL and video display

registers before they will be used again by the display processor.

Finally, the LINEWID register is set, th

s need only be done before the processor finishes reading the data for

the current display line. We then pre-compute the data we will need for

the next horizontal interrupt to shave few more cycles off the critical

path and exit.

ne immediately - before the display processor has time to start the

current line or garbage