*****************************************************************

This  is the third in a series of tutorials that I hope  will  be

found  to  be  useful  to  both  new  and  experienced  users  of

communications  facilities.

*****************************************************************

Q:  Why is it that I experience so much more line noise than  the

people I call?  It seems that I see noise on my screen with  some

frequency,  but if I ask the party that I have called if he  sees

it too, I'm usually told his screen is clean.  Is there something

wrong with my system?

A:  The odds are twice as great that you will have line noise  if

you  place a call to a computer than if a computer were  to  call

you.  It is normal and easily explainable.

While  it is true that the odds are twice as great that you  will

experience  or  know  about  noise in the  case  where  you  have

initiated the call, the incidence of noise is the same regardless

of who places that call (assuming the same lines and circuits are

being used in both cases).  The reason for this is that when  you

are  in Terminal mode (placing the call), your system is  set  to

full-duplex operation and when it is in Host mode (auto  answer),

it is in half duplex.

Full  duplex means that whatever you type on your  keyboard  does

not  get  sent  to  your screen.  It is  sent,  instead,  to  the

communications port and from there it travels through your modem,

along  the telephone lines to an answering modem, and then  to  a

host sytem.  The host system then sends it back to you.  In  half

duplex, on the other hand, whatever you type is sent to both your

communications port and to your screen.  From this it is  obvious

that  every character seen on your screen when you have placed  a

call  has  gone through the telephone system while only  half  of

what  is  seen  on  the host system's  screen  has  been  on  the

telephone circuit before it got there.

Further,  line  noise  can be unidirectional.  That  is,  it  may

appear  as  data  travels in only one  direction  or  the  other.

Regardless  of  that fact, it will be seen by the  terminal  mode

user (data must go both ways before it reaches the screen) and if

it appears only on the link from the host to the terminal user it

will never be seen by the host.

Q:  The last tutorial you wrote told us about MNP and ARQ  modems

being able to eliminate most line noise.  How do they do that?

A:  Part of that answer is still a mystery to me, but I know  how

it  does it in theory at least.  I will tell you why part of  the

answer  remains  a  mystery  in  a  moment.   First,  recall  the

discussion  we  had about file transfer protocols.  All  of  them

utilize  some form of CRC mechanism to insure that the  receiving

system  had  received  all  of  the  contents  of  a  packet   of

information  without  having dropped any bits or  picked  up  any

extra  bits.   The CRC is a byte or a word of data  that  is  the

result of an algoritm that 'folds' every byte in the data  packet

onto itself in such a way as to result in a pattern of bits  that

can be calculated by the receiving system as each byte of data is

received  and  then compared with the CRC  that  is  subsequently

received.  If there is a mismatch then the data (or CRC byte) did

not  get  received correctly.  The MNP and ARQ  modems  implement

this  strategy within themselves.  All data that  is  transmitted

from  one  of  these modems is re-packaged into  what  the  modem

manufacturers  call 'frames' (packets) before being  transmitted.

Each frame is followed by a CRC byte or word that is stripped off

by  the  receiving modem and used to determine if the  frame  was

received correctly.  Line noise simply makes that CRC check  fail

and the result is an automatic retransmission of the frame.

As you can see from the above, the modem is now acting just  like

your  computer  does during file transmissions using  a  protocol

transfer  method.  This is not done for 'free'.  The overhead  of

doing  so results in less than rated speeds in every case.   That

is, the theoretical maximum data rate of a 1200 baud modem is 120

characters  per second, but MNP and ARQ modems are  sending  more

characters between themselves than the sending system itself.  If

there  are  errors and, thus, an automatic  retransmission  of  a

frame,  the  sending  modem is very likely to  have  to  ask  the

sending  computer  to  wait for it.  It is  estimated  that  this

overhead (even without errors) results in a degradation of  about

12%  in  terms of the maximum possible performance of  the  modem

yielding about 106 characters per second possible throughput.  To

counter that built in degradation, the modems strip the start and

stop bits from each byte and send only 8 bits rather than the  10

(or  eleven) that are sent by non-error-correcting modems.   This

increases  the efficiency by about 20%.  The net effect is  that,

assuming  no  errors,  the possibility of  about  108%  of  rated

performance.   (It  is possible to get about 130  characters  per

second  rather than 120 if there are no errors - this also  fails

to  account for additional 'compression' methods built into  some

of these modems).

So,  where is the confusion?  Well, the above assumes there is  a

stream  of data being sent that can be 'framed'.  How the  modems

function  when a user is merely typing one or two  characters  or

words  at  a time before the other side responds  is  a  mystery.

Indeed,  as  each  character  is  typed  it  is  sent  down-line.

Presumably there is a timeout of some kind in the modem that says

that if another character is not entered within x milliseconds it

is presumed that the frame is complete and it is sent along  with

its  CRC.   However it does it in practice, it does  seem  to  be

effective at eliminating line noise.

Q:   So MNP and ARQ modems are faster and eliminate  line  noise.

Sounds  like  the way to go.  Are there any  negatives  to  their

usage?

A:   Interesting  question.   Assuming  that  you  use   protocol

transfer   methods  in  addition  to  the  error  detection   and

correction logic of the modems themselves, I can only think of  a

couple of negatives at the moment.  The first, of course, is  the

lack of standards, particularly at the higher baud rates.  Second

is  the  fact that every time you use one to call a  system  that

does  not use MNP or ARQ (the vast majority of them do not)  then

you automatically lose part of their opening screens.

Let  me  explain that.  When an MNP or ARQ modem  first  connects

with  another modem the calling modem issues a sequence of  bytes

that  is  asking the answering modem if it is also  MNP  or  ARQ.

These bytes include an id and an indication of the level of  MNP,

for  example,  that  the  caller is  using.   The  first  set  of

characters that come back from the called modem are then consumed

by  the  modem rather than passed through to the  user's  screen.

Thus,  they are lost to your system.  Very often it is  necessary

for  the calling system user to press his Enter key in  order  to

cause  subsequent  characters  to be  passed  through  the  modem

(telling  it  in  effect, to turn off MNP or ARQ).   This  is  an

annoyance  to the terminal mode user but it can be worse for  the

host system.

With the introduction of release 12.20 of GT PowerComm there  has

been  some controversy as to the existence of the opening  prompt

that  it issues in which it asks if the caller wants to use  ANSI

graphics  or  not.   Many users seem mildly  annoyed  that  their

selection is not recorded somewhere so they don't have to  answer

that prompt more than once.  What they fail to understand is that

the  prompt is there for several reasons.  MNP is a good  example

of what I mean as is the possibility of noise on the line.

When  an  MNP  call comes in, those  initial  characters  I  just

mentioned 'hit' the prompt and result in reissuance of it.  We do

not permit a default to that prompt so that we do not go past  it

with  noise  or MNP.  By the time a Y or N is  entered,  the  MNP

sequence  of  handshake signalling is done.  If we did  not  have

that  initial  prompt then the first question the user  would  be

asked  would be his first name.  Ask any Sysop how  many  garbage

names  he  has  in his user base.  If there are any  then  I  can

reasonably  assure  you that his system does not have  a  leading

prompt such as ours to protect him from noisy incoming calls  (or

MNP).

Q:  Is 9600 baud the theoretical limit to technology in terms  of

modems?

A:    Hardly.   It  appears  that  9600  'baud'   stretches   the

reliability limits of today's unconditioned telephone system, but

modems  exist  that  are much, much  faster  than  that  already.

19,200 bits per second modems are functional on conditioned lines

even  now.   As  to limits, well, did  you  know  that  satellite

communications   capabilities  exist  that  already  permit   the

transfer of over a million bits per second?

Over  the past 20 years there has been a rather constant rate  of

improvemnt  in all aspects of data processing technology.   As  a

rule  of  thumb that is pretty close consider this:   Every  four

years   there   has   been   a   three   fold   improvement    in

performance/capacity  for  only  a two fold  increase  in  price.

Sometimes  we forget how long this trend has been in effect,  but

an  IBM advertisement a few years back made it pretty clear.   At

that  time the ad suggested that if the automobile  industry  had

enjoyed the same rate of improvements over the past 20 years that

the  data  processing industry has enjoyed, then every  adult  in

this country could afford to own a Rolls Royce, as it would  cost

only  about  $20  and, incidently, it would get  about  2,000,000

miles  to  the  gallon  of gasoline.   For  a  more  contemporary

example,  we  need only look back at the original IBM  PC.   That

machine  had  320K disk drives and a clock speed  of  4.77  micro

seconds.  Today you can buy a Compaq 386 that is 17 times  faster

than  the  original PC (throughput) and you can get  it  off  the

shelf  with  130  megabyte hard disk.  The price  of  this  newer

machine is less than three times the original PC, closer to twice

the  price.  No, we are not at the limit of technology, not by  a

long shot.

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X