1990/09

Making a 1.44 meg write on 720k's

By David Reynolds

The purpose of this file is to allow a 1.44 meg 3.5" disk

drive to read from and write to 720k floppies at 1.44 meg.  This

is a hardware solution, and I'm currently working on a software

version, give me some time...

First, some history, and techy-type stuff:

In the beginning, there were 8 inch disks.  They were single

sided, and had a relatively low storage capacity (by todays stand

ards.)  Then, someone came along and decided that they'd add a

second head to these disk drives.  But, in order for this to

work, they had to also polish the other side of the diskette in

the manufacturing process.  Thus, two different disk formats were

born.

Now, as motor-speed tolerances improved and stepper motors

got finer and finer step, the engineers decided that they would

be able to get more capacity out of a drive.  This was fine in

theory, but in practice, there just wasn't enough particles on

the diskette to do this.  The solution was to pack the metal

oxide tighter, and Quad-Density floppies were invented.

So, with 3 different formats available, and the manufactur

ing setup to handle this, it seemed reasonable to continue this

through the 5.25" floppy.  But, they already had the oxide packed

tightly enough, so they just polish the 5.25" floppies out part

way on a 360k disk.  That's why you CAN format a 360k to 1.2meg,

you just get errors by the dozen after 400+.

With the 3.5" disks, though, they had to completely revamp

the manufacturing process.  The oxide density just wasn't great

enough.  So it wasn't really feasible to manufacture both par

tially and fully polished floppies in this size.

The problem is;  the manufactures are GREEDY.  They decided

that it would be easy to include a hole in the 3.5" high-density

(1.44 meg) media, and the info to read that hole on the 1.44 meg

disk.

The result?  Your cheap 720k disks are JUST AS HIGH A QUALI

TY AS THOSE EXPENSIVE 1.44's!!!  They just don't have the stupid

hole in them.

You can just take a hammer and nail, and knock a hole in

them, but this is messy and unreliable (though it does work if

you measure carefully.)

A better solution?  Modify your drive.

Here's how the drive works:

The drive has a physical controller on it.  This handles

motor step, speed, etc.  It also decides if the disk is 1.44 or

720 by reading the hole.  WITH A MICRO-SWITCH!

That's right, the whole thing (hole thing) is based on whether or

not that little switch is up (720) or down in the hole (1.44)

The solution:

READ THIS !!! __________________________________________________

PROCEDURE CAN SCREW-UP YOUR DRIVE!!!   IF YOU DON'T KNOW HOW TO

USE A SOLDERING IRON, STOP NOW!!!

Ok, I assume you all know what you are doing. . . ..

Remove the disk drive from the computer, MAKING SURE THE MA

CHINE IS OFF!

Remove the front panel on the drive.

You should be able to locate (usually on the left side) a small

black switch, looking something like this:   /------\

|      |

\-|  |-/

||

\/

Now, unsolder the switch, and solder two wires in it's place.

Solder an on-off switch (single pole, single throw) on the wire.

Drill a hole in the front panel and mount the switch.

Replace the front panel on the drive (careful that the wires

don't interfere with any moving parts!)

Remount the drive.

With the cover off (for trouble-shooting), boot the computer,

from the hard disk or from the 5.25" floppy, then put a blank

720k disk in the 1.44 meg drive.  Place the switch in the up

position, and format the disk.  If it formats to 720k, place the

switch in the down position, and try again.  Label the 1.44 meg

and 720k position.

If it still doesn't work, either trouble-shoot your work, or

solder back in the switch and forget about it.

If someone gives you a disk formatted on a 720k disk drive,

your drive can no longer auto-detect this.  So, if it's 720, you

need to switch the switch to 720k.  Simple.  If you don't, you'll

get a "General Failure" error.  Just flip the switch and retry.

Same for 1.44 megs...

Hope this has been helpful!

If you have any questions, I'm:

David -*- Captain Midnight -*- Reynolds

-*- Chris Knight -*-

-*- DCKR -*-

OR ON THE NET AS:

chip@seeker.uucp

uunet!seeker!chip

decwrl.com!vixie!seeker!chip

chip!seeker!prememos@pacbell.com