The A-Max circuit diagram included was traced from the actual A-Max

cartridge and nobody has been keen enough to build it yet. But I am 99.99%

sure that this is the correct circuit and it was rechecked several times

and seems to fit in nicely with the way the A-Max software accesses the

drive port.

The circuit should be able to be simplified a fair bit if you only

want to use a Mac drive and are not interested in putting in the Rom

sockets.

CYA

P.S	The circuit is an IFF pic displayable in Deluxe Paint or Dutils etc.			A-MAX Version 1.0

ROM VERSIONS:

The Rom loading code does recognise the type of Roms being read,

so if you want to use 64K roms (for some reason) then all you will have

to do is to replace the Roms file with a copy of the 64K Roms.

BOOT BLOCK:

The boot block on this disk write enables the Kickstart ram on

a 1000 so that A-Max can use the extra 256K.

If the boot block gets corrupted then use the FixBootBlock

program.

HARD DRIVE INSTALLATION:

To install A-Max and the Roms onto a hard drive type:

MakeDir DH0:A-Max

Copy A-Max:A-Max to DH0:A-Max all

Copy A-Max:Roms to DH0:A-MAx

To tell A-Max where to find the Roms file, add the following

line to the Startup-Sequence:

Assign A-Max: dh0:A-Max

NOTE: If you have a 1000 and wish to use the kickstart ram for

A-Max then you will have to boot of the A-Max disk to write enable the

ram (or copy the Boot-Block).

STARTUP PREFERENCES

1) Video Options:

The standard Mac screen size is 512x342. Wide screens are

640 and High screens are 400 lines (512 PAL).

The "Video mode" gadget controls how the Mac screen is displayed.

Interlaced allows the whole screen to be displayed. "ECS" (extended chip

set) allows up to 480 lines to be displayed flicker-free. "ECS lace"

displays 960 lines interlaced. "A2024", enables 1008x800 video mode for

the A2024 full page moniter.

Slow scroll and fast scroll scrolls the screen according to the

position of the pointer. Paged will swap which portion of the screen is

being displayed (when using this mode use the right mouse button to

display the opposite portion of the screen).

The colors button specifies the two colours used by A-Max.

Default will use the A-Max default colour and system will use the first

two colours from the preferences set up (Amiga).

2) Memory Options:

No Expansion - Use only contiguos memory from location 0.

No $C00000   - Use all memory below $C00000 (disables extra

512K on A500 and A2000).

User defined - Allows you to select the amount of memory to use.

(any ram not selected is used for the Ram drive)

3) Serial/Parallel and imagewriter Emulation options:

The Mac has two serial ports and no parallel port, one is known

as the "modem port" and the other as the "printer port". There are

two gadgets to control how they are emulated.

The "port" gadget controls whether the output for the mac

serial port comes out of the amiga's parallel or serial ports. The

usual choice is to have the modem port (Port A) data coming out of

the amiga's serial port, and the printer port (Port B) coming out

of the parallel port. Serial rates are between 110 & 19200 baud.

The second gadget controls what sort of printer emulation

happens at each output. "None" means the output passes through unchanged,

this is usualy used on the serial (Comms) port or with an Apple

ImageWriter or LaserWriter connected to the Amiga. The remaining options

enable emulation of the Imagewriter printer if you have an Epson

9 or 24 pin dot matrix printer connected to your Amiga.

4) Save will save the preferences to a file in the current directory.

5) Go A-Max will load the Roms from disk (not cartridge).

BOOT SCREEN

-The Mac can boot of any startup disk in any drive.

-Press the Right Mouse button for a preferences requestor.

A-MAX OPERATIONS

1)Keyboard differences:

A-Max emulates a Mac plus keyboard which has a numeric

keypad and arrow keys. There are three keys which are not on

the Amiga keyboard and are emulated by:

Command	- either Amiga Key

Option	- either Alt Key

clear	- Del Key

For programmers, the Mac interrupt switch is shift-ESC.

2)Disk Eject:

The Mac has a software eject (no hardware eject !@#).

A disk can be ejected upon request by the user. You should NEVER

therefore eject a disk unless A-Max is allowing you to. The Mac

knowing that you can't eject a disk doesn't update the buffers to

disk immediately it waits until you wish to eject the disk before

dumping the buffers and closing all files.

A-Max indicates that a disk is ready to be ejected by displaying

its drive number in the right hand corner (df0 = 1 ).

If you eject a disk without the number showing A-Max will flash

the drive number telling you to reinsert the disk.

To request to eject a disk you can:

- Select the disks icon and choose Eject in the menu

(or press Amiga-E).

- Use the Eject option in most dialog boxes.

- Use th Finder Shutdown option.

3)Finder Shutdown:

On Finder versions 5.3 or less the shutdown option will crash

the Amiga. Finder versions 5.4 or greater there is a Shutdown and a

restart option. The shutdown option will work, but the restart option

will not. You can also use the "shutdown" program.

4)Mouse Buttons:

The Mac has a single mouse button which is the Amiga's

left button.

5)Disk Drives:

A-Max can use the Amiga's drives as well as an Apple 800k

drive connected to the cartridge. A-Max will allow you to use the

following formats in the Amiga drives:

- A-Max format; read,write & format

- Magic Sac/Specter 128(s-s-ST); Read only.

- Mini transfer disks, created on a Mac with the

disk transfer software; read only.

6) Sound:

Only 'beep' supported.

7)Ram Disk:

Any memory that you are not using (as selected with the

"Memory size" gadget) will be used for a Ram disk. The disk will

survive reboots and can be booted of if it contains the correct files.

The Ram disk can be 'inserted' and 'ejected' using the F1 key.

The first time this is done you will have to format the disk.

To boot of a Ram disk press the F1 key at the boot screen.

8)Clock:

A-Max will use the date and time from the Amiga clock if you

have an A2000 or A500 with the 512K memory expansion.

SYSTEM DISKS

A-Max supports all known Apple system disks, however versions

3.2 or greater are recommended.

If you are using the Apple Imagewriter or LaserWriter printer

emulation in A-Max, or a PostScript laser printer, you will also need

the printer drivers - "ImageWriter" for 9 pin graphics, "LQ ImageWriter"

for 24 pin, "LaserWriter" and "Laser Prep" for postscript printers.

EXPANSION MEMORY

A-Max can support all of the Amiga's ram however the mac always

has it's memory starting from location 0 and is limited to 4MB, whereas

the Amiga supports up to 9MB of RAM which can be discontinuos. A-Max

will therefore function fine with the first 512K but some applications

will not function with extra memory. The solution is to use the "Memory

size" gadget to set the amount of ram to 512K.

If you have the new chip set with 1Mb of chip ram you will be

able to have a 1MB memory size.

MultiFinder (apples multitasking system) can also help as

it controls memory in a way compatible with A-Max.

TRANSFERING SOFTWARE

NOTE: The Mac drive has five different speeds of which 2 are

readable on the amiga (this gives 272K of spaces readable by both systems.)

An Apple external drive is recommended as files can be copied

directly between A-Max and Apple disks.

The disk transfer sorftware allows you to copy disks without

an external drive however, you will need access to a Mac to produce

the transfer disks. The software formats disks on the Mac that are

readable by A-Max. These disk have a capacity of 272K so disks can

be copied in several chunks.

1)Mini Transfer disks:

This provides a quick method of copying data less than 272K.

Step A: On a Mac execute the Disk transfer software

on the "Utilities disk". Click the "Make mini

transfer disk" button and insert a blank disk.

Step B: Quit the program and copy the files to the Mini

transfer disk.

Step C: Execute the Disk transfer program again, and click

on the "Prepare mini transfer disk" button.

The disk will now be readable by A-Max and you can copy the files

to an A-Max format disk.

2)Full Disk Transfer:

This allows a complete 800K Mac to A-MAX copy storing data on several

transfer disks. Once created A-Max reads the data from the original source

disk and the created disks to create one A-Max format disk.

Click on "Full disk transfer to begin". After copying, run the

"Disk Reciever" program under A-Max (on the utilities disk). The click

on the "Recieve full disk" button and the full disk will be reconstructed.

3)AmigaDos file transfer:

A-Max uses a format called FTF (file transfer format) to transfer

files between the Amiga and A-Max. This transfer must be done with text

or data files only.

To transfer files from Amiga to A-Max run the "File transfer" program

on this disk and format a disk using the format gadget. Click on the "To TD"

and select the file you wish to transfer. In A-Max execute the "File

transfer" program from the utilities disk and select the "from TD" gadget.

---

Firstly, I know everyone hates a cracker with a conscience but

if you want to use A-Max seriously it is really worth buying the hardware

and an Apple drive.

If you do have the hardware and an apple drive then this version

of A-Max will still recognise the drive and will load a hell of a lot

faster than from the cartridge.

The utilities disk mentioned above is in two partions, one that can

be read by a Mac and the other which can be read by A-Max. I am currently

trying to get a copy of this disk in complete A-Max format so that you

can at least have the option to copy the Mac half back to Mac format (if

you know someone with an apple drive).	You will find most of the utilities

disk in the system draw on the SuperPaint disk.

I'm also trying to get a comms program so that software can be

downloaded from a Mac, but until then the only way to copy disks is to use

an external apple drive. These disks will be available soon at a pirate near

you.

Disks Available:

1) SuperPaint (with utilities)

2) Mac 3D

3) Print shop

4) MacDraw/MacPaint/MacWrite

I haven't tried too hard but I couldn't find an Amiga copier

to copy A-Max format disks, So all the above disks will have to be copied

in A-Max by dragging the Icons.

If you find a copier that will do it put in a message below.

M-U-Later

INSANE

__

///   ...And remember where you found this first :

__   ///

\\\ ///           T H E   C R U S A D E R S   B B S

\\///

\//   For our HeadQuarter board, dial (Norway) - 6 - 810881

or 810864

For our Golden Gate board, dial (Norway) - 42 - 46718