AMIGA ACCELERATOR SYSTEM

This is a project of an accelerator that I made in the Summer 1990.

Look at this project like a shareware thing, if you build this accelerator

and works well, please send me 10$-15$ so I can buy a 68020/30 (I'm a

student) and make a low cost accelerator based on it without pals.

It uses a 68000/16 and few other components. To use this accelerator

you have to remove the 68000 from its socket and use the signals of

it, plus one, CDAC, that you can find in the expansion slot

(coprocessor slot in A2000) on pin 15.

In the net list I called U6 the socket of old 68000 like if it was

a real IC, but they are only contact points with the Amiga motherboard.

U1 is the new 68000. I used a 16MHz one, but I suppose that this

circuit works well also with a 12.5MHz, because it'll run at 14.32MHz

(try it and make me know).

In the picture, the signals from the left side (green) are connected

to the 68000 socket and the other from the right side (red) with the

new microprocessor. There is also the CDAC signal (black) that you

must take from the motherboard or you can create using the 7M

(the standard 7MHz processor clock) and delaying it of 35ns with

a delay line (it is a 90 degrees phase shifted clock).

You can see that there are three main sections in the pic. The first one

( U4a ) prepares the 14MHz clock for the 68000 XORing 7M with CDAC,

the second ( U2 + U3a ) delays the dtack signal for a correct

bus cycle timing and the third (U5 + U3b) makes the E signal (at 0.7MHz)

and synchronizes properly the microprocessor with it to do a regular cycle

when it uses the 8520.

Use the net list to build the board and don't yell to me if something

goes wrong. Be patient and alert, don't make it, if this is your

first experience with a soldering iron.

If you want to know something more about this project E-mail me

at livio@alessia.dei.unipd.it (USENET) or write at my physical address:

Livio Plos

via Amalteo 15

33170 Pordenone

Italy

Comments are welcomed.

Remember that I don't assume any responsibility on the accuracy,

performance or reliability of this board and its documentation.

Thanks to my friends Marco Pontil and Andrea Gottardo that supported me.