=========================================================================

Another original file presented by Sine Wave.

Sysop of The Grey Zone BBS 205-774-7453

=========================================================================

ALL-BAND TRANSMIT MODIFICATIONS

During a widespread emergency, it may become necessary to transmit on

specific frequencies to signal for help or to pass emergency radio

traffic.  These frequencies might be monitored by the Civil Air Patrol,

the National Guard, United States Coast Guard, the military, or other

public safety agencies.

The well-equipped survival station should have complete access to

receive all frequencies on the shortwave band, as well as many

frequencies in the VHF and UHF spectrum.  Modern worldwide amateur radio

epuipment not only tunes in the ham bands, but also allows for general

coverage receive, and with modification, general coverage transmit.

Modern VHF and UHF ham radio walkie-talkies also allow for VHF and UHF

public safety band receive as well as public safety band transmit with

the proper modification.

Before we get into modification details, let me first point out that

using frequencies without FCC authorization could result in stiff

penalties.  This could include imprisonment, a $10,000 fine for every

day you are transmitting on the air, and the confiscation of your

equipment.  The only time you are allowed to use frequencies that you

are not authorized is in a real emergency when safety of life or

protection of property is an immediate threat.  Possessing equipment

that has been modified and is capable of transmitting outside of normal

ham radio band limits is not necessarily illegal--but just as soon as

you press the microphone in a non-emergency situation outside of your

authorized frequencies, this indeed is not allowed.  Just remenber that

operating on frequencies without specific FCC authorization is simply

not legal except in an emergency.

The popular Yaesu 757 high frequency transceiver is easily modified

for all-band transmit.  Simply locate the tiny white switch that is

hiding under a wire bundle inside the top front of the set near the

frequency selector mechanism.  Simply slide the switch in the opposite

direction, and your transmitter is now unlocked.

Yaesu 2-meter hand-helds are not capable of going beyond their normal

2-meter range, well outside of normal ham or MARS limits.  You cannot

extend it into the 150MHz region.

The ICOM 02-AT 2-meter hand-held and the Kenwood 2600 hand-held will

both go into the 150 MHz range if some modifications are made.

On the Kenwood 2600, locate the two diodes standing on end with

Teflon covering the top part of their leads (which at one time were

soldered together, broken, and then resoldered).  Simply clip these two

leads to unlock the transmitter, but be sure to reset the microprocessor

by pushing the reset button before the set is capable of transmitting on

any frequency at 150 MHz.

On the ICOM 02-AT, as well as on the ICOM 04-AT for VHF and/or UHF

work, both units require semi-major diode surgery for frequency

expansion.  For VHF, two diodes are removed and three are added for the

ICOM 02-AT to extend all the way up to 165 MHz.  For UHF work, two

diodes are added to extend the range of the ICOM 04-AT.  The

modification is tricky and a bit complicated.  (Ten dollars to Radio

School, 2414 College Drive, Costa Mesa, California 92626, brings you all

the instructions plus all the diodes needed for either the VHF or UHF

mod.)

The popular Kenwood TS430S is modified for all-band high frequency

transceive operation by unplugging the 3-wire plug on the RF board that

mates with socket 10.  When looking at the front top of the radio, it's

to the left front side right beside a larger white plug.  You'll need to

carefully examine the circuit board and locate the number 10 witha

circle areound it to insure you have the right plug.

On the brand new ICOM 735, to unlock the transmitter you simply cut

diodes D33 and D34 that are standing on end near the microprocessor

section toward the rear top of the radio.

On the ICOM IC-745, locate the RF board on the side of the radio and

cut the light brown wire at jack 7, which is going to pin 1.  Your

transmitter is now unlocked.

On the IC-751, locate the RF board on the side of the radio and cut

the black wire going to pin1 of jack 2.  Your transmitter is now

unlocked.

On the big Kenwood 940 base station, locate IC number 109.  Now find

diode 130 and cut it for all-band transmit.  If you want just MARS

coverage, locate IC 111 and 112, and snip diode 135 beside it.

On the Kenwood 930, the all-band transmit modification is a bit more

lengthy--several jumpers are required which is beyond the scope of this

article. Perhaps if response is good i'll prepare an article on that

specific radio.

None of the 2-meter mobile sets are easily converted to 150 meg

frequencies.  Many people have tried all sorts of modifications, but

the best they can get is 149.99--and nothing higher.

The ICOM thumbwheel VHF set, IC 2AT, can also be modified for 150

MHz transceive; but once you make the mod, you lose all of your 2-meter

frequencies.  You would be better to simply go out and buy a

type-accepted VHF transceiver as opposed to ruining your present 2-meter

ICOM IC 2AT set!

The older high frequency radios with VFO tuning (as opposed to PLL

tuning) do not easily allow any type of modification for expanded

transmit or receive coverage.  It would require so much modification

that you would be ill-advised to try and step up any Yaesu FT101 series,

980, Kenwood 120, Kenwood 520, 820, or any of the old ICOM sets.

For the old ICOM 720, the transmitter is unlocked by snipping the

blue wire that's at the very end of the top hatch cover to the left

middle side of the tranceiver.

On the ICOM M-700 marine transceiver, the sets are already capable of

working on ham radio frequencies.  Despite what the literature says, no

lock-out is employed because the factory already did their diode trick.

Simply punch in the frequencies and you're on the air.  Remember, any

ham frequency on 40, 80, or 160 meters requires the mode switch to be

put to the hard left positions of "A3," which enables the lower sideband

filter.  All other frequencies to include all marine frequencies are

upper sideband--including those marine frequencies at 2, 4, 6, and 8

MHz.  All marine is upper sideband.

The well-trained survivalist will know the radio frequency band well

to include radio operating procedures for different services.  This will

assist you if it becomes necessary to signal a radio call on these bands

in an emergency.

Today's modern ham radio equipment gives us complete access to roam

the airwaves and eavesdrop on almost any type of radio communications

that are out there, on the air.  CRB Research, P.O.Box 56, Commack, NY

11725, offers numerous confidential frequency lists of those agencies

you may wish to tune into with your general coverage amateur radio

transceiver to gain more information about local emergencies and local

happenings.  Write for their big free catalog.

While it's not illegal to modify amateur radio equipment for all-band

transmit, it is illegal to use it if it's not an emergency.  While you

may possess a valid citizen's band radio license, using non-type

accepted equipment on these bands is also not allowed, except in an

emergency.

Although you may possess a marine radiotelephone license or an

aeronautical license, using non-type accepted epuipment is also not

allowed, except in an emergency.

Take your time when modifying equipment, and use modified equipment

outside of your regular band limits only in an emergency situation.

Enjoy.....

Sine Wave

�������������������������������������������������������������������ͻ

�   Downloaded from The Grey Zone BBS 205-774-7453 19,200 bps       �

�������������������������������������������������������������������͹

�  VGA graphics, Printer utilities, CAD programs, Space/Aviation,   �

�  Artificial Intelligence, Assemblers, Home control via computer,  �

�  New Technology, Cellular phones, Radio Hobbies, Satellite TVRO,  �

�  War Dialers, Hack/Phreak, Phrack magazine, Survival/Homesteading,�

�  Surveillance, Trojan Horse protection, Cryptography, and more... �

�������������������������������������������������������������������ͼ

��������������������������������������������������������������������

ݰ     The CORPORATE HQ of SHAWN-DA-LAY BOY PRODUCTIONS, inc.     ��

ݰ             Tfile Distribution Center / MASS Megs              ��

ݰ 415/236/2371          RoR - Alucard               415/236/2371 ��

ݰ Dr. Murdock ~ Sir Death ~ Dark Nite ~ RatSnatcher ~ Pressed Rat��

ݰShawn-Da-Lay Boy Production Inc. Rat Head Systems : 415/524/3649��

ݰ����       The Gates of Hell are open Night and Day;        ������

ݱ��   �     Smooth is the Descent and Easy is the Way      �   ����

��������������������������������������������������������������������

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Salted Slug Systems                      Strange              408-454-9368

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Tomorrow's 0rder of Magnitude            Finger_Man           408-961-9315

My Dog Bit Jesus                         Suzanne D'Fault      510-658-8078

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X