Amiga 500 processor accelerator project

by Leslie Ayling

p.o. box 334

GLEBE 2037

Welcome once again to cheapskates corner, where we try and buy the

world for a song. This time around we shall increase the speed of the

processor to just over 14Mhz, double the standard speed, for under $25

!!!!

You will need the following -

1. an MC68000p12A 16MHz processor. These are available in Sydney from

VSI electronics, 16 DICKSON ave ARTARMON 2064 ph (02)4398622  cost is

$18.90 plus tax ($22.68 tax inc.) 2. a 74F74 D type flip flop. Geoff

wood at Lane Cove or Rod Irving should keep them. 3. a few short

lengths of hook-up wire. 4. a SPDT switch.

Refer to the accompanying circuit diagram Note: Keep the leads to the

switch as SHORT as possible. The circuit is relatively straight

forward. the 74F74 is wired as a positive edge triggered divide by two

circuit, which provides the alternate clock frequency of 14.18Mhz

approxiamately. The old 68000 8Mhz processor is removed and replaced

with the 16Mhz version which has is clock pin (15) bent out

horizontally so we can connect our new circuitry to it. The SPDT

switch either connects the normal 7Mhz signal or the new 14Mhz signal

to the 68000's clock input.

The switch can be changed while the machine is on, however it will

simply lock up.Just reset as usual from the keyboard once you have

changed speed.

I have a revision 5 board with new 1.3 roms which cope with the new

speed quite happily. Using the program CRAMDEN-SI from MEGADISC 10 to

compare the difference the mod makes : the standard position gives-

performance relative to amiga 1000 : 1.0

performance relative to ibm pc/xt : 3.2

On high speed the performance is-

performance relative to amiga 1000 : 1.7

performance relative to ibm pc/xt  : 5.2

so roughly a 70% increase in performance. Even though the clock speed

is doubled to the processor, the performance dosen't increase to 2.0

because the agnus and the other support chips are still running at the

same speed as before thus holding the processor up for a slightly

higher percentage of the total time than before.

This mod basically gives the same capability as the CMI accelerator

board priced at $399 in sydney, with the exception that it isn't

software controlled. However when it's only costing 1/16th the price

one cant really complain!!.

If you have any queries or would like the mod installed in your

machine, either contact me direct or via Megadisc.

The machine should work on the 1000 and 2000 however the pin numbers

may be different.I suggest you compare the a500 schematics with that

of your machine and make note of the differences.

Hope this is of some help...........regards   Leslie Ayling.

#addendum# One machine this mod was carried out on had trouble with

the one of the disk drives at high speed. It appeared to be trying to

step the head of the internal drive faster than the drive could

physically move and refused to boot.When I connected a driv e with a

5ms track-track access time and booted off of it the problem

dissappeared. I have a patch for this problem for normal DOS disks to

slow down the step rate which sits in the boot sector.It may be a

problem however for protected software or non-standard boot sector

type programs.Don't despair however the patch is very small and should

fit in somewhere anyway.Just drop me a line. If your machine exhibits

the problem, the patch disk is available DIRECT from me with various

usefull disk speedup utils and a Trackdisk step-rate alteration

program, plus loads of other priceless information on gurus, reset

program flow, diagnostics all on an ARP1.3 system disk for the

princely sum of $5 !! There should be no problem with HARD disks

however if anyone encounters any please let me know........thanks.