Dirt Cheap Frame Grabber

by Michael Day

as of 22 December 1991

Public Domain

This device is free and released to the public domain.

If you actually find a use for it, all the more power to ya!

The purpose of the DCFG is to simply provide a very simple method

to grab video pictures and display them on a computer monitor.

The DCFG requires at least a 80286 or better and a VGA

display to run. While it is possible to change the program to

allow operation on a 8086 and/or a different display, I'm not

doing it. If you want to, you can. My intent was to just prove

that it was possible. I'm not taking it any farther at this

point.

The frame grabber program at this point is very simple, and as a

result has some problems. The heart of the program relies on a

very fast IO port to memory instruction. It can operate as fast

as the computer can shove data from the port to the memory. This

is as fast as DMAing the data could do.  This method is sensitive

to computer speed. As such, you may not get a stable picture at

first. If you don't, use the left or right arrow keys to adjust

the horizontal sample size. Since this was written for a 33Mhz

486, you will probably have to use the left arrow to reduce the

sample size. The up/down arrows adjust the vertical sample size,

but this normally doesn't need to be done.

You will probably notice some sparkles (pixels shifting left and

right one pixel position). This is caused by sample jitter and is

normal. All sampling systems have this problem. Some less than

others. Even your TV has the problem, but it is normally good

enough that you don't see it.

The jitter is caused by the fact that the sampler is not

perfectly synchronized to the incoming video signal. Your TV has

a lot of circuitry in it to achieve the synchronization. Being

that this is a dirt cheap frame grabber, there is only limited

synchronization of the picture, and no synchronization of the

sampler. If you want to try to improve on it, please do.

To use the Dirt Cheap Frame Grabber, plug it into a PC printer

port, and attach the other end to the video output from a VCR or

TV Monitor and run the program. Use the left or right arrow keys

to adjust the horizontal sample size to obtain a stable picture.

Dirt Cheap Frame Grabber

(DB25P)

DO0 ---680ohm-------------*------->|------*

pin 2                     |       red     |

|       led     |    * = connection

DO1 ---680ohm-------*-----*               |

pin 3               |     |               |    ) = no connect

*--> 2Kohm (trimmer)  |       (jump over)

|               |

DO2 ----------*-----------)------| |------*

pin 4         |           |     .1uf      |    IC1 = LM339

3| / |6      |               |          quad

DC3           / - |-------*               |        comparitor

(slct)    1 /     |       |               |

DS6 -------| IC1a |      220ohm           |

(ack)       \     |7      |               |

pins 10,17    \ + |---*   |               |

\ |   |   |               |

|   |               |

/ |4  |   |               |

DC2           / - |---)---*               |

(init)    2 /     |   |   |               |

DS5 -------| IC1b |   |  180ohm           |

(pe)        \     |5  |   |               |

pins 12,16    \ + |---*   |               |

\ |   |   |               |

|   |               |

/ |10 |   |               |

DC1           / - |---)---*               |

(afd)    13 /     |   |   |               |

DS4 -------| IC1c |   |  330ohm           |

(slct)      \     |11 |   |               |

pins 13,14    \ + |---*   |               |

\ |   |   |               |

|   |               |

/ |8  |   |               |

DC0           / - |---)---*               |

(stb)    14 /     |   |   |               |

DS7 -------| IC1d |   |  120ohm           |

(busy)      \     |9  |   |               |

pins 11,1     \ + |---*   |               |

12| \ |   |   |               |

|       |   |               |

Gnd ----------*-------)---*---------------*------*--------*

pins                  |   |               |      |        |

18,19,20,21,          |  100Kohm          |     75ohm     |

22,23,24,25           |   |      .1uf     |      |        |

*---*-------| |-----)------*------o )

2N2222 |                   |             RCA

*---------C   E                   |            phono

|           B                     |            jack

|           |      1N914          |

DO3 ----*--4.7Kohm--*-------->|-----------*

pin 4

Dirt Cheap Frame Grabber

Circuit Description

It is assumed that the video signal will be a NTSC type signal

complying to the RS170A video standard with negative going sync

and a signal level between 0.5Vpp to 2Vpp (1Vpp desired). This is

the typical signal that is available on the video output jack of

a VCR or TV monitor.

The video input comes in on the RCA phono jack. The 75ohm

resistor provides the proper load for the connection. The .1mf

capacitor isolates the signal from the rest of the circuit and is

used as a part of the level shifter to clamp the video input to

ground reference on the computer. The 1N914 diode and 4.7K ohm

resistor provides a voltage reference 0.6V above ground for the

2N2222 clamp transistor. The transistor and diode get their

supply voltage from the DO3 output pin. Typically this is around

3.3volts and should have enough power for our needs assuming that

the output driver is a 74LS373 driver which is the standard part

for a PC printer port.

The transistor provides a low impedence charge path for the

capacitor to clamp the sync tip to ground so as to insure the

video signal sync tip is referenced to ground. The 100K ohm

resistor provides a high resistance discharge path for the

capacitor. Enough to provide a discharge path for the capacitor,

but not enough to seriously affect the video signal level.

The clamped video signal is then applied to all four comparitors

in the LM339. The LM339 gets its power from the DO2 output pin

Like the clamp transitor, normally there should be enough power

from the data output pin to supply the IC. A .1mf capacitor is

used to stablize the power source. You can probably getaway

without using the capacitor, but I'm paranoid.

The four comparitors recieve their reference voltage from a

resistor ladder consisting of 120ohm, 330ohm, 180ohm, and 220ohm

resistors in series with the 120ohm resistor tied to the common

ground. The voltage level on the 120ohm resistor provides the

sync detector reference.  The other three resistor positions

provide three increasing levels of amplitude detection.

The resistor ladder requires a regulated source voltage with an

adjustment to allow compensation for various input signal levels

depending on the signal source. The 2Kohm trimmer pot provides

for the bias adjustment which allows the circuit to be adapted to

the absolute signal source level. The regulated source voltage is

derived from the red LED. A red LED will have a typical voltage

drop of about 1.7 volts. The absolute level is not important

since the trimmer can compensate for minor differences. The LED

gets its power through the two 680ohm resistors attached to the

D0 and D1 outputs. Two outputs are used here to insure that there

is enough power to drive the LED and the resistor ladder while

maintaining  a  regulated  voltage source  across  the  LED.  The

comparitors receive their pullups from the control lines.

When wiring the circuit, remember that you are working with video

signals. That means you are working with signals in the RF range

(up to 4MHz). Keep all wiring as short as possible and the

components close together. All grounds should be tied to one

single point at the 25pin connector.

If you are good at this stuff, it is possible to wire the whole

circuit inside the connector shell (don't use a shielded shell,

it will short out the circuitry). It's a very tight fit, but if

you pick a large shell, and are vary careful, it can be all

stuffed in there. The trimmer pot should be made available for

easy adjustment (I drilled a hole in the shell for access).

Also, the led should be visible to be able to quickly determine

if the circuit is working.

Simply plug the connector into a printer port on the PC, and run

the frame grabber program. When running the program, the LED

should turn on. That indicates that the circuit has enough power

to operate. If the LED does not come on, or it flickers, there is

not enough power, and you can't use that printer port with the

circuit. Try another port or a different computer.

The resulting detected output from the port is as follows:

76543210

1000xxxx  sync

0000xxxx  black

0001xxxx  dark gray

0011xxxx  gray

0111xxxx  white

Note that bits 0-3 are unknown and should not be relied on as

being valid.

-----------------------------------------------------------------

Program Description

The program works by grabbing samples from the printer port in

chunks. Once each time through the loop, it sucks in a frame from

the video input.  Actually, it sucks in more than a frame (more

than two frames). We do this because we don't know where in the

video data stream we are, so we have to grab enough data to

insure that we get one complete frame. That means grabing

something more than two video frames (a video frame is 1/60

second of data). Since I'm lazy, I just set the loop to grab

30,000 data points. Typically that will grab about 45ms of data

on a standard buss. Two video frames would be 33.3ms. If you have

a faster data buss, then it will grab 37.5ms of data. If you are

using a very fast buss such as 12Mhz or more, then you may have

to increase the sample count size. The maximum sample size

allowed is 65520. More than that and we run out of segment space

in memory. Also during this period I keep the system interrupts

off so as to not have distortions of the collected data caused by

the system deciding to go off and do something.

Additionally, I pre-configure the data by waiting in

the sample loop for the vertical sync pulse. You can determine

the vertical sync by watching the sync line. When it goes on for

longer than 10us, you are in the vertical sync time. Less then

10us, and it is a horizontal sync pulse.

Waiting for the vertical sync allows me to not have to search for

it later since I know that the frame will now start at the

beginning of my collected data. Hmm, that means that I could

reduce the collection array size since I know where I'm starting

from now. Oh well... You can play with that if you want. It works

the way it is right now.

Once I get the data, I throw it on the display by first checking

for a starting horz sync pulse. I then display the data for the

given X size of program. In order to increase the size of the

picture, I duplicate each pixel twice. This gives a wider piture

than we would otherwise have. At the 1.5us collection rate, we

can only get about 40 usable pictures alomng the horizontal scan

line.

Next, I step down several scan lines and repeat the mess. I skip

several scan lines so as to maintain the aspect ratio. Since we

can pick up all the scan lines, that means we are getting 262

scan lines for each frame. That is way too many compared to the

40 or so horizontal pixels. So we throw a few of them away to

even things up.

I repeat this until I run into the program Y limit. At that point

I go collect another frame from the printer port. Repeating until

a return or Esc key is pressed.

-----------------------------------------------------------------

Parts List

1ea  DB25P connector

1ea  RCA phono jack

1ea  2K ohm trimmer pot

2ea  680 ohm resistor

1ea  220 ohm resistor

1ea  330 ohm resistor

1ea  180 ohm resistor

1ea  120 ohm resistor

1ea  100K ohm resistor

1ea  75 ohm resistor

1ea  4.7K ohm resistor

2ea  0.1uF capacitor

1ea  red LED

1ea  1N914 diode

1ea  2N2222 transistor (NPN)

1ea  LM339 quad comparitor

That's all folks!

<eof>

##################################