FAXPHONE 25

and

FAX-225

RS-232

Serial Interface

Technical Information

FAXPHONE 25/FAX-225 Serial Interface                             Page: 1

1. INTRODUCTION

This  document  describes  the  serial  interface   of   a   CANON

FAXPHONE-25 or FAX-225 (they  are  essentially  the  same  unit)  FAX

machine, and my experiences with it. The majority of the  information

presented here is a summary from the CANON document  titled  "RS-232C

INTERFACE KIT FOR FAXPHONE 25/FAX-225  -  TECHNICAL  SPECIFICATIONS".

This document bears the following message, which should be considered

to apply to that information:

---------------------------------------------------------------------

NOTICE

This manual provides information necessary for users to  operate  the

Faxphone 25  or  FAX-225  as  a  remote  fax  by  using  the  RS-232C

interface.

This manual relates to the use of the Canon Faxphone 25  and  FAX-225

only. No software programming, its design or implementation should be

considered part of this product. the end-user is  solely  responsible

for obtaining all required software programming, design,  coding  and

installation of which Canon shall have no responsibility whatsoever.

Canon shall not be liable for loss of  profit  or  other  incidental,

consequential or special damages arising out the use or inability  to

use  the  forgoing  produce  with  respect  to  the  information   or

programming samples contained in this manual.

(Specifications  contained  herein  are  subject  to  change  without

notice).

---------------------------------------------------------------------

The  same  goes  for  me  also.  In  other  words,  YOU  USE  THIS

INFORMATION ENTIRELY AT YOUR OWN RISK!!!

Please treat this document and acompanying software as a  kind  of

SHAREWARE... I ask no monetary compensation for this effort,  however

if you write software to control the FAX, create new fonts, or create

any other works from this information that do nifty things  with  the

FAXPHONE 25/FAX-225 RS-232 port, I'd like you to send me copy.

Dave Dunfield

Dunfield Development Systems

P.O. Box 31044

Nepean, Ontario (Canada)

K2B 8S8

FAXPHONE 25/FAX-225 Serial Interface                             Page: 2

2. SERIAL INTERFACE

The FAXPHONE 25/FAX-225 features a simple three wire RS-232 serial

interface for remote operation.

Mode:       Asynchronous

Speed:      4800 or 9600 baud

Start bits: 1

Data bits:  8

Parity:     None

Stop bits:  1

2.1 Connector Pinout

The  FAXPHONE  25/FAX-225  features  a  small  5-pin   circular

connector which is located directly behind the handset jack. On my

FAXPHONE 25, this connector was covered by a small  plastic  plate

which had to be removed.

The easiest way to connect to this interface is with  a  serial

cable available from CANON. This  cable  converts  to  a  standard

DB-25 female connector which can  be  directly  attached  to  most

PC's.

If you do not have this cable, the pinout for the FAX connector

is as follows:

______

/      \

/  1    2  \

|  3  []  4  |

\      5   /

\______/

Pin     DB-25       Name

1        2         Transmit Data (TXD)

2        3         Receive  Data (RXD)

3        7         Signal Ground

4        1         Frame Ground

5        1         Frame Ground

The CANON cable also ties pin 4(RTS) to  pin  5(CTS),  and  pin

6(DSR) to pin 20(DTR) on the DB-25 connector.

NOTE: For FAXPHONE 25, you should check the ROM revision number

which is indicated on a round label affixed to the bottom  of  the

unit (This is a small round sticker containing a number only).  If

the number is 01-09, CANON requests that you contact  them  before

attempting to  use  the  RS-232  interface.  My  FAXPHONE  25  has

revision 11, and it appears to work OK.

FAXPHONE 25/FAX-225 Serial Interface                             Page: 3

2.2 Activating the serial interface

To enable the RS-232 interface, you must turn on DIP switch  #4

on the bottom of the FAX unit. (My FAXPHONE 25  has  the  switches

set to 0101 0000).

To set the serial speed (baud rate):

1.  Press [FUNCTION] and [4] to enter USER DATA

2.  Press [0] until you see the message prompt

"RS-232C BAUD RATE?".

3.  Press [1] to active this function. The display

will show the currently configured baud rate.

4.  Press [0] to toggle to the next baud rate

(4800/9600). Press [1] when the speed you

want is displayed.

To enter RS-232 Remote  Mode  (and  accept  commands  from  the

serial port):

1.  Press [FUNCTION] and [4] to enter USER DATA

2.  Press [0] until you see the message prompt

"RS-232C Remove Mode?".

3.  Press [1] to active remote mode.

The IN USE indicator will light.

4.  To cancel Remote Mode, press the red STOP button.

FAXPHONE 25/FAX-225 Serial Interface                             Page: 4

3. SERIAL FAX COMMANDS

The FAXPHONE  25/FAX-225  accepts  and  acts  upon  commands  when

operating in its RS-232  Remote  mode.  Most  useful  functions  will

require several of these commands to perform.

The FAX has a short  timeout  for  receiving  commands  and  their

responses, which makes it difficult to manually enter  the  commands.

In most cases, you will need to use a program to send and  coordinate

the proper sequence of commands to accomplish a particular function.

Although the command code appear to be simple parameter lists, the

CANON documents do not identify the individual  value  meanings,  but

define only the following command strings:

3.1 Load Font

Loads a text font into the  FAX  machine.  This  must  be  done

before any operations involving text are performed.

Send:       CC=010704/CD=01/FN=00//

Receive:    ACK

Send:       DT=<font data>EOP

Receive:    ACK

<font data> consists of 96  character  entries  each  of  which

consists of 48 bytes, for a total of 4608 bytes of data.

Each character is 16 bits wide, by 24 bits high. Each  pair  of

bytes in an entry represent one line, beginning at the top of  the

character.

3.2 Load Text

Loads a text file into the FAX machine, for later  printing  or

transmission. You must load a font prior to using this command.

Send:       CC=010704/CD=00/FN=10//

Receive:    ACK

Send:       DT=<text data>EOP

Receive:    ACK

<text data> consists of ASCII text  (character  values  between

SPACE 0x20 and DEL 0x7F only). Each line terminates with  CARRIAGE

RETURN (0x0D) and LINE FEED (0x0A), each page ends with FORM  FEED

(0x0C).

A maximum of 100K of text may be loaded at any one time.

Maximum line length is 100 characters.

FAXPHONE 25/FAX-225 Serial Interface                             Page: 5

3.3 Transfer Image

Transfers a compressed image (using MH method) to or  from  the

FAX machine.

Send:       CC=010704/CD=03/FN=15//

Receive:    ACK

Send:       DT=<image data>[MPS or EOP]

Receive:    ACK

-- OR --

Send:       CC=010407/CD=03/FN=15//

Receive:    ACK

Receive:    DT=<image data>[MPS or EOP]

Send:       ACK

<image data> is compressed using the MH method. MH data is sent

out byte-by-byte in the CCITT T4 format. MH data is compressed  in

Huffman's code. Thus, the last byte of  a  line  does  not  always

contain 8 bits. If the last byte does not contain 8  bits,  insert

'0' in the remaining bits of the byte.

To indicate the beginning of a page,  sent  the  following  EOL

code before the data: [0x00, 0x80]

To indicate the end of a line, add the following  EOL  code  to

the end of each line: [0x00, 0x80]

To indicate the end of a page, add the following  RTC  code  to

the last line (9 bytes): [0x00,  0x08,  0x80,  0x00,  0x08,  0x80,

0x00, 0x08, 0x80]

Use MPS to indicate more  pages  are  to  follow.  Use  EOP  to

terminate the last page. Up to 108K of image data may be loaded.

[DT=<1st page>RTC][MPS][.. ..<last page>RTC][EOP]

__/          \___________________

/                                 \

[EOL<1st line>EOL...<last line>EOL]

FAXPHONE 25/FAX-225 Serial Interface                             Page: 6

3.4 Load Destination

Loads the FAX with 16 identifiers  which  indicate  speed  call

keys to be used as the destinations for transmitted FAX's.

Send:       CC=010704/CD=00/FN=40//

Receive:    ACK

Send:       DT=<destination data>EOP

Receive:    ACK

<destination data> consists of  16  two  bytes  pairs,  plus  a

trailing 0xCC byte for a total of 33 bytes.

The first byte of each pair indicates the type  of  speed  call

key, the second indicates the actual key to be used:

Byte 1           Byte 2

(X)              (Y)

One touch spd dial:  0x00    0-9=0x00-0x09, A-C=0x0A-0x0C, #=0x0E, *=0x0F

Coded speed dial:    0x01    01-66=0x00-0x66  (BCD format)

No destination       0xCC    0xCC

[DT=<destination data>EOP]

__/                  \_______________________

/                                             \

[X1][Y1][X2][Y2] ... [X15][Y15][X16][Y16][0xCC]

FAXPHONE 25/FAX-225 Serial Interface                             Page: 7

3.5 Transfer Telephone

Transfers telephone number information to or from the FAX. This

command loads/queries the ONE TOUCH and CODED speed call numbers.

Send:       CC=010704/CD=00/FN=20//

Receive:    ACK

Send:       DT=<tel data>EOP

Receive:    ACK

-- OR --

Send:       CC=010407/CD=00/FN=20//

Receive:    ACK

Receive:    DT=<tel data>EOP

Send:       ACK

<tel data> consists of several groups, totaling 1371 bytes.

The first group (192 bytes) is ONE TOUCH SPEED DIALING FOR FAX.

It consists of 12 entries (For keys 0-9, # and *), each  of  which

begins with 0x00, followed by 15 bytes containing up to 30  digits

(in BCD) format. Use 0x0C for  any  digits  which  are  not  used,

0x0E=Pause, and 0x0F= Space.

If the A,B and C keys are configured for ONE TOUCH DIALING, the

second group consists of three more entries (as above) except that

each entry is 33 bytes long (0x00 + 32 bytes).

If the A,B and C keys are configured  for  GROUP  DIALING,  the

second group consists of 3 entries, each of which contains 16  X-Y

pairs (See LOAD DESTINATION), and a trailing 0xCC byte  (total  99

bytes).

The third group (6 bytes) contains the configuration of the  A,

B and C keys. The first three bytes contains the type for each key

(0x00=Speed dial, 0x01=Group dial). the second three bytes conains

the number of group  dialing  destinations  registered  under  the

corresponding key.

The forth group (15 bytes) is used to identify  keys  used  for

RELAY or CONFIDENTIAL document  sending  for  the  one-touch  keys

(0-9, #, *, A-C). Bit 7 of each byte indicates RELAY sending,  and

bit 6 indicates CONFIDENTIAL sending for each respective key.

The fifth group  (15  bytes)  contains  information  about  the

confidential mailbox numbers of the one-touch  keys  (0-9,  #,  *,

A-C), used for sending confidential documents. Each entry contains

the mailbox number for the respective key  (in  BCD  format).  Two

bytes of zero follow this data

FAXPHONE 25/FAX-225 Serial Interface                             Page: 8

The sixth group (240 bytes), is ONE  TOUCH  SPEED  DIALING  FOR

VOICE. It consists of 15 entries, each of which begins with  0x00,

followed by 15 bytes containing up to 30 BCD digits. The format is

the same as for the first group listed in this section.

The seventh group (800 bytes) contains 50 entries for the CODED

SPEED DIALING keys. Each entry begins with 0x00,  followed  by  15

bytes containing up to 30 BCD digits. Format is the  same  as  the

first group listed in this section.

FAXPHONE 25/FAX-225 Serial Interface                             Page: 9

3.6 Transfer Special Telephone

Loads telephone number information into "special"  Coded  speed

dial numbers (51-66) which are only accessable through the  RS-232

port. These make good "scratchpad" locations  to  store  telephone

numbers that are being specified from the serial port.

Send:       CC=010704/CD=00/FN=50//

Receive:    ACK

Send:       DT=<tel data>EOP

Receive:    ACK

<tel data> consists of 16 entries, each of which is 16 bytes

long (total 256 bytes). Each entry begins with 0x00, followed by 15

bytes containing up to 30 digits (in BCD) format. Use 0x0C for any

digits which are not used, 0x0E=Pause, and 0x0F=Space.

Note that the "memory protect" switch must be OFF to be able

to load the telephone numbers.

FAXPHONE 25/FAX-225 Serial Interface                             Page: 10

3.7 Print Text

Prints a previously loaded text file on the FAX printer.

Send:       CC=010402/CD=00/FN=10//

Receive:    ACK

3.8 Print Image

Prints a previously loaded FAX image on the FAX printer.

Send:       CC=010402/CD=03/FN=15//

Receive:    ACK

3.9 Send Text

Sends  a  previously  loaded  text  file  to  the  destinations

indicated by LOAD DESTINATION and TRANSFER TEL commands.

Send:       CC=010405/FN=10//

Receive:    ACK

3.10 Send Image

Sends  a  previously  loaded  FAX  image  to  the  destinations

indicated by LOAD DESTINATION and TRANSFER TEL comamnds.

Send:       CC=010405/FN=15//

Receive:    ACK

3.11 Scan Image

Reads an image from the FAX scanner, and loads it into the  fax

machine's memory.

Send:       CC=010104/CD=03/FN=15//

Receive:    ACK

Fax scans document...

Receive:    IH=nn//

Successful scan results in status of 'IH=00//'.

FAXPHONE 25/FAX-225 Serial Interface                             Page: 11

4. COORDINATING COMMANDS

According to the CANON document, the following are the  recomended

orders for executing commands to accomplish the indicated functions:

4.1 Printing Text

LOAD FONT

LOAD TEXT

PRINT TEXT

4.2 Printing Images

TRANSFER IMAGE (to FAX)

PRINT IMAGE

4.3 Scanning Documents

SCAN IMAGE

TRANSFER IMAGE (from FAX)

4.4 Sending Text

LOAD DESTINATION

TRANSFER TELEPHONE or TRANSFER SPECIAL TELEPHONE

LOAD FONT

LOAD TEXT

SEND TEXT

4.5 Sending Images

LOAD DESTINATION

TRANSFER TELEPHONE or TRANSFER SPECIAL TELEPHONE

TRANSFER IMAGE (to FAX)

SEND IMAGE

FAXPHONE 25/FAX-225 Serial Interface                             Page: 12

5. CONTROL CODES

5.1 Fax Control Codes

The following character codes are used as  control  information

during communication with the FAXPHONE 25/FAX-225:

MPS     0x02            Multi-Page Signal

Indicates the image data for this page

ends, and the image data for the next

page follows without a change in

transmission mode.

EOP     0x04            End of Procedure

Used to terminate all data entries to

The FAX machine.

ACK     0x06            Acknowledge

Indicates that data or commands have

been received without error.

NAK     0x15            Negative Acknowledge

Indicates that data or commands have

not been received properly.

5.2 Printer Control Codes

The  following  character  codes  are  interpreted  as  special

actions during the reception of ASCII text:

HT      0x09            Horizontal Tab

Advances the print position to the next

TAB position, which are preset every 8

character spaces.

LF      0x0A            Line feed

Advances the print position to the next

line while remaining at the same column.

FF      0x0C            Form Feed

Causes the FAX to cut the recording paper

and begin a new page.

CD      0x0D            Carriage Return

Moves the print position to the beginning

of the current line.

FAXPHONE 25/FAX-225 Serial Interface                             Page: 13

6. ERROR CODES

When an error condition occurs, the FAXPHONE  25/FAX-225  emits  a

loud beeping, and displays the  following  error  codes  on  the  LCD

display in the form "ONLINE ERROR nn":

00          Data error during reception

Overrun or Framing error occured on the data

stream from the RS-232 imput.

04          Input/Output Data Error

The FAX did not receive data within 30 seconds

from a time when it was expected from the

computer.

07          Receive buffer overflow

The computer has sent more than 100K bytes of

data to the FAX, and has overflowed memory.

10          Command/Parameter error

There is an error in either the command code or

parameter values.

FAXPHONE 25/FAX-225 Serial Interface                             Page: 14

7. CHARACTER FONTS

The  FAXPHONE  25/FAX-225  has  no  internal  knowledge  of   what

characters received from the RS-232 port should look like. This means

that before you perform any operations which use character data,  you

need to download a FONT, which describes the exact bitmap image to be

used for each printable character.

See the "Load Font" command for information on  the  format  of  a

font file.

7.1 Pre-defined Fonts

The following fonts are included in this archive:

HIGHRES .FNT    -   A reasonable high resolution font which I

created using the FONT EDITOR.

MATRIX  .FNT    -   This is a basic 8x8 character matrix font,

which I converted to the 16x24 format used

by the FAXPHONE 25/FAX-225. It is still an

8x8 font, and gives a "dot-matrix" look.

MEDRES  .FNT    -   This is a version of MATRIX which has been

edited to increase its point resolution.

THINLINE.FNT    -   My first attempt at a high resolution font.

This one has very thin character lines, which

has the effect of producing very light text.

7.2 The Font Editor

You can create new fonts and edit the existing fonts  using  my

FONT EDITOR, which is supplied as the program FE.COM. The  program

is fairly self explainitory. Just run it and specify the  name  of

the file containing the font as an argument.

FAXPHONE 25/FAX-225 Serial Interface                             Page: 15

8. EXAMPLE PROGRAM

Included in this archive  is  a  simple  program  (FAX.COM)  which

performs the basic operations available on the FAX machine.  The  'C'

source code for this program is also included (FAX.C), for use as  an

"example" of RS-232 communication with the FAX.

This program (FAX.C) was written using my MICRO-C compiler,  which

is a shareware product available from me. See  the  included  CATALOG

file for more information.

The basic operations performed by FAX.COM are:

8.1 SCAN Document

Syntax: FAX SC <filename>

The document is scanned by the FAX machine, and loaded into the

specified file.

8.2 PRINT TEXT File

Syntax: FAX PT <filename> [font file]

The  specified  file  is  printed  as  ASCII  text,  using  the

specified  FONT.  If  no  FONT  is  given,  the  default  used  is

MEDRES.FNT.

8.3 PRINT IMAGE File

Syntax: FAX PI <filename>

The previously scanned image file is printed on the FAX.

8.4 SEND TEXT Fax

Syntax: FAX ST <filename> <tel #> [font file]

The specified file  is  sent  as  an  ASCII  text  FAX  to  the

specified telephone number. The telephone number may  contain  the

digits 0-9, 'P' or ',' may be used to indicate a  PAUSE,  and  any

other character is treated as a space. If not FONT  is  givem  the

default used is MEDRES.FNT.

8.5 SEND IMAGE Fax

Syntax: FAX SI <filename> <tel #>

The previously scanned image file is  send  as  a  FAX  to  the

specified telephone number. The telephone number may  contain  the

digits 0-9, 'P' or ',' may be used to indicate a  PAUSE,  and  any

other character is treated as a space.

FAXPHONE 25/FAX-225 Serial Interface

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

2. SERIAL INTERFACE                                                    2

2.1 Connector Pinout                                                2

2.2 Activating the serial interface                                 3

3. SERIAL FAX COMMANDS                                                 4

3.1 Load Font                                                       4

3.2 Load Text                                                       4

3.3 Transfer Image                                                  5

3.4 Load Destination                                                6

3.5 Transfer Telephone                                              7

3.6 Transfer Special Telephone                                      9

3.7 Print Text                                                     10

3.8 Print Image                                                    10

3.9 Send Text                                                      10

3.10 Send Image                                                    10

3.11 Scan Image                                                    10

4. COORDINATING COMMANDS                                              11

4.1 Printing Text                                                  11

4.2 Printing Images                                                11

4.3 Scanning Documents                                             11

4.4 Sending Text                                                   11

4.5 Sending Images                                                 11

5. CONTROL CODES                                                      12

5.1 Fax Control Codes                                              12

5.2 Printer Control Codes                                          12

6. ERROR CODES                                                        13

7. CHARACTER FONTS                                                    14

7.1 Pre-defined Fonts                                              14

7.2 The Font Editor                                                14

8. EXAMPLE PROGRAM                                                    15

8.1 SCAN Document                                                  15

8.2 PRINT TEXT File                                                15

FAXPHONE 25/FAX-225 Serial Interface                   Table of Contents

Page

8.3 PRINT IMAGE File                                               15

8.4 SEND TEXT Fax                                                  15

8.5 SEND IMAGE Fax                                                 15