4-25-90                  MODEM FLOW-CONTROL

************** FLOW CONTROL\PACING BETWEEN MODEM AND DTE **************

Xon or Xoff sent from DTE to Modem or

RTS toggled from DTE to Modem.

--------   RTS        --------

|        |----------->|        |

| Term   |----------->|        |

|        | Xoff\Xon   |Modem A |

| (DTE)  |            |        |

--------              --------

Modem Setting                      Result

&E3 or                     No effect.  Modem does not stop sending data,

&E4, &E12                   Xon and Xoff treated as data.

&E4, &E13                  No effect on Xon\Xoff characters.  Modem stops

or starts sending data to DTE depending on the

state of RTS pin 4 ( Lo = stop  Hi = start ).

&E5, &E6, &E13              Modem stops sending data when it receives an Xoff

character from the DTE and starts sending data to

the DTE when it receives an Xon character.

Xoff or Xon is discarded.

&E5, &E7, &E13              Modem stops sending data when it receives an Xoff

character from the DTE and starts sending data to

the DTE when it receives an Xon character.

Xoff or Xon is also sent to remote modem.

&E5, &E6, &E12              Xon and Xoff characters are discarded and have no

effect on data flow from modem to DTE.

&E5, &E7, &E12              No effect.  Modem does not stop sending data,

Xon and Xoff treated as data.

Modem generates Xon or Xoff to DTE or

Modem toggles CTS output to DTE.

--------     CTS      --------

|        |<-----------|        |

| Term   |<-----------|        |

|        |  Xoff\Xon  |Modem A |

| (DTE)  |            |        |

--------              --------

Modem setting                     Result

&E3                       Modem does not generate Xon or Xoff to DTE.

&E4                       Modem does not generate Xon or Xoff to DTE.

Modem drops CTS pin 5 when send buffer fills to

1024 characters and raises CTS when send buffer

empties to 767 characters.

&E5                       Modem sends Xoff when send buffer fills to 1024

characters (&B0), 256 characters (&B1).  Modem

sends Xon when send buffer empties to 767 (&B0)

characters, 127 characters (&B1).

**************** FLOW-CONTROL DURING ERROR CORRECTING LINK *******************

Flow-control\pacing between two error correcting modems, the following is

true:

1)  A reliable link is established between Modem A and Modem B.

2)  Flow-control and pacing may be either Xon\Xoff or hardware.

For this example the computer is dumping data to Modem B for the printer.  The

serial port speed of both modems is 9600 bps and the modem link is 2400 bps.

When the printer buffer fills it will initiate flow control to Modem A.   The

computer will indirectly get the message to stop sending data to the printer.

9600 bps               2400 Link                   9600 bps

--------       -------------                  -------------        --------

|        |-----| |B |        |                |       |B |  |------|        |

|Printer |-----| |U |        |--------\-------|       |u |  |------|  CPU   |

|        |-----| |F |  Modem |--------\-------|Modem  |f |  |------|        |

| (DTE)  |     | |F |    A   |                |  B    |f |  |      |        |

--------       -------------                  -------------        --------

Modem A settings                    Modem B settings

&E5, &E6, &E13                       &E5, &E6, &E12

Result:

Computer is dumping data to Modem B for the printer.  Here is the sequence of

flow-control events:

1)  The printer buffer fills and sends an Xoff to Modem A.

2)  Modem A tells Modem B to stop sending data. Modem A does not send the Xoff

character, it tells Modem B to stop via the error correction protocol.

3)  Modem B stops sending data to Modem A.

4)  The computer continues to send data to Modem B until the modem buffer

fills.  Then Modem B initiates an Xoff to stop the computer.

Modem A settings                 Modem B settings

&E4, &E13                        &E4, &E12

Result:

Computer is dumping data to Modem B for the printer.  Here is the sequence of

flow-control events:

1)  The printer buffer fills and drops RTS pin 4 to Modem A.

2)  Modem A tells Modem B to stop sending data via the error correction

protocol.

3)  Modem B stops sending data to Modem A.

4)  The computer continues to send data to Modem B until the modem buffer

fills.  Then Modem B drops CTS pin 5 to stop the computer.

The Flow-control\pacing feature can be used to convert between different flow-

control methods, (i.e. Xon\Xoff at the printer and CTS at the computer).

**** NORMAL MODE FLOW-CONTROL (&E11) FROM MODEM TO MODEM AND MODEM TO DTE ****

IN NORMAL MODE, WITH THE SERIAL PORT SPEED FASTER THAN THE MODEM BAUD-RATE,

THE FOLLOWING IS TRUE:

1)  Modem A has a serial port speed of 2400.

2)  Modem B has a serial port speed of 9600 (speed conversion).

3)  Modem A may be any manufacturers modem.

This feature is usually used to prevent the DTE device attached to Modem A

from missing data after sending an Xoff character to the host CPU.  If we

just pass through the Xoff character directly to the CPU data in the buffer of

Modem B will continue to be sent and the DTE of Modem A will miss this data.

So, the key advantage of normal mode flow-control is that buffered data as

well as the host computer is stoped when the Modem A DTE sends an Xoff

character.

2400 bps            2400 Link                9600 bps

--------         --------                  -------------        --------

|        |-------|        |                |       |B |  |------|        |

| Term   |-------|        |--------\-------|       |u |  |------|  CPU   |

|        |-------|Modem A |--------\-------|Modem  |f |  |------|        |

| (DTE)  |       |        |     Xoff-->    |  B    |f |  |      |        |

--------         --------                  -------------        --------

Modem B Setting                Result

&E10                     Xon and Xoff treated as data

&E11, &E6                Xoff or Xon will cause Modem B to

with &E4 or &E5            stop or start sending data to Modem A.

The Xoff or Xon is not sent directly to CPU.

Modem B will initiate flow-control to the host CPU

when it's transmit buffer fills to 1024 characters.

Modem B will restart the CPU when the transmit

buffer empties to 255 characters.

&E11, &E7                Xoff or Xon will cause Modem B to stop or start

with &E5                   sending data to Modem A.  Xoff or Xon is also sent

to the CPU as if it were data.  We don't recommend

this setting because we run into lock-up problems

with some computers with this setting.  The only

reason to use this setting is to prevent Modem B

from buffering data sent from the CPU during the

time that DTE A has sent the Xoff.

**** NORMAL MODE FLOW-CONTROL (&E11) FROM MODEM TO MODEM AND MODEM TO DTE ****

IN NORMAL MODE, WITH THE SERIAL PORT SPEED SLOWER THAN THE MODEM BAUD-RATE,

THE FOLLOWING IS TRUE:

1)  Modem A has a serial port speed of 2400 bps.

2)  Both modem baud-rate and Modem A serial port speed are faster than Modem B

serial port speed.

3)  Modem B has a serial port speed of 1200 bps.

4)  Modem A may be any manufacturers modem.

In this application normal mode flow-control will cause Modem B to send an

Xoff\Xon to Modem A for two reasons.

a)  Modem B buffer fills to 1024 characters.  This may happen because the

modem can only dump data to the printer at 1200 bps while it is receiving it

at 2400 bps from Modem A.

b)  Modem B buffer fills to 1024 characters.  This may happen if the printer

sends an Xoff to Modem B and Modem B has pacing enabled.  This will cause

Modem B to stop dumping data to the printer, so the receive buffer will fill

and Modem B will send an Xoff to Modem A.

2400 bps            2400 Link                1200 bps

--------         --------                  -------------        --------

|        |-------|        |                |       |B |  |------|        |

|PC      |-------|        |-------\--------|       |u |  |------|PRINTER |

|        |-------|Modem A |-------\--------|Modem  |f |  |<-----|        |

|        |       |        |    <--Xoff     |  B    |f |  | Xoff |        |

--------         --------                  -------------        --------

Modem b setting                  Result

&E10                      Modem B does not generate Xon or Xoff to Modem A.

&E11                      Modem B sends Xoff to Modem A when Modem B receive

buffer fills to 1024 characters.  Modem A should

send the Xoff character to the PC which will stop

sending data.   Modem B sends Xon to Modem A when

receive buffer empties to 767 characters.  Modem A

should send the Xon to the PC restarting the data.

######################