066010303040000132002006006008000

1   4 January 1896                          #0

2 Atari Corp Confidential0

F0110030001

9[..........................................]0110

EngineeringHardwareSpecification

ofthe

AtariSTComputerSystem

TheAtariCorporation

Sunnyvale,California

THESCOPEOFTHISDOCUMENTislimitedtoa descriptionofthe

ST computer system hardware characteristicsandidiosyn

crasies. Thisdocumentdoesnotprovide a detaileddescription

ofSTcomponentparts,peripheraldevices,dev-iceprotocols,or

software.  References are provided fordetailedinquiry.

1. SystemArchitecture

----------                   ----------

|Graphics |                 |Music    |

|Subsystem|                 |Subsystem|

----------                   ----------

|                           |

|        ----------        |

--------|Main    |--------

|System  |

----------

|

|        ----------

TheST               --------|Device   |

|Subsystems|

----------

ThehardwarearchitectureoftheAtari Corporation ST

(Sixteen/Thirty-two)computersystemconsistsofamainsys-tem,

agraphicssubsystem,amusic subsystem, and severaldevice

subsystems(mostofthedevicesubsystemsrequireSTresident

intelligence). TheSTisbasedonthe MC68000 16bit data/24

bit address microprocessor unit capableofdirectlyaccessing

upto16MbytesofROM and RAM memory.

HardwarefeaturesoftheSTcomputersysteminclude:

MainSystem

o 16bitdata/24bitaddress8MHzmicroprocessorunit

o 192KbyteROM,cartridgeexpandableto320Kbyte

o 512KbyteRAMor1MbyteRAM

o directmemoryaccesssupport

GraphicsSubsystem

o 32KbyteBitMapvideodisplaymemory(fromabove)

o 320x200pixel,16colorpalettefrom512selections

o 640x200pixel,4colorpalettefrom512selections

o 640x400pixel,monochrome

MusicSubsystem

o programmablesoundsynthesizer

o musicalinstrumentnetworkcommunication

DeviceSubsystems

o intelligentkeyboard

o twobuttonmouse

o RGBcolorandmonochromemonitorinterfaces

o printerparallelinterface

o RS232serialinterface

o MIDImusicalinstrumentinterface

o onboardfloppydiskcontrollerandDMAinterface

o harddiskdriveDMAinterface

-----AtariSTSystemBlockDiagram---------------

9[...................................................]1110

------------

|MC68000MPU|

|           |

------------

|                          ----------

|<------------------------>|192Kbyte|<--->EXPAN

---------|------------------------->|ROM     |

|        |                          ----------

|        |                          ----------

|        |                         |512Kor1M|

|        |                      -->|byteRAM|<--

----------       |       ----------   |   ----------   |

|Control |<----->|<----->|Memory  |<--                  |

|Logic   |-------|------>|Controller|<--                  |

----------       |       ----------   |   ----------   |

|||||           |       ----------    -->|Video   |<-- RFMOD

|||||           |<----->|Buffers |<----->|Shifter |---->RGB

|||||           |      |         |       ----------     MONO

|||||           |       ----------

|||||           |       ----------        ----------

|||||           |<----->|MC6850  |<----->|Keyboard|<--->IKBD

||||------------|------>|ACIA    |      |Port    |

||||            |       ----------        ----------

||||            |       ----------        ----------

||||            |<----->|MC6850  |<----->|MIDI    |---->OUT/THRU

|||-------------|------>|ACIA    |      |Ports   |<----IN

|||             |       ----------        ----------

|||             |       ----------        ----------

|||             |<----->|MK68901 |<----->|RS232   |<--->MODEM

||--------------|------>|MFP     |<--   |Port    |

||              |       ----------   |   ----------

||              |                     |   ----------

||              |                      ---|Parallel|<--->PRINTER

||              |                      -->|Port    |

||              |       ----------   |   ----------

||              |<----->|YM-2149 |<--    ----------

|---------------|------>|PSG     |------>|Sound   |---->AUDIO

|               |      |         |---   |Channels|

|               |       ----------   |   ----------

|               |                     |   ----------

|               |       ----------    -->|Floppy  |<--->FLOPPY

|               |<------|WD1772  |<----->|DiskPort|    DRIVE

|               |   -->|FDC     |       ----------

|               |  |   ----------

|               |  |   ----------        ----------

|               |   -->|DMA     |<----->|HardDisk|<--->HARD

|               |<----->|Controller|      |Port    |    DRIVE

----------------------->|         |       ----------

----------

2. MainSystem

----------                   ----------

|         |                 |         |

|         |                 |         |

----------                   ----------

|                           |

|        ----------        |

--------|Main    |--------

|System  |

----------

|

|        ----------

--------|         |

|         |

----------

9[..........................................]0110

Themainsystemincludesthemicroprocessorunit, mainmemory

(both ROM and RAM), andgeneralpurposeDMAcon-troller.

Themainsystemis limited to memory expansiononlyand

shouldnotbeconsideredanopensystem(exceptviathehigh

speeddeviceinterfaces).

2.1. MicroprocessorUnit

TheSTcomputersystemisbasedonan8MHzMC68000 16bit

data/24bitaddressmicroprocessorunit(withaninter-nal32

bitarchitecture). SomefeaturesoftheMC68000are: eight 32

bitdataregisters,nine32bitaddressregisters,a16Mbyte

directaddressing range, 14 addressing modes,memory mapped

I/O, five datatypes,anda56instructionset. TheMPUis

directly supported by an MK68901 MultiFunctionPeripheral

providinggeneralpurposeinterruptcon-trolandtimers,among

otherthings.

2.2. MemoryConfiguration

Theconfigurationofmainmemoryconsists of five 64Kbyte

setsofROM(standardset0toset2,expansionset3andset4)and

oneconfigurable bank (standard bank0) of 128Kbyte,512

Kbyte,or2MbyteRAM. TheconfigurationofmainmemoryROMis

ascertainedthrough software identification.

The configuration of main memoryRAMisachievedviathe

programming  of  the  Memory   Configuration   Register

(read/write, reset: allzeros). RAMconfigurationmustbe

assertedduringthefirststepsofthepowerupsequenceandcan

be determinedbyusingthefollowingshadowtestalgo-rithm:

START  o write0x000a(2Mbyte,2Mbyte)totheMemory

ConfigurationRegister.

BANK0  o writePatternto0x000000-0x0001ff.

o readPatternfrom0x000200-0x0003ff.

o ifMatch,thenBank0contains128Kbyte;gotoBANK1.

o readPatternfrom0x000400-0x0005ff.

o ifMatch,thenBank0contains512Kbyte;gotoBANK1.

o readPatternfrom0x000000-0x0001ff.

o ifMatch,thenBank0contains2Mbyte;gotoBANK1.

o panic: RAMerrorinBank0.

BANK1  o writePatternto0x200000-0x2001ff.

o readPatternfrom0x200200-0x2003ff.

o ifMatch,thenBank1contains128Kbyte;gotoFIN.

o readPatternfrom0x200400-0x2005ff.

o ifMatch,thenBank1contains512Kbyte;gotoFIN.

o readPatternfrom0x200000-0x2001ff.

o ifMatch,thenBank1contains2Mbyte;gotoFIN.

o note: Bank1nonexistent.

FIN    o writeConfigurationtotheMemoryConfiguration

Register.

o noteTotalMemorySize(TopofRAM)forfuture

reference.

RAMmemoryaccesscyclesare interleaved between the

microprocessor unitandthevideocontroller,thusallowing

videodisplaymemorytoresideefficientlyaspart of main

memory(theMPUstillmaintainsfullmemorybandwidth).

_2.3. DirectMemoryAccess

AdirectmainmemoryRAMaccesschannel is shared to

provide support for bothlowspeed(250to500Kbits/sec)and

highspeed(upto12Mbits/sec)8bit device controll-ers.

The baseaddressfortheDMAreadorwriteoperationisloaded

intotheDMABase Address and Counter Register(read/write,

reset: all zeros).  Since onlyonecounterregisterand

channelisprovided,onlyoneDMAoperationcanbeexecutedata

time.

TheactualDMAoperationisperformedthrougha32byteFIFO

programmed via the DMA ModeControlRegister(wordaccess

writeonly,reset:notaffected)andDMASectorCountRegister

(word access writeonly,reset:allzeros). Theprogress,

success,orfailureofaDMAoperationisreportedthrough the

DMA Status Register (wordaccessreadonly,reset:one)which

isclearedbytogglingWrite/_Readin theDMAModeControl

Register.

Busaccessesaregranted to the DMA controller and

MC68000 MPU on an egalitarian first come,firstserved

basis. Theaccessremainsineffectuntilan operation is

completeoruntilcontrolisotherwiserelinquished.

9[..........................................]0110

3. GraphisSubsystem

----------                   ----------

|Graphics |                 |         |

|Subsystem|                 |         |

----------                   ----------

|                           |

|        ----------        |

--------|         |--------

|         |

----------

|

|        ----------

--------|         |

|         |

----------

Thebasiccomponents of the graphics subsystem arevideo

displaymemoryandthevideocontroller. Thedefini-tionsof

BitMap, BitBlk, BitBlt, coordinate system, andcolor model

arelefttohigherlogical(software)levelsofsystemspecifica

tion.

3.1. VideoDisplayMemory

Videodisplaymemoryisconfiguredasnlogical planes

interwoven by 16 bitwordsintocontiguousmemorytoformone

32Kbyte(actually0x7d00)physical plane starting atany256

bytehalfpageboundary(inRAMonly). Thestartingaddressof

display memory is placed in the Video BaseAddress

Register (read/write, reset: allzeros)whichisthenloaded

intotheVideoAddress Counter Register (readonly,reset:all

zeros)andincremented. Thefollowingisadiagramofpossible

physicalconfigurationsofvideodisplaymemory:

--------

16bitword  |       |

----------------------------------------

4plane      |plane0|plane1|plane2|plane3|plane0|

----------------------------------------

2plane      |plane0|plane1|plane0|plane1|plane0|

----------------------------------------

1plane      |plane0|plane0|plane0|plane0|plane0|

----------------------------------------

9[..........................................]0110

Displaymemoryresidesaspartofmainmemory and hasan

identicalbit,byte,andwordarrangementwiththephysi-cal

screenoriginlocatedattopleft(bit15):

high    low

----------------

word0       |byte0|byte1|

----------------

fedcba9876543210

3.2. VideoConfiguration

TheSTpossessesthreemodes of video configuration:

320 x 200 resolution with4planes,640x200resolutionwith

2planes,and640x400resolutionwith1 plane.  Themodes

are setthroughtheShiftModeRegister(read/write,reset:all

zeros). Asixteenwordcolorlookup palette isprovided with

nine bits ofcolorperentry. ThesixteenColorPalette

Registers(read/write, reset: not affected)contain three

bits ofred,green,andbluealignedonlownibbleboundaries.

Eightintensity levels of red, eightintensity levels of

green, andeightintensitylevelsofblueproduceatotalof512

possiblecolors.

In320x2004planemodeall sixteen palette colorscan

be indexed, while in640x2002planemodeonlythefirst

fourpaletteentriesareapplicable.  In 640 x 400monochrome

modethecolorpaletteisbypassedaltogetherandisinstead

providedwithaninverterforinversevideo con-trolled by

bit0ofpalettecolor0(normalvideoisblack0,white1).

Colorpalettememoryisarrangedthe same asmain memory.

Palette color 0 is alsousedtoassignabordercolorwhile

ina multi-plane mode.  In monochromemodethebordercolor

isalwaysblack.

Thegeneralflowofthevideocontrollerisasfollows:

BitMap planesaretakenawordatatimefromvideodisplay

memoryandplacedinthevideoshiftregisterwhereone bitfrom

each planeisshiftedoutandcollectivelyusedasanindex

(plane0istheleast significant bit) to a colorlookup

palette entrywhichissuppliedto3bitdigitaltoanalog

converterstoproduceRGBoutput. Thefollowingisablock

diagramofthevideocontroller:

9[...................................................]1110

-----VideoControllerBlockDiagram---------------

--------         --------       --------  lsb --------

|3     |------>|0123|----->|     0|----->|16x9|-

-------- |   |  |       |     |     1|----->|Lookup|-|->R

|2     |------   |       |     |     2|----->|       |-

-------- ||   |  |       |     |     3|----->|       |-

|1     |--------   |       |     |       |--   |       |-|->G

-------- ||-    |   --------       --------  |  |       |-

|0     |----------                               |  |       |-

|       ||-                                      |  |       |-|->B

|       ||                                       |  |       |-

|       |-                                        |   --------

|       |                                         |  --------

--------                                          -->|Inverter|->MONO

--------

LogicalBitMap         VideoDisplay  VideoShift    ColorPalette

Planes                 Memory         Register       and3BitDACs

9[..........................................]0110

4. MusicSubsystem

----------                   ----------

|         |                 |Music    |

|         |                 |Subsystem|

----------                   ----------

|                           |

|        ----------        |

--------|         |--------

|         |

----------

|

|        ----------

--------|         |

|         |

----------

TheSTmusicsubsystemiscomposed of a programmablesound

synthesizerandamusicalinstrumentserialinterface.Theinter

faceprovideshighspeed serial communication ofmusicaldata

toandfrommoresophisticatedsynthesizerdev-icesubsystems.

4.1. SoundSynthesizer

TheYM-2149ProgrammableSoundGeneratorproducesmusicsyn

thesis, soundeffects,andaudiofeedback(egalarmsandkey

clicks). Withanappliedclockinputof2MHz,the PSGis

capable ofprovidingafrequencyresponserangebetween30Hz

(audible)and125KHz(post-audible).  The generatorplaces a

minimal amount ofprocessingburdenonthemainsystem(which

actsasthesequencer)andhastheability toperform using

threeindependentvoicechannels. Thethreesoundchannel

outputsaremixed,alongwith Audio In, andsent to an

externaltelevisionormonitorspeaker(thePSGhasbuiltin

digitaltoanalogconverters).

Thesoundgenerator'sinternalregisters are accessedvia

the PSG Register SelectRegister(writeonly,reset:regi

stersallzeros). Thetonegeneratorregisters controla basic

squarewavewhilethenoisegeneratorregistercon-trolsa

frequencymodulatedsquare wave of pseudo randompulse

width.  Tonesandnoisecanbemixedoverindividualchannels

byusingthemixercontrolregister. Theamplituderegisters

allowthespecificationofafixedamplitudeorofavariable

amplitudewhenusedwiththeenvelope generator.The envelope

generator registers permit the entryofaskewedattack-

decay-sustain-releaseenvelopeintheform ofacontinue-attack-

alternate-holdenvelope.

4.2. MusicalInstrumentCommunication

TheMusicalInstrumentDigitalInterface(MIDI) allowsthe

integrationoftheSTwithmusicsynthesizers,sequenc-ers,

drumboxes,andotherdevices possessing MIDI inter-faces.

High speed (31.25 Kbaud)serialcommunicationofkeyboardand

programinformationisprovidedby two ports,MIDI OUT and

MIDIIN(MIDIOUTalsosupportstheoptionalMIDITHRUport).

TheMIDIbuspermitsupto16channelsinoneof three

network addressingmodes: Omni(allunitsaddressedsimul-

taneously,powerup default), Poly (each unit addressed

separately),  and  Mono  (each  unit  voice  addressed

separately). Informationiscommunicatedviafivetypes ofdata

format (data bytes,mostsignificantbit: status1,data0)

whichareprioritizedfrom highest to lowest as:System

Reset (default conditions, should notbesentonpowerupto

avoiddeadlock),SystemExclusive (manufacturerunique data:

Sequential Circuits, Kawai, Roland,Korg,Yamaha),SystemReal

Time(synchronization), System Common(broadcast), and

Channel (note selections,programdata,etc).

5. DeviceSubsystems

----------                   ----------

|         |                 |         |

|         |                 |         |

----------                   ----------

|                           |

|        ----------        |

--------|         |--------

|         |

----------

|

|        ----------

--------|Device   |

|Subsystems|

----------

TheSTsupportssevendevicesubsystems:  an intelli-gent

keyboard, video interface,parallelinterface,RS232interface,

MIDIinterface,floppydiskinterface, and harddiskinterface.

Included with each device interfacedescriptionisaportpin

assignmentchartwiththe ST andprogrammablesignalsjustified

left(pinsthatarenotcon-nectedarenotshown). The

connector type on the ST isshown above eachpin list

withanSdesignatingafemalesocketandaPdesignatingamale

plug.

5.1.IntelligentKeyboard

TheAtariIntelligentKeyboard(ikbd)transmitsencoded

make/break  key  scan  codes (with two key rollover),

mouse/trackballdata,joystickdata,andtimeof day.  Theikbd

receivescommandsaswell,withbidirectionalcommuni-cation

controlledontheSTsidebyan MC6850 AsynchronousCommuni

cations InterfaceAdaptersuppliedwithtransmitandreceive

clockinputsof500KHz. Thedatatransferrate isaconstant

7812.5bits/secwhichcanbegeneratedbysettingtheACIA

CounterDivideSelecttodivideby 64.  All ikbdfunctions

such as key scanning, mousetracking,commandparsing,etc.

areperformedbya1MHzHD6301V18bitMicro-computerUnit.

Theikbdisequippedwithacombination mouse/joystickport

and ajoystickonlyport. TheAtariTwoButtonMouseisa

mechanical,opto-mechanical,oropticalmousewith thefollowing

minimalperformancecharacteristics:aresolutionof100

counts/inch(4counts/mm),amaximumvelocity of 10

inches/second (250 mm/second), and amaximumpulsephase

errorof50percent. TheAtariJoystickis a 4 direction

13switch-type joystick with one firebutton. Theikbdcan

reportmovementusingone of three mouse/joystick modes:

mousewithjoystick,disabledmousewithjoystick,andjoys-tick

withjoystick.

-----Mouse/Joystick0PortPinAssignments---------------

ST                                  DB9P

----                                   ----

IKBDMATRIX      1|<---XBPulse/UpSwitch--------|

IKBDMATRIX      2|<---XAPulse/DownSwitch------|

IKBDMATRIX      3|<---YAPulse/LeftSwitch------|

IKBDMATRIX      4|<---YBPulse/RightSwitch-----|

IKBDMCU         6|<---LeftButton/FireButton---|

7|----Power---------------------->|

8|----Ground----------------------|

IKBDMCU         9|<---RightButton/Joy1Fire----|

----                                   ----

SignalCharacteristics

mousepins1-4         TTLlevels.

joystick0pins1-4     TTLlevels.

pin6                  TTLlevels,closuretoground.

pin7                  +5VDC.

pin9                  TTLlevels,closuretoground.

9[...................................................]1110

MousePhaseDirections

POSITIVERIGHT(UP)            NEGATIVELEFT(DOWN)

.   -----      -----                 -----      -----

XA(YA)    |    |    |    |               |    |    |    |

|    |    |    |               |    |    |    |

.--      -----      -----     -----      -----      --

.      -----      -----           -----      -----

XB(YB)       |    |    |    |         |    |    |    |

|    |    |    |         |    |    |    |

.-----      -----      --     --      -----      -----

9[..............................................]0110

-----Joystick1PortPinAssignments---------------

ST                                 DB9P

----                                   ----

IKBDMATRIX      1|<---UpSwitch-------------------|

IKBDMATRIX      2|<---DownSwitch-----------------|

IKBDMATRIX      3|<---LeftSwitch-----------------|

IKBDMATRIX      4|<---RightSwitch----------------|

IKBDMCU         6|<---FireButton-----------------|

7|----Power---------------------->|

8|----Ground----------------------|

----                                   ----

SignalCharacteristics

pins1-4,6             TTLlevels.

pin7                  +5VDC.

_5.2. VideoInterface

TheSTvideodisplayinterfacesupportslow resolution(320

x200or640x200)televisionreceiversandcompositemonitors,

lowresolution(320x200or640x200)RGB moni-tors,and

mediumresolution(640x400)monochromemonitors.

BothNTSCandPALcolorencodingstandardsareavailable intwo

separateconfigurationsofthevideointerfacehardware.For

UnitedStatestelevision receivers, the modulated RFdisplay

signal isprovidedontwoadjacentbroadcastchan-nels:

channel2(55.25MHz)andchannel3(61.25MHz).  Inorder to

protect against monitor damagewhileinmediumresolution

monochromemode,the Monochrome Monitor Detectsignal is

provided as an interruptontheMK68901MultiFunction

Peripheralandcanbetestedtodeterminethepres-ence of a

monochromemonitor(activelow,anychangewillgeneratethe

interruptifenabled).

External/Internalsyncsand50/60Hz field rates areselec

ted viatheSyncModeRegister(read/write,reset:allzeros).

TheYM-2149ProgrammableSoundGeneratorI/OPortAprovides a

GeneralPurposeOutputforuseinthecontroloffunctionssuch

astheremoteselectionof monitor externalor internal sync.

Twoautovectorinterruptsaregeneratedtoallowsoftwaresynch

ronizationwithhorizontalandverti-cal blanking intervals.

The Horizontal Blanking(down)CounterMFPTimerBhasan

activehighinputsignalandpro-duces an interruptwhenthe

countertimesout(EventCountMode).  The horizontal blanking

counter actually  usesdisplay enable,thefirstofwhich

occursattheendofthefirstdisplayline. Thevideodisplay

shiftmodeshould beselectedduringtheverticalblanking

interval.

-----VideoPortPinAssignments---------------

Television

ST                         RCAPinJack

----                                   ----

core|----RFModulatedVideo--------->|

shield|----Ground----------------------|

----                                   ----

Monitor

ST          CircularDIN13S

----                                   ----

1|----AudioOut------------------>|

2|----CompositeVideo------------>|

PSGI/OA        3|----GeneralPurposeOutput----->|

MFP              4|<---MonochromeDetect-----------|

5|<---AudioIn--------------------|

6|----Green---------------------->|

7|----Red------------------------>|

8|----PeritelPower-------------->|

9|----HorizontalSync------------>|

10|----Blue----------------------->|

11|----Monochrome----------------->|

12|----VerticalSync-------------->|

13|----Ground----------------------|

----                                   ----

SignalCharacteristics

pin1                  1VDCpeaktopeak,10Kohm.

pin2                  1VDCpeaktopeak,75ohm.

pin3                  TTLlevels,assertionnotassigned.

pin4                  TTLlevels,activelow,

1Kohmpullupresistorto+5VDC.

pin5                  1VDCpeaktopeak,10Kohm.

pins6-7               1VDCpeaktopeak,75ohm.

pin8                  +12VDCpullup.

pin9                  5VDCactivelow,3.3Kohm.

pin10                 1VDCpeaktopeak,75ohm.

pin11                 1VDCpeaktopeak,75ohm.

pin12                 5VDCactivelow,3.3Kohm.

5.3. ParallelInterface

TheSTparallelinterface supports Centronics STROBEfrom

theYM-2149PSGfordatasynchronizationandCentronicsBUSYto

the MK68901 MFP (ACKNLG is not supported) forhandshaking.

Eight bits of read/write dataarehandledthroughI/OPortB

onthePSGat a typical data transferrateof4000

bytes/second.

-----ParallelPortPinAssignments---------------

ST          DB25S

----                                   ----

PSGI/OA        1|----CentronicsSTROBE---------->|

PSGI/OB        2|<---Data0--------------------->|

PSGI/OB        3|<---Data1--------------------->|

PSGI/OB        4|<---Data2--------------------->|

PSGI/OB        5|<---Data3--------------------->|

PSGI/OB        6|<---Data4--------------------->|

PSGI/OB        7|<---Data5--------------------->|

PSGI/OB        8|<---Data6--------------------->|

PSGI/OB        9|<---Data7--------------------->|

MFP             11|<---CentronicsBUSY-------------|

18-25|----Ground----------------------|

----                                   ----

SignalCharacteristics

pin1                  TTLlevels,activelow.

pins2-9               TTLlevels.

pin11                 TTLlevels,activehigh,

1Kohmpullupresistorto+5VDC.

5.4. RS232Interface

TheSTRS232interfaceprovidesvoltagelevel synchro-nous

orasynchronousserialcommunication. FiveEIARS232Chandshake

controlsignalsaresupported:  Request To Sendand Data

TerminalReadyaretransmittedthroughtheYM-2149PSGI/OPortA

whileClearToSend,DataCarrierDetect,andRing Indicator

are receivedthroughtheMK68901MFP. TheMFPUSARTtransmit

andreceiveclockinputs are controlledby the Baud Rate

GeneratorMFPTimerDwhichissuppliedwith2.4576MHzand

cansupportasynchronous data transferrates from50to19200

baud. Onebytetransmitandreceivedatabuffersaremanagedby

theMFP USART, which providesmonitoringofbufferconditions

andcommunicationerrors.

-----RS232PortPinAssignments---------------

ST          DB25P

----                                   ----

1|----ProtectiveGround-----------|

MFP              2|----TransmittedData----------->|

MFP              3|<---ReceivedData---------------|

PSGI/OA        4|----RequestToSend------------>|

MFP              5|<---ClearToSend---------------|

7|----SignalGround---------------|

MFP              8|<---DataCarrierDetect---------|

PSGI/OA       20|----DataTerminalReady-------->|

MFP             22|<---RingIndicator--------------|

----                                   ----

SignalCharacteristics

pins2-5               RS232Clevels.

pins8,20,22           RS232Clevels.

5.5. MIDIInterface

TheSTMIDIinterfaceprovidescurrent loop asynchro-nous

serialcommunicationcontrolledbyanMC6850ACIAsup-plied

withtransmitandreceiveclock inputs of 500 KHz.The data

transferrateisaconstant31.25Kbaudwhichcanbegeneratedby

settingtheACIACounter Divide Select todivide by16. The

MIDIspecificationcallsforserialdatatoconsistofeightdata

bitsprecededbya start bit andfollowedbyonestopbit.

-----MIDIPortPinAssignments---------------

MIDIOUT/THRUST          CircularDIN5S

----                                   ----

MIDIIN     1|----THRUTransmitData--------->|

2|----ShieldGround---------------|

3|<---THRULoopReturn------------|MIDIACIA

4|----OUTTransmitData---------->|

5|<---OUTLoopReturn-------------|

----                         ----

MIDIIN

ST        CircularDIN5

----                            ----

MIDIACIA  4|<---INReceiveData-------------|

5|----INLoopReturn------------->|

----                                   ----

SignalCharacteristicscurrentloop

5ma,zeroiscurrenton.

5.6. DiskDriveInterface

TheSTfloppydiskdriveinterfaceisprovided throughthe

DMA controller toanonboardWD1772FloppyDiskCon-troller.

Atotaloftwodaisychained floppy disk drives(drive 0 or

1)canbesupported. CommandsaresenttotheFDCbyfirst

writingtotheDMA Mode Control Register toselect the FDC

internalcommandregisterandthenwritingthedesiredonebyte

commandtotheDisk Controller Regis-ter.  The entirefloppy

diskDMAreadorwritesequenceisasfollows:

o selectfloppydrive0or1(PSGI/OPortA).

o selectfloppyside0or1(PSGI/OPortA).

o loadDMABaseAddressandCounterRegister.

o toggleWrite/_Readtoclearstatus(DMAModeControlRegister).

o selectDMAreadorwrite(DMAModeControlRegister).

o selectDMASectorCountRegister(DMAModeControlRegister).

o loadDMASectorCountRegister(DMAoperationtrigger).

o selectFDCinternalcommandregister(DMAModeControlRegister).

o issueFDCreadorwritecommand(DiskControllerRegister).

o DMAactiveuntilsectorcountiszero(DMAStatusRegister,

donotpollduringDMAactive).

o issueFDCforceinterruptcommandonmulti-sectortransfers

exceptattrackboundaries(DiskControllerRegister).

o checkDMAerrorstatus(DMAStatusRegister,nondestructive).

Thedetectionoffloppydiskremovalis not supportedin

hardware.

TheSThard disk drive interface is also provided

throughtheDMAcontroller,howevertheAtariHardDiskCon-

trollerisoffboardandis sent commands using an ANSI

X3T9.2 SCSI-like(SmallComputerSystemsInterface)command

descriptorblockprotocol. TheAtari Hard Disk Interface

(AHDI) supports aminimalsubsetofSCSIcommands(Class0

OpCodes),whicharedispatchedusingthefollowingfixedsixbyte

Atari ComputerSystemInterface(ACSI)commandpacketformat:

-----ACSICommandDescriptorBlock---------------

Byte0 |xxxxxxxx|

||||||||

|||--------OperationCode

-----------ControllerNumber

Byte1 |xxxxxxxx|

||||||||

|||---------BlockAddressHigh

-----------DeviceNumber

Byte2 |xxxxxxxx|

||||||||

-----------BlockAddressMid

Byte3 |xxxxxxxx|

||||||||

-----------BlockAddressLow

Byte4 |xxxxxxxx|

||||||||

-----------BlockCount

Byte5 |xxxxxxxx|

||||||||

-----------ControlByte

The following is a summary of available  command

OpCodes:

-----AHDICommandSummaryTable---------------

------------------------------

|OpCode  |Command           |

------------------------------

|0x00    |TestUnitReady   |

|0x05    |VerifyTrack      | *

|0x06    |FormatTrack      | *

|0x08    |Read              | *

|0x0a    |Write             | *

|0x0b    |Seek              |

|0x0d    |CorrectionPattern|

|0x15    |ModeSelect       |

|0x1a    |ModeSense        |

------------------------------

* multisectortransferwithimpliedseek

NOTE: subjecttochange.

CommandsareissuedtotheAtariHDCinamanner similar to

thatoftheFDC,withthemajordifferencebeingthehandshaking

ofamulti-bytecommanddescriptor block.  Theentireharddisk

DMAreadorwritesequenceisasfollows:

o loadDMABaseAddressandCounterRegister.

o toggleWrite/_Readtoclearstatus(DMAModeControlRegister).

o selectDMAreadorwrite(DMAModeControlRegister).

o selectDMASectorCountRegister(DMAModeControlRegister).

o loadDMASectorCountRegister(DMAoperationtrigger).

o selectHDCinternalcommandregister(DMAModeControlRegister).

o issuecontrollerselectbytewhileclearingA0.

o setA0forremainingcommandbytes.

o afterlastcommandbyteselectcontroller(DMAModeControl

Register).

o DMAactiveuntilsectorcountiszero(DMAStatusRegister,

donotpollduringDMAactive).

o checkDMAerrorstatus(DMAStatusRegister,nondestructive).

o checkHDCstatusbyteandifnecessaryperformECCcorrection

followingaVerifyTrackorReadSectorcommand.

Theformatofbothfloppyandhard disks contain 512bytedata

sectors.

-----FloppyDiskPortPinAssignments---------------

ST                           CircularDIN14S

----                                   ----

1|<---ReadData-------------------|

PSGI/OA        2|----Side0Select--------------->|

3|----LogicGround----------------|

4|<---IndexPulse-----------------|

PSGI/OA        5|----Drive0Select-------------->|

PSGI/OA        6|----Drive1Select-------------->|

7|----LogicGround----------------|

8|----MotorOn------------------->|

9|----DirectionIn--------------->|

10|----Step----------------------->|

11|----WriteData----------------->|

12|----WriteGate----------------->|

13|<---Track00--------------------|

14|<---WriteProtect---------------|

----                                   ----

NOTE: shieldgroundmustnotbeconnectedontheSTside,the

cablemusthaveReadDataandWriteDatapairedwithLogic

Grounds.

SignalCharacteristics

pin1                  TTLlevels,activelow,

1Kohmpullupresistorto+5VDC.

pin2                  TTLlevels,activehigh,

highatsystemreset.

pin4                  TTLlevels,activelow,

1Kohmpullupresistorto+5VDC.

pins5-6               TTLlevels,activelow,

highatsystemreset.

pins8-12              TTLlevels,activelow(inverted).

pins13-14             TTLlevels,activelow,

1Kohmpullupresistorsto+5VDC.

-----HardDiskPortPinAssignments---------------

ST                                 DB19S

----                                   ----

1|<---Data0--------------------->|

2|<---Data1--------------------->|

3|<---Data2--------------------->|

4|<---Data3--------------------->|

5|<---Data4--------------------->|

6|<---Data5--------------------->|

7|<---Data6--------------------->|

8|<---Data7--------------------->|

9|----ChipSelect---------------->|

MFP             10|<---InterruptRequest-----------|

11|----Ground----------------------|

12|----Reset---------------------->|

13|----Ground----------------------|

14|----Acknowledge---------------->|

15|----Ground----------------------|

16|----A1------------------------->|

17|----Ground----------------------|

18|----Read/Write----------------->|

19|<---DataRequest----------------|

----                                   ----

SignalCharacteristics

pins1-8               TTLlevels.

pin9                  TTLlevels,activelow.

pin10                 TTLlevels,activelow,

1Kohmpullupresistorto+5VDC.

pin12                 TTLlevels,activelow,

systemreset.

pin14                 TTLlevels,activelow.

pins16,18             TTLlevels.

pin19                 TTLlevels,activelow,

1Kohmpullupresistorto+5VDC.

9[..........................................]0110

6.Components

ThestandardconfigurationsoftheAtariST main sys-tem,

graphicssubsystem,musicsubsystem,anddevicesubsys-temsare

madeupofthefollowingmajorhardwarecomponents:

Main           o 8MHzMC68000MicroprocessorUnit

o MK68901MultiFunctionPeripheral

o 192KbyteROM

o 512Kbyteor1MbyteRAM

o MemoryControllerChip

o ControlLogicChip

o DMAControllerChip

Graphics       o 32KbyteDisplayMemory(frommainRAM)

o VideoShiftRegisterChip

Music          o YM-2149ProgrammableSoundGenerator

Device         o AtariIntelligentKeyboard(ikbd)

1MHzHD6301V1MicrocomputerUnit

o AtariTwoButtonMouse

o 2MC6850AsynchronousCommunications

InterfaceAdapters

o WD1772FloppyDiskController

7. Expansion

TheAtariSTcanbe expanded by cartridge ROM onlywhich

cancontainamaximumof128Kbyte. Thedisclosureofcartridge

presence or memory size is not supported inhardware.

The ROM cartridge slot hasthefollowingpinassignments

(onlythelower15addresslinesareavailable):

-----ROMPortPinAssignments---------------

ST                                    40S

----                                   ----

1|----Power+5VDC--------------->|

2|----Power+5VDC--------------->|

3|<---Data14---------------------|

4|<---Data15---------------------|

5|<---Data12---------------------|

6|<---Data13---------------------|

7|<---Data10---------------------|

8|<---Data11---------------------|

9|<---Data8----------------------|

10|<---Data9----------------------|

11|<---Data6----------------------|

12|<---Data7----------------------|

13|<---Data4----------------------|

14|<---Data5----------------------|

15|<---Data2----------------------|

16|<---Data3----------------------|

17|<---Data0----------------------|

18|<---Data1----------------------|

19|----Address13----------------->|

20|----Address15----------------->|

21|----Address8------------------>|

22|----Address14----------------->|

23|----Address7------------------>|

24|----Address9------------------>|

25|----Address6------------------>|

26|----Address10----------------->|

27|----Address5------------------>|

28|----Address12----------------->|

29|----Address11----------------->|

30|----Address4------------------>|

31|----ROM3Select---------------->|

32|----Address3------------------>|

33|----ROM4Select---------------->|

34|----Address2------------------>|

35|----_UpperDataStrobe--------->|

36|----Address1------------------>|

37|----_LowerDataStrobe--------->|

38-40|----Ground----------------------|

----                                   ----

8. MemoryMap

Thefirst2KbyteofST memory is reserved for the

exception vector table and supervisor stack. Thisarea

alongwithI/Ospaceisprotectedforsupervisor references

only.  Accessing supervisor protected areaswhileintheuser

statewillresultinabuserror. A4wordportion ofROM is

shadowed at the startofRAMfortheresetstackpointerand

programcounter. Writingtothis area or anyROMlocation

willalsoresultinabuserror. ThefollowingisamapofST

memory:

-----STMemoryMap---------------

----------------

000000  |ROM           |  Reset: SupervisorStackPointer

000004  |ROM           |  Reset: ProgramCounter

000008  |RAM           |  0KbyteRAM

----------------

.

----------------

080000  |RAM           |  512KbyteRAM

----------------

.

----------------

100000  |RAM           |  1MbyteRAM

----------------

.

.

.

----------------

fa0000  |ROM           |  320KbyteROM

----------------

.

----------------

fc0000  |ROM           |  Reset: SupervisorStackPointer

fc0004  |ROM           |  Reset: ProgramCounter

fc0008  |ROM           |  192KbyteROM

feffff  |ROM           |  0KbyteROM

----------------

.

.

.

----------------

ff8000  |I/O           |  ConfigurationRegisters

ff8200  |I/O           |  DisplayRegisters

ff8400  |I/O           |  Reserved

ff8600  |I/O           |  DMA/DiskRegisters

ff8800  |I/O           |  SoundRegisters

----------------

.

----------------

fffa00  |I/O           |  MC68xxxRegisters

fffc00  |I/O           |  MC68xxRegisters

----------------

9. I/OMap

TheSTI/Ospacerangesfromff0000toff ffff, with

MC68000andMC6800peripheralinternalregistersstartingatff

fa00andfffc00respectively.  Accessing reserved I/O

addresses willresultinabuserror. Bitvaluesforvari-ous

read and/or write registers are labeled as active

One/_Zero (alwaysmaskoutunusedfieldbits). Thefollow-ing

isamapofSTI/Ospace:

_____________

Configuration

ff8001  R/W     |----xxxx|  MemoryConfiguration

||||

-------  Bank0     Bank1(notused)

0000      128Kbyte 128Kbyte

0001      128Kbyte 512Kbyte

0010      128Kbyte   2Mbyte

0011      Reserved

0100      512Kbyte 128Kbyte

0101      512Kbyte 512Kbyte

0110      512Kbyte   2Mbyte

0111      Reserved

1000        2Mbyte 128Kbyte

1001        2Mbyte 512Kbyte

1010        2Mbyte   2Mbyte

1011      Reserved

11xx      Reserved

_______

Display

ff8201  R/W            |xxxxxxxx|  VideoBaseHigh

ff8203  R/W            |xxxxxxxx|  VideoBaseLow

ff8205  R              |xxxxxxxx|  VideoAddressCounterHigh

ff8207  R              |xxxxxxxx|  VideoAddressCounterMid

ff8209  R              |xxxxxxxx|  VideoAddressCounterLow

ff820a  R/W            |------xx|  SyncMode

||

|----  External/_InternalSync

-----  50Hz/_60HzFieldRate

ff8240  R/W    |-----xxx-xxx-xxx|  PaletteColor0/0(Border)

|||||||||

||||||||----  Invert/_NormMono

|||||||||

|||||| ------  Blue

||| ----------  Green

--------------  Red

ff8242  R/W    |-----xxx-xxx-xxx|  PaletteColor1/1

ff8244  R/W    |-----xxx-xxx-xxx|  PaletteColor2/2

ff8246  R/W    |-----xxx-xxx-xxx|  PaletteColor3/3

ff8248  R/W    |-----xxx-xxx-xxx|  PaletteColor4

ff824a  R/W    |-----xxx-xxx-xxx|  PaletteColor5

ff824c  R/W    |-----xxx-xxx-xxx|  PaletteColor6

ff824e  R/W    |-----xxx-xxx-xxx|  PaletteColor7

ff8250  R/W    |-----xxx-xxx-xxx|  PaletteColor8

ff8252  R/W    |-----xxx-xxx-xxx|  PaletteColor9

ff8254  R/W    |-----xxx-xxx-xxx|  PaletteColor10

ff8256  R/W    |-----xxx-xxx-xxx|  PaletteColor11

ff8258  R/W    |-----xxx-xxx-xxx|  PaletteColor12

ff825a  R/W    |-----xxx-xxx-xxx|  PaletteColor13

ff825c  R/W    |-----xxx-xxx-xxx|  PaletteColor14

ff825e  R/W    |-----xxx-xxx-xxx|  PaletteColor15

ff8260  R/W            |------xx|  ShiftMode

||

00      320x200,4Plane

01      640x200,2Plane

10      640x400,1Plane

11      Reserved

________

RESERVED

ff8400          |----------------|  Reserved

________

DMA/Disk

ff8600          |----------------|  Reserved

ff8602          |----------------|  Reserved

ff8604  R/W    |--------xxxxxxxx|  DiskController

(WordAccess)

ff8606  R      |-------------xxx|  DMAStatus(WordAccess)

|||

||----  _ErrorStatus

|----- SectorCountZeroStatus

------DataRequestInactiveStatus

ff8606  W      |-------xxxxxxxx-|DMAModeControl(WordAccess)

||||||||

|||||||-----  A0

||||||------  A1

|||||-------  HDC/_FDCRegiSel

||||--------  SectorCountRegSel

|||0           Reserved

||----------  Disable/_EnableDMA

|-----------  FDC/_HDC

------------  Write/_Read

ff8609  R/W            |xxxxxxxx|  DMABaseandCounterHigh

ff860b  R/W            |xxxxxxxx|  DMABaseandCounterMid

ff860d  R/W            |xxxxxxxx|  DMABaseandCounterLow

_____

Sound

ff8800  R              |xxxxxxxx|  PSGReadData

||||||||      I/OPortB

--------------  ParallelInterfaceData

ff8800  W              |xxxxxxxx|  PSGRegisterSelect

||||

-------  RegisterNumber

0000      ChannelAFineTune

0001      ChannelACoarseTune

0010      ChannelBFineTune

0011      ChannelBCoarseTune

0100      ChannelCFineTune

0101      ChannelCCoarseTune

0110      NoiseGeneratorControl

0111      MixerControl-I/OEnable

1000      ChannelAAmplitude

1001      ChannelBAmplitude

1010      ChannelCAmplitude

1011      EnvelopePeriodFineTune

1100      EnvelopePeriodCoarseTune

1101      I/OPortA(OutputOnly)

1111      I/OPortB

ff8802  W     |xxxxxxxx|  PSGWriteData

||||||||      I/OPortA

|||||||-------  FloppySide0/_Side1Select

||||||--------  Floppy_Drive0Select

|||||---------  Floppy_Drive1Select

||||----------  RS232RequestToSend

|||-----------  RS232DataTerminalReady

||------------  Centronics_STROBE

|-------------  GeneralPurposeOutput

--------------  Reserved

||||||||      I/OPortB

--------------  ParallelInterfaceData

_______

MC68xxx

fffa01                  |xxxxxxxx|  MFPGeneralPurposeI/O

fffa03                  |xxxxxxxx|  MFPActiveEdge

fffa05                  |xxxxxxxx|  MFPDataDirection

fffa07                  |xxxxxxxx|  MFPInterruptEnableA

fffa09                  |xxxxxxxx|  MFPInterruptEnableB

fffa0b                  |xxxxxxxx|  MFPInterruptPendingA

fffa0d                  |xxxxxxxx|  MFPInterruptPendingB

fffa0f                  |xxxxxxxx|  MFPInterruptIn-ServiceA

fffa11                  |xxxxxxxx|  MFPInterruptIn-ServiceB

fffa13                  |xxxxxxxx|  MFPInterruptMaskA

fffa15                  |xxxxxxxx|  MFPInterruptMaskB

fffa17                  |xxxxxxxx|  MFPVector

fffa19                  |xxxxxxxx|  MFPTimerAControl

fffa1b                  |xxxxxxxx|  MFPTimerBControl

fffa1d                  |xxxxxxxx|  MFPTimersCandDCtl

fffa1f                  |xxxxxxxx|  MFPTimerAData

fffa21                  |xxxxxxxx|  MFPTimerBData

fffa23                  |xxxxxxxx|  MFPTimerCData

fffa25                  |xxxxxxxx|  MFPTimerDData

fffa27                  |xxxxxxxx|  MFPSyncCharacter

fffa29                  |xxxxxxxx|  MFPUSARTControl

fffa2b                  |xxxxxxxx|  MFPReceiverStatus

fffa2d                  |xxxxxxxx|  MFPTransmitterStatus

fffa2f                  |xxxxxxxx|  MFPUSARTData

______

MC68xx

fffc00                  |xxxxxxxx|  KeyboardACIAControl

fffc02                  |xxxxxxxx|  KeyboardACIAData

fffc04                  |xxxxxxxx|  MIDIACIAControl

fffc06                  |xxxxxxxx|  MIDIACIAData

10. InterruptTable

ThefollowingtableslisttheSTinterrupt and signal

priorityassignments:

-----MC68000InterruptAutovector---------------

-----------------------------------------------

|Level        |Definition                    |

-----------------------------------------------

|7(HIGHEST)  |NMI                           |

|6            |MK68901MFP                   |

|5            |                               |

|4            |VerticalBlanking(Sync)      |

|3            |                               |

|2            |HorizontalBlanking(Sync)    |

|1(LOWEST)   |                               |

-----------------------------------------------

NOTE: onlyinterruptprioritylevelinputs1and2areused.

-----MK68901InterruptControl---------------

-----------------------------------------------

|Priority     |Definition                    |

-----------------------------------------------

|15(HIGHEST) |MonochromeMonitorDetect   I7|

|14           |RS232RingIndicator        I6|

|13           |SystemClock/BUSY         TA|

|12           |RS232ReceiveBufferFull     |

|11           |RS232ReceiveError           |

|10           |RS232TransmitBufferEmpty   |

| 9           |RS232TransmitError          |

| 8           |HorizontalBlankingCounter TB|

| 7           |DiskDriveController       I5|

| 6           |KeyboardandMIDI           I4|

| 5           |TimerC                     TC|

| 4           |RS232BaudRateGenerator   TD|

| 3           |GPUOperationDone          I3|

| 2           |RS232ClearToSend         I2|

| 1           |RS232DataCarrierDetect   I1|

| 0(LOWEST)  |CentronicsBUSY             I0|

-----------------------------------------------

NOTE: theMC6850ACIAInterruptRequeststatusbitmustbe

testedtodifferentiatebetweenkeyboardandMIDIinterrupts.

11. CaseDesIgn

TheSTisprimarilydesigned as a keyboard computerwith

externalvideodisplay,diskdrives,andpowersupply.

Thetoppanelcontainsthekeyboard,ventilationslots(bot-tom

also), and poweronLED. Thesidepanelscontaintheexpan

sionROMslotandmouse/joystickports. Thebackpanelcontains

the on/off switch, resetbutton,channelselectswitch,power

connector,andall remaining device connec-tors:  video,

parallel,RS232,MIDI,anddiskdriveinter-faces.

Thedeviceportsarelabeledwith International Elec-tro

technicalCommission-likesymbolsincombinationsofpic-tograms

andphonetictranscriptions(captions). InthecaseofMIDI,the

specificationexplicitlyrequiresthattheMIDIconnectorsbe

labeledMIDIOUT,MIDIIN,andMIDI THRU (ifapplicable).

12. PowerSupply

AnexternalDCpowersupplyprovidespowertothe mainsystem

board, keyboard, expansionROM,andexpansionRAM.

Allpowerlevelsareregulatedfor over-voltage and over-

current protection. Thefollowingareminimalpowersupply

specifications:

ACInput               115VAC10%at60Hz(fused)

DCOutput              +5VDCat3A5%

+12VDCat.03A10%

-12VDCat.03A10%

-----PowerPortPinAssignments---------------

ST                                   DIN7P

----                                   ----

1|<---+5VDC----------------------|

3|----Ground----------------------|

4|<---+12VDC---------------------|

5|<----12VDC---------------------|

6|<---+5VDC----------------------|

7|----Ground----------------------|

----                                   ----

AppendixA--ikbdKeyboardLayout

ThesurfaceoftheAtariIntelligentKeyboardisformedfrom

four distinct ergonomic units: aQWERTYtypewritermatrix,a

functionkeyarray,ascreencontrolcluster, anda calculator

numerickeypad. Asawhole,theUnitedStatesversionofthe

keyboardlayoutisDEC VT100-like with theexceptionofthe

followingitems:

o removedkeys--[break],[linefeed],[noscroll],[setup].

o repositionedkeys--[capslock],cursorcontrolkeys,

programmablefunctionkeys.

o addedkeys--[alternate],[help],[undo],[insert],

[clear/home],10programmablefunctionkeys.

o improvements--DECVT200-styleinvertedTcursorcontrol

keys,fullcalculatorfunctionnumerickeypad.

Pleasenotethattheleft[shift]keyis intentionallylarge

toallowspaceforanextraISOrequiredkey(seeISO2530-

1975).  Atari ikbd International Layouts will  bedesigned

forthefollowingcountries:

o UnitedKingdom

o UnitedStates

o Finland

o Norway/Denmark

o Sweden

o Japan

o Germany

o FrenchCanada

o France

o Italy

o Spain

NOTE: the'HELP'and'UNDO'keyswillbeleftuntranslated

AppendixB--References

General

AHitchhiker'sGuidetotheBIOS

DigitalResearchGEMSoftwareDocumentation

MainSystem

MotorolaMC6800016-BitMicroprocessor User's Manual,

FourthEdition

MostekMK68901MultiFunctionPeripheralDataSheet

GraphicsSubsystem

AdeleGoldbergandDavid Robson, 'Smalltalk-80:  The

LanguageandItsImplementation',Addison-Wesley,Read-

ingMassachusetts,1983,Chapter18.

MusicSubsystem

GeneralInstrumentAY-3-8910ProgrammableSoundGenera-

torDataSheet

MIDIMusicalInstrumentDigitalInterfaceSpecification

1.0

DeviceSubsystems

AtariIntelligentKeyboard(ikbd)ProtocolandSpecifi-

cation

MotorolaMC6850Asynchronous Communications Interface

AdapterDataSheet

CentronicsParallelInterfaceSpecification

ElectronicIndustriesAssociationRS232CStandard

WesternDigitalWD1770/1772FloppyDiskControllerData

Sheet

SpecificationoftheAtari Computer System Interface

(ACSI)

SpecificationoftheAtariHardDiskInterface(AHDI)

AppendixC--Notes

General

851125A  Anaddresserroroccurswhenawordinstructionis

usedonabyteaddress.

MainSystem

841017B  TheDMABaseAddressandCounterRegistermust be

loadedinlow,mid,highorder.

GraphicsSubsystem

841017C  None.

MusicSubsystem

841017D  TheYM-2149PSGI/Ospaceandregistersshould be

setupascriticalregionsinsoftware.

DeviceSubsystems

841017F  PollorservicetheDiskDrive Controller inter-

ruptontheMK68901MFPGeneralPurposeI/ORegis-

tertodetectthecompletionofaWD1772FDC com-

mand.  Do not poll the FDCBusyorDMASector

CountZerostatusbits.

841017G  SelecttheSectorCount Register before testing

theDMAStatusRegisterErrorbit.

841017H  Donotsetthe30msSettlingDelaybiton WD1772

FDCtype2and3commandexecutions.

841017I  Aforceinterruptshouldbe issued after a few

seconds (ietimeout)onallcommandssenttothe

WD1772FDC.

841017J  WaituntiltheWD1772FDCMotorOnstatus is low

beforedeselectingafloppydrive.

841017K  Afloppydiskdriveconfigurationtableshould be

maintained in software toaccommodateadiverse

selectionof3.5inch floppy disk drives.  Two

floppydiskdrivescurrentlyunderevaluationhave

thefollowingcharacteristics:

o 500Kbyteunformatted,80cylinders,one head,

3mssteppingrate.

o 1Mbyteunformatted,80cylinders,twoheads,3

mssteppingrate.

TableofContents

1. SystemArchitecture...............................   1

2. MainSystem.......................................   4

2.1. MicroprocessorUnit...........................   4

2.2. MemoryConfiguration..........................   4

2.3. DirectMemoryAccess..........................   5

3. GraphicsSubsystem................................   7

3.1. VideoDisplayMemory..........................   7

3.2. VideoConfiguration...........................   8

4. MusicSubsystem...................................  10

4.1. SoundSynthesizer.............................  10

4.2. MusicalInstrumentCommunication..............  10

5. DeviceSubsystems.................................  12

5.1. IntelligentKeyboard..........................  12

5.2. VideoInterface...............................  14

5.3. ParallelInterface............................  15

5.4. RS232Interface...............................  16

5.5. MIDIInterface................................  17

5.6. DiskDriveInterface..........................  18

6. Components........................................  23

7. Expansion.........................................  24

8. MemoryMap........................................  25

9. I/OMap...........................................  27

10. InterruptTable..................................  31

11. CaseDesign......................................  32

12. PowerSupply.....................................  33

AppendixA--ikbdKeyboardLayout....................  34

AppendixB--References..............................  35

AppendixC--Notes...................................  36