--------------------------------------------------------------------------

How to Make an Economical High Voltage / High Frequency Capacitor

First Published by Information Unlimited

Transcribed to the electronic media for you by Thallion of WUFO MCMXCIV.

--------------------------------------------------------------------------

The following information shows how to make a reasonable reliable

capacitor that can be used for medium powered tesla coils [or other High

Voltage projects]. Higher powered operation is also possible by simply

scaling up the dimensions but reliability becomes critical at voltages over

10KV.

The design is not new as it was often used in the old days of radio

where resonant radio frequency circuits required a ringing type low loss

device with reverse recovery properties.

The approach shown here utilizes certain dimensions of pieces we

happened to have laying around our lab, however, deviations from these

values work, but will give different amounts of capacitance. This change of

parameters is easily calculated by the following formula:

.224

Capacitance =   --- * KA(N-1)

D

K = Dielectric constant [Glass = 7, Plexiglass = 3.5 etc.]

A = Active area of conductive plates in square inches

D = Dielectric thickness in inches

N = Number of plates

CONSTRUCTION STEPS:

-------------------

1. Obtain 11 pieces of 8x6x1/8 thick window glass from your local hardware

store. Thoroughly wash and dry pieces to remove all handprints and dirt.

This is very important.

2. Carefully and accurately cut 10 pieces of aluminum foil or thin gauge

sheet metal as shown below. Use a template of approximately the shape

shown. Note to round all sharp corners to reduce potential corona

points. Pieces must be flat and ripple free.

3. Find a large clean flat surface such as a table and lay out 2 rows of 5

pieces of glass each. Identify as Row I and Row II.

4. Coat each piece of glass with mineral oil using a small brush. This

should provide an adhesive action to the foil and glass.

A. Assemble 5 pieces by laying on cut foil pieces as shown in assembly

I here below.

B. Repeat for 5 pieces as shown in assembly II.

Note that the foil pieces are identical between assembly I and II, the

difference being opposite surfaces being used.

-------------------------------------------------------------------------

PICTURE A:

_______________________   Connection Tabs      _______________________

|  _____________________|______    |     ______|_____________________  |

| |                         ___) <-+->  (___                         | |

| |                        /                \                        | |

| |                      /                    \                      | |

| |  Aluminum foil     /                        \      Al foil       | |

| |                  /  |                      |  \                  | |

| |________________/    |                      |    \________________| |

|_______________________| <-- Glass plates --> |_______________________|

Assembly I                                         Assembly II

* Note that the aluminum foil should have round corners.

--------------------------------------------------------------------------

5. Very carefully alternately stack the assemblies from I and II into a

sandwich configuration. Do this step carefully as foil pieces must not

be allowed to shift on glass or breakdown will definitely occur. Note

odd piece of glass for sandwiching last outer foil surface.

6. Bind sandwiched assembly together using tape, nylon string, tyewrap or

similar. Whatever you use, make sure it does not conduct.

7. Attach all of the connection tabs on one end of assembly together. This

may be done folding over tabs and crimping together. A hole may be

placed in the tab cluster for attaching connecting wires via screw and

nut. Repeat for other end of assembly. Again be careful not to shift

foil on plate.

--------------------------------------------------------------------------

PICTURE B:

=========  <- Glass plate

*********  <- Aluminum foil / Leads

=======================================

*****************************************

=======================================   *

*****************************************     *

*   =======================================   *

*     *****************************************

*   =======================================   *

*****************************************     *

*   =======================================   *

*     *****************************************

*   =======================================   *

*****************************************     *

*   =======================================   *

*     *****************************************

*   =======================================   *

*****************************************     *

*   =======================================   *

*     *****************************************

*   =======================================   *

*****************************************     *

*   =======================================   *

*                                             *

* <- Lead 1                                   *  <- Lead 2

Exaggerated view of assembly showing alternating foil plates.

--------------------------------------------------------------------------

8.  Locate a plastic container such as a cake pan or other similarly shaped

object. Place the capacitor assembly in this container and immerse in

mineral oil.

9.  At this stage of the assembly it is a good idea to measure the capacity

using a low cost LCR bridge. Value should be approximately that shown

by formula. Fine adjustments can be done by removing part of the outer

foil surfaces or even adding another plate.

10. The following step involves placing the entire assembly under a vacuum

and pumping out the air bubbles. This may take several hours and will

greatly help in allowing reliable operation. A less effective but other

acceptable method is to slowly apply voltage to the assembly in

increments using a variac. Air pockets now are vibrated out by the

mechanical motion of the plates.

--------------------------------------------------------------------------