Diagnostic Error Codes for the IBM PC, PCXT, and PC-AT.

Code Description

==============================================================================

101    Main system board failed.

109    Direct memory access test error

121    Unexpected hardware interrupts occurred.

131    Cassette wrap test failed.

199    User indicated configuration not correct.

201    Memory test failed

301    Keboard did not respond to software reset correctly or a stuck

key failure was detected. If a stuck key was detected, the scan

code for the key is displayed.

302    User indicated error from the keyboard test.

401    Monochrome memory test, horizontal sync frequency test, or video

Test failed.

408    User indicated display attributes failure.

416    User indicated character set failure.

424    User indicated 80X25 mode failure.

432    Parallel port test failed ( monochrome adapter ).

501    Color memory test failed, horizontal sync frequency test, or

video test failed.

508    User indicated display attribute failure.

516    User indicated character set failure.

524    User indicated 80x25 mode failure.

532    User indicated 40x25 mode failure.

540    User indicated 320x200 graphics mode failure.

548    User indicated 640x200 graphics mode failure.

601    Diskette power on diagnostics test failed.

602    Diskette test failed

606    Diskette verify function failed.

607    Write protected diskette.

608    Bad command diskette status returned.

610    Diskette initialization failed.

611    Timeout - diskette status returned.

612    Bad NEC - diskette status returned.

613    Bad DMA - diskette status returned.

621    Bad seek - diskette status returned.

622    Bad CRC - diskette status returned.

623    Record not found - diskette status returned.

624    Bad address mark - diskette status returned.

625    Bad NEC seek - diskette status returned.

626    Diskette data compare error.

7xx    8087 math coprocessor

901    Parallel printer adapter test failed.

10xx   Reserved for parallel printer adapter.

1101   Asynchronous communications adapter test failed.

1201   Alternate Asynchronous communications adapter test failed.

1301   Game control adapter test failed.

1302   Joystick test failed.

1401   Printer test failed.

15xx   Sdlc communications adapter errors.

1510   8255 port B failure.

1511   8255 port A failure.

1512   8255 prot C failure.

1513   8253 timer 1 did not reach terminal count.

1514   8253 timer 1 stuck on.

1515   8253 timer 0 did not reach terminal count.

1516   8253 timer 0 stuck on.

1517   8253 timer 2 did not reach terminal count.

1518   8253 timer 2 stuck on.

1519   8273 port B error.

1520   8273 port A error.

1521   8273 command/read timeout.

1522   Interrupt level 4 failure.

1523   Ring Indicate stuck on.

1524   Receive clock stuck on.

1525   Transmit clock stuck on.

1526   Test indicate stuck on.

1527   Ring indicate not on.

1528   Receive clock not on.

1529   Transmit clock not on.

1530   Test indicate not on.

1531   DATA set ready not on.

1532   Carrier detect not on.

1533   Clear to send not on.

1534   DATA set ready stuck on.

1536   Clear to send stuck on.

1537   Level 3 interrupt failure.

1538   Receive interrupt results error.

1539   Wrap data miscompare.

1540   DMA channel 1 error.

1541   DMA channel 1 error.

1542   Error in 8273 error checking or status reporting.

1547   Stray interrupt level 4

1548   Stray interrupt level 3

1549   Interrupt presentation sequence timeout.

17xx   Fixed Disk errors.

1701   Fixed disk Post error.

1702   Fixed disk adapter error.

1703   Fixed disk drive error

1704   Fixed disk adapter or drive error.

18xx   I/O Expansion unit errors.

1801   I/O Expansion unit POST error.

1810   Enable/Disable failure.

1811   Extender card wrap test failed (disabled).

1812   High order addtess lines failure (disabled).

1813   Wait state failure (disabled).

1814   Enable/Diaable could not be set on.

1815   Wait state failure (enabled).

1816   Extender card wrap test failed (enabled).

1817   High order adress lines failure (enabled).�j

1818   Disable NOT functioning

1819   Wait request switch not set correctly

1820   Receiver card wrap test failure

1821   Receiver high order address lines failure

20xx   Bisync communications adapter errors

2010   8255 port a failure

2011   8255 port b failure

2012   8255 port c failure

2013   8253 timer 1 did not reach terminal count

2014   8253 timer 1 stuck on

2016   8253 timer 2 did not reach terminal count or timer 2 stuck on.

2017   8251 Data set ready failed to come on

2018   8251 Clear to send not sensed

2019   8251 Data set ready stuck on

2020   8251 Clear to send stuck on

2021   8251 hardware reset failed

2022   8251 software reset failed

2023   8251 software "error reset" failed

2024   8251 transmit ready did not come on

2025   8251 receive ready did not come on

2026   8251 could not force "overrun" error status

2027   Interrupt failure-no timer interrupt

2028   Interrupt failure-transmit, replace card or planar

2029   Interrupt failure-transmit, replace card

2030   Interrupt failure-receive, replace card or planar

2031   Interrupt failure-receive, replace card

2033   Ring indicate stuck on

2034   Receive clock stuck on

2035   Transmit clock stuck on

2036   Test indicate stuck on

2037   Ring indicate stuck on

2038   Receive clock not on

2039   Transmit clock not on

2040   Test incicate not on

2041   DATA set ready not on

2042   Carrier detect not on

2043   Clear to send not on

2044   DATA set ready stuck on

2045   Carrier detect stuck on

2046   Clear to send stuck on

2047   Unexpected transmit interrupt

2048   Unexpected receive interrupt

2049   Transmit data did not equal receive data

2050   8251 detected overrun error

2051   Lost data set ready during data wrap

2052   Receive timeout during data wrap

21xx   Alternate bisync communications adapter errors

2110   8255 port a failure

2111   8255 port b failure

2112   8255 port c failure

2113   8253 timer 1 did not reach terminal count

2114   8253 timer 1 stuck on

2116   8253 timer 2 did not reach terminal count or

2117   8251 Date set ready failed to come on

2117   8251 Clear to send not sensed

2118   8251 Data set ready stuck on

2119   8251 Clear to send stuck on

2120   8251 hardware reset failed

2121   8251 software reset failed

2122   8251 software "error reset" failed

2123   8251 transmit ready did not come on

2124   8251 receive ready did not come on

2125   8251 could not force "overrun" error status

2126   Interrupt failure-no timer interrupt

2128   Interrupt failure-transmit, replace card or

2129   Interrupt failure-transmit, replace card

2130   Interrupt failure-receive, replace card or planar

2131   Interrupt failure-receive, replace card

2133   Ring indicate stuck on

2134   Receive clock stuck on

2135   Transmit clock stuck on

2136   Test indicate stuck on

2137   Ring indicate stuck on

2138   Receive clock not on

2139   Transmit clock not on

2140   Test incicate not on

2142   DATA set ready not on

2142   Carrier detect not on

2143   Clear to send not on

2144   DATA set ready stuck on

2145   Carrier detect stuck on

2146   Clear to send stuck on

2147   Unexpected transmit interrupt

2148   Unexpected receive interrupt

2149   Transmit data did not equal receive data

2150   8251 detected overrun error

2151   Lost data set ready during data wrap

2152   Receive timeout during data wrap

All personal computer error codes for the diagnostic and advanced

diagnostic package are represented with the device number followed by two

digits other than 00. The device number plus 00 represents successful

completion of the test.

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X