IBM MIDI INTERFACE

Version 1.0

Harley A. Burton

Copyright 1987

Burtech Ind.

GENERAL

The  Musical  Instrument Digital Interface (MIDI) worked out  by

Casio, Roland, and others a few years ago has be come the widely

accepted  standard  of  the  musical  industry  for  interfacing

everything from keyboard synthesizers, tape recorders, mixers to

drum  machines and everything else conceivable.   The impact  of

this  technology  on  the  music  industry  has  been  dramatic.

Although  hardware and software exists for this  technology,  in

general it has been quite expensive.   As an example, the Roland

MPU401  interface,  the most powerful interface for  the  IBM-PC

family of computers is about $600 list price.   While there have

been  others developed for a lower cost,  for example the  board

listed  in  the  June  1986 BYTE  magazine  article,  these  are

somewhat  complex.   I hope to offer  the  hobbiest/experimenter

with  a lower cost alternative,  and yet a challenge to  develop

some or all of the software as a project.

Without going deeply into the technical side of MIDI,  it  seems

to  at  first  scare the world.   It runs at a high  data  rate,

32.1250Kbps,  use the odd interface of approximately 5ma current

loop  and  be optically coupled.   This  sounds  like  something

totally out of the realm of the home hobbiest.   Not at all,  in

fact, it is not only possible, it is quite easy.

Originally, IBM made the Asynchronous Communications Adapter for

our IBM-PC's and they didn't know anything about MIDI.  They did

however  give us the tools we need that can be used to interface

to MIDI systems with minimum hardware modification and  software

generation.

From here on, I will be discussing modifying an IBM Asynchronous

Communications Adapter to work with MIDI systems.   While I have

no  experience with clone serial communications boards,  I  will

assume that they could be modified similarly and will attempt to

do  it sometime.

This  discussion  will be quite technical,  but not  beyond  the

average experimenter.   I hope you will try the ideas  presented

here  and  pass your findings along to all of us on the  various

BBS's.

DATA RATE GENERATION

The original IBM Asynchronous Communications Adapter was capable

of serial data rates from about 45 baud to 19200 bits per second

in 5,6,7 and 8 data bits with 1,  1.5 and 2 stop bits, even, odd

or no parity.   In MIDI,  we would like to run at 31250 bits per

second,  8 data bits,  no parity and one stop bit.   Well,  just

change    the   initialization   number,    we   would    think.

Unfortunately,  the  comm port can't be programmed as it  stands

for  that baud rate which brings us to the  first  modification.

The  baud  rate  timing  in  the comm port  is  generated  by  a

18.432#MHz oscillator labeled in my schematic as U10.   This  is

then  divided by 10 in U6 to get 1.8432#MHz,  the standard  baud

rate generator.   The 8250 UART U11 which actually does all  the

work  has an on-chip oscillator which could be used to  generate

the desired frequency.

As it turns out,  by removing U6,  the 7490 used to divide by 10

and  connecting  a 3#MHz crystal between pins 16 and 17 of  U11,

the  8250,  the  comm port will now operate at  31250  bits  per

second, the MIDI standard.

The comm port will no longer operate as a standard serial board,

but who cares.  These things are cheap enough to convert to MIDI

permanently.

On the Serial/Printer Adapter which is the type used in the IBM-

PC/AT,   the   generation  is  slightly  different.    The  base

oscillator is 1.8432#MHz as it comes from the factory.   This is

buffered in U11,  a 74LS05.   The final serial port device is  a

NS#16450,  an  upgraded 8250.   This board requires breaking the

foil trace between pin 2 on U11 and pin 16 on U5,  the NS#16450.

This can be simply a cut with an X-Acto knife.   Then connect  a

3#MHz crystal between pins 16 and 17 as in the other board.

The  maximum operating frequency of both boards is 3.1#MHz,  but

there  is adequate "head room" for this to work  satisfactorily.

It also turns out that by using the 3#Mhz crystal,  the internal

divisor ratio is such that when you tell the port to  initialize

for 19200#bps,  it will actually configure itself for 31250#bps.

The ratio of 3.0#Mhz to 1.8432#MHz equals the ratio of 31250#bps

to   19200#bps  (3/1.8432  =  31250/19200).    This  will   have

implications which will be described later in the software.

5 Ma CURRENT LOOP

The   original  IBM  Asynchronous  Communications  Adapter   had

provisions  for the archaic 20 ma current loop  standard.   This

has  disappeared  on the Serial/Printer Adapter and  most  clone

adapters.   For our purposes,  this is a shame.  I will describe

first  converting the IBM Asynchronous Communications Adapter to

5#ma  current  loop  then provide some  thoughts  on  the  other

adapters.

On  the  IBM  Asynchronous  Communications  Adapter,  the  20#ma

current  loop was configured by turning jumper block XU5  around

opposite   of  that  required  by  RS-232  (yes,   I  am   being

intentionally vague, because I can't remember off hand which way

that  is).   The 20#ma current loop is provided by 3 sections of

U8  paralleled  and  driving the 20#ma loop through  a  100  ohm

resistor,  R3.  By changing this resistor to a 390 ohm resistor,

the  current should be a little hot 5#ma.   A 470  ohm  resistor

could  be used if you are not driving long 5#ma  lines.   Either

will be within safety range of the MIDI requirements.   A 220 in

series  with a 180 ohm,  would of course be exact.   Any of  the

above will work fine.

On the recieve side, the IBM Asynchronous Communications Adapter

already  has  an opto-coupler interface provided  by  U4.   This

should be useable without modification.

An adapter will be needed to go from the 25 Pin "D" connector on

the  back  of the Asynchronous Communications Adapter to a  MIDI

5#pin  DIN connector used for MIDI.   The parts for this can  be

purchased  from  Radio Shack and since I  don't  have  schematic

capability with this word processor, I won't go into it here.

For  a board other than the Asynchronous Communications  Adapter

such as a clone or the IBM Serial/Printer Adapter,  an RS-232 to

5#ma  adapter will need to be built.   I won't design one  here,

it's  really pretty trivial,  but I would suggest receiving  the

RS-232  with a MC1489 then driving the 5#ma with 3 sections of a

74LS04  tied  in parallel then driving MIDI through  a  390  ohm

resistor.   The  receive side could use any of the  optocouplers

that are usually specified for MIDI, such as the one in the BYTE

article mentioned above.   This could then drive a MC1488 RS-232

line  driver.   A  small  5#volt power supply will  have  to  be

provided in this interface.

I  might  suggest that if you have to build the RS-232  to  5#ma

converter,  use  some  of the extra sections of the  devices  to

build some MIDI through buffers, splitters, etc.

MIDI SOFTWARE

Without  going  into  the software in great  detail,  there  are

several  built-ins and hints that I can give which may  help  in

writing  your own software package.   All of the software  ideas

are  in  IBM BASIC for clarity.   These built-ins  are  probably

available in other languages as well.

The  first thing we will need is beat timing or tempo.   I  have

chosen the easy way out again.  I built a tempo generator with a

type 555 timer and a few components that generates a pulse at 24

times  the quarter note beat.   This allows me to generate  32nd

notes  and 16th triplets,  not that I would ever play that fast.

I  built  it  with  a range of 20 to 240  beats  per  minute  (8

pulses/second  to 96 pulses/second).   This I connected  to  the

trigger input of a game port.   Again, I had one and didn't feel

like  building a trigger timer board.   This I control manually,

but  you could use the internal timer section 2 of the  8253  on

your IBM-PC mother board.  I don't know how to do this and would

appreciate if anyone knows how, please tell me.

I  use  the  "ON  TRIG(1)"  statement in  BASIC  to  jump  to  a

subroutine in which I keep track of which portion of a beat I am

in.   This  provides  the  basic timing for  both  transmit  and

receive.   I  will come back to this later.  I wish that IBM had

given  us better resolution on the ON TIMER function,  it  could

have  been  used  as  a beat counter  but  with  only  1  second

resolution there isn't much you can do with it unless your music

is VERY VERY slow.

The  initialization  of  the  serial port  is  quite  simple  to

implement  for  MIDI.   I use the "OPEN  COM1:19200,N,8,1,CS,DS"

statement  to  initialize the port.   Remember that  because  we

changed  the crystal to 3#MHz,  opening at 19200 in software  is

actually  opening at 31250 bps.   This works!!!   I  don't  like

error  checking  in  hardware,  so  I disable the  CTS  and  DTS

signals.

I  have notes to be transmitted assigned to portions of a  beat.

These  are  built off-line by a conversion  routine  that  takes

information  from  the music processor SongWright and  builds  a

file  of notes and timing to be transmitted over MIDI.   I won't

go into how this works until I get it totally ready, but what it

does is this:

When a TRIG(1) interrupt occurs,  the software checks if  a

note  or group of notes are to be sent.   If they  are,  it

builds the group of notes with statements like

J$ = CHR$(14)+CHR$(23)+CHR$(77)...

Then transmits the string with a

PRINT#1,J$

Since the output is a buffered,  interrupt driven transmit,

the  notes are transmitted at 31250 bps.  with  no  further

assistance  from the user.   Unless there is a LOT of notes

to  be sent,  as in 16 voice chords,  along with  big  drum

sounds, they should sound correct.  This is a limitation of

the  transmission  media,  MIDI,  however and not  the  IBM

hardware.

I also use the "ON COM(1)" statement to set up a receive routine

for  MIDI.   This allows me to avoid polling the receive side so

that I can do other things.  When a note is received via MIDI, I

jump to a routine that gets the note,  then reads the portion of

beat counters to see what beat time I should assign to  it.   It

checks for several other things, like note off, system messages,

etc.   These  are  interpreted and used  where  appropriate.   I

assume  that  I can receive anything anytime and I have to  know

what  I  received  and  what to do with it so  this  routine  is

potentially a little longer than necessary.

CONCLUSION

I  have attempted to describe a hardware and software experiment

into  the fascinating world of MIDI.   I hope to  continue  with

this experiment and eventually build a full MIDI system with all

of  the  features we have dreamed about.   I would like  to  get

input from all you and hope you will help me in this experiment.

If you develop any software in this line,  please make it public

domain so that we all can enjoy it.

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X