100 watt RMS amplifier

Circuit Description:

This is a 100 watt basic power amp that was designed to be (relatively) easy

to build at a reasonable cost. It has  better performance (read: musical

quality) than the standard STK module amps that are used in practically

every mass market stereo receiver manufactured today.  When I originally

built this thing, it was because I needed a 100 WPC amp and didn't want

to spend any money. So I designed around parts I had in the shop.

The design is pretty much a standard one, and I'm sure there are commercial

units out there that are similar. To my knowlwdge, it is not an exact copy

of any commercial unit, nor am I aware of any patents on the topology.

To experienced builders: I realize that many improvements and refinements

can be made, but the idea was to keep it simple, and should be do-able

by anyone who can make a circuit board and has the patience not to

do a sloppy job.

The input stage is an LF351 op amp which provides most of the open loop gain

as well as stabilizes the quiescent dc voltage. This feeds a level shift

stage which references the voltage swing to the (-) rail.  The

transconductance stage is a darlington, to improve high-frerqency linearity.

The 2SC2344 by itself has a rather large collector-base capacitance

which is voltage dependent. The MPSA42 presents this with a low-z and has

a C(ob) of only a few pf that is effectively swamped by the 33pF

pole-splitting cap. The stage is supplied by the 2SA1011 active load

(current source) which is about 20 ma. The current to the stage is

limited by the 2N3094 to about 70 ma under worst case.

The output is a full complementary darlington with paralleled outputs.

Although you could "get away with" only one if only 8 ohm easy-to-drive

loads are used, this is not recommended. The use of parallel devices

increases the ability to drive reactive loads (which can pull a

significant current while the voltage waveform crosses zero and puts a

high voltage and a high curent across the transistor simultaneously),

gives the amp a higher damping factor, and reduces the maximum current

each transistor has to supply to peaks (remember, the gain of a

power transistor drops as the current increases).

Compensation is two-pole and one zero. The op-amp's pole and the pole

generated by the 33pf cap and the 470 ohm bias resistor of the MPSA42

dominate. (the 33pF gets multiplied by the stage gain.) The 22 pf

feedback capacitor provides lead compensation, and is taken from the

output of the tranconductance stage rather than the output itself.

In this way, the phase lag introduced by the output transistors is not

seen by the high-frequency feedback. This intorduces a closed-loop

pole which limits the high-frequency response.  The two compensation

capacitors must be type 1 creamic (NPO) or silver mica - with ZERO

voltage coefficient.

The amp was designed to run 2 channels off a +/- 55 volt unregulated supply,

reducing to +/- 48 volts under full load. It used a 40-0-40 volt, 5 amp

toroid transformer, a bridge rectifier, and 10,000 uf of filter cap per side.

If a standard EI transformer is used, a 6-amp rated unit should be used.

With this power supply, it produces 100 watts continuous, both channels

driven into 8 ohms resistive with no clipping. Dynamic headroom is about a

db and a half. For more headroom, unloaded voltages to +/- 62 volts can

be used with no circuit modification.

By the way, the schematic is in Postscript.

Limitations:

With no modifications the amp will drive 4-ohm speaker systems with no

current limiting. The short-circuit current limit is set to about 4.5 amps

peak, which will handle conventional speaker loads.(It will, of course,

produce higher peak currents as the output voltage swing approaches the

rail.) If you are going to be running some of those high-end speakers

with impedance minima of half an ohm, or that stay reactive throughout

most of the audio band ( ie, 0.5 +j3.2 ohms) you will probably already

own a better amp than this. If the higher-power Motorola power transistors

are used, it will drive a 2-ohm resistive load without problems (except

heat).

I have never heard any slew-induced distortion on this amp with a CD

player's band-limited (22KHz) signal. I suppose that real high-end

freaks could pick it to pieces by hitting it with a TTL square wave

mixed with a 19KHz stereo pilot tone and crank it up. I guarantee

that there will be spurs all over the spectrum, but who listens to

that?

Possible Modifications: (What if I want mo' power???)

The Toshiba output transistors (2SD424/2SB554 pair) shoud not be used with

supply voltages above +/-60 volts. If you plan on cranking it up, use

more in parallel or use the 250 watt Motorola pairs (MJ15024/MJ15025).

If very low impedances are expected, raise the bias in the  transconductance

stage to give more base drive to the output darlingtons or add another

current gain stage. Higher-Beta (and faster) power transistors can't handle

reactive loads worth a crap. Don't substitute high-fT parts unless you

are sure they have adequate second-breakdown capability.

The NE5532 op-amp can be used in the input stage. If more than one are used

off the +/-15 volt shunt regulators (balanced ins, anti-slew Bessel filters,

etc.) the 2.7K dropping resistors may need to be reduced to say, 1.8K ohm

to maintain regulation. The 2.7K resistors will allow up to 4 LF351 type

op amps off the regulator (I used a quad 347 for balanced inputs to

avoid hum in a DJ setup).

Construction tips:

The output transistors and thermal compensator (2SC1567) will need to be

mounted on a common heat sink - a finned unit measuring 5 in. high by

8 in. wide with 1.25 in fins should do nicely for one channel. (They look

nice if you make the sides of the case out of them). Most normal applications

won't require more cooling than this. The reason the 2SC1567 was chosen for

the output bias regulator is because it is fully insulated - the ECG

version will require additional mounting hardware. TO-3 hardware for the

outputs is cheap and easy to get.

The driver transistors and voltage amps (2SC3344/2SA1011 pairs) will all

require heatsinking as well. Individual TO-220 heat sinks on the circuit

board will suffice - the voltage amps dissipate about 1.4 watts each.

A common piece of 1/8 in. thick 1 in. wide X 4in. long angle aluminum

will suffice for all 4 on each channel, but bear in mind that it must be

oriented to take advantage of natural convection, and the transistors

must be insualted.

Keep the imput grounds separate from everything else, and return them at

ONE point. Failure to do so WILL result in high distortion (5% or so),

or even oscillation.

The output stage bias should be set to about 25  milliamps in the output

transistors. This value takes a while to stabilize, and you may have to

monitor it over an hour or so during initial setup. To measure it,

measure the voltage across the emitter resistor and use Ohm's law.

This way, you can check the current sharing in the parallel output

transistors at the same time and change them if there is a serious

discrepancy. With parts of the same date code, they should not be off by

more than 10% after it has warmed up. Higher output stage biases can be

used, but it takes more care in setting it. If you want an idle current

of more than 50 milliamps per side, increase the value of the emitter

resistors.

Initial Checkout:

DO NOT just plug something like this in! A seemingly insignificant error

can set your house on fire! (As well as blow out $30 worth of transistors

in a microsecond.) A variac will work in theory, but the amp may latch to

the rail if the supply drops too low. I suggest the use of a ballast

resistor - a 60 to 100 watt light bulb in series with the AC mains. You get

a bright flash when the caps charge, and then it goes (almost) out

as the idling supply current reaches its nominal low value. The amplifier

will then work normally at low volumes. If the amp draws too much current

for whatever reason, the lightbulb will glow brightly, increase resistance,

and limit the power to the circuit. Usually, there will either be a

mis-wire (use your DMM) or oscillation (will show up on a scope or

RF power measuring device). If the bulb goes dim-bright-dim-bright...

then the amp is marginally stable and the grounding layout should be checked.

Compensation capacitor values may need to be adjusted if any significant

changes were made. Mine is stable the way it is.

Additional Notes:

The schematic is in postcript, so it should just be able to be printed out.

The emitters of the transistors are labelled by an "e". I was too lazy to

put arrows on the transistor symbols - and I've been using it that

way for over a year now.

Trouble finding parts? MCM (1-800-543-4330) has all the transistors.

Total cost for a stereo version should be between $150 and $250,

depending on what kind of bargains you can find on the case, transformer,

and heatsinks. If you have to pay "list" for everything, it will likely

cost about $1000 to build.

.sig:

Think SPEAKER.......            /|            warren@eggo.csee.usf.edu

||/ |           gaiewski@sunflash.ec.usf.edu

"Great sound  starts with    ||  |                     or

The Source"             ||\ |            warren@renoir.cftnet.com

Powered by Pyle              \|             whichever don't bounce

DISCLAIMER:

The information included herin is provided as-is, with no warranties

express or implied. No resposibility on the part of the author is assumed for

the technical accuracy of the information given herein or the use or mis-use

of said information.

The equipment described in this article was designed, fabricated, and

tested on my own personal time using my own personal resources.