*********************************

LINE NOISE EXPLAINED

Thanks to Robert K. Ricketts

*********************************

Line Noise and the Problems it Can Cause with File Transfers

Many people have left messages on my bulletin board asking me why

there are so many 'garbage' characters on their screens and why file

transfers are riddled with errors.  These garbage characters are really

line noise and can be introduced in many different places.  One of the more

common and familiar introduction points of line noise is in the telephone

company's system and even here there are several ways noise is introduced.

A signal is routed through multiple stations before it eventually makes it

to the other end and some of these stations aren't exactly new.  Older

areas may have older, less sophisticated equipment that is more apt to be

affected by ambient noise. This is one reason some people continue to have

noise problems even after hanging up and calling back multiple times.

Also, a given physical connection at one of these junctions may not be up

to snuff.  If your particular bout of line noise is solved by hanging up

and calling back, then it's probable that you were previously connected

through an intermittent or 'dirty' connection. Some of these trunk lines

(large, multi-caller 'pipes') may pass through an area that has alot of

ambient RFI (Radio Frequency Interference) present although this is not

usually the case.

Another common noise introduction point is in your home.  Most

residential homes have televisions, radios, microwave ovens, VCR's, and if

you are reading this, a micro-computer.  All these devices radiate radio

waves that can (and often do) get into the phone lines and cause noise.

Electric motors and mechanical dimmer controls can introduce noise into the

electrical wiring in your house and cause problems.  If your line noise

problem does not go away after repeated hanging up and calling back, then

you may be suffering from one of these household problems.  If you are

suffering from this problem, you can take steps to eliminate it.  First of

all, turn off EVERYTHING except the fridge (If it IS the fridge, then

you're SOL.  Can't live life with your ice  box unplugged) and see if the

noise persists.  If it goes away, then start turning things back on,

checking the computer each time until you see the noise start up again.  It

may be that a single device is not bugging you but several devices plotting

together to annoy you.  This elimination tournament may take awhile.

Another area to check is your wiring at the computer.  Use noise

supressors on your power connections to both the PC and the modem (if

external).  Use a shielded RS-232 cable to connect your modem to the PC.

Ribbon cables (especailly long runs of it) are great antennas and will

cause problems.  Re-route the RS-232 cable so it does not run next to the

PC power supply or any other transformer.  Many 'clone' monitors do not

have internal metal shielding and can radiate lots of noise.  Make sure the

cable does not run near the monitor.  If you are particularly

adventuresome, you can line the interior of the monitor with foil and

ground it with a ribbon grounding strap. Be VERY CAREFUL if you attempt

this.  Monitors generate THOUSANDS of volts of electricity and can knock

you clear into next week.  You'd best NOT attempt this unless you are

experienced in electronics.  If you live near a freeway or highway, then

interference from CB radio can present a problem.  Many interstate truckers

have 100+ watts of power (illegally) on their CB rigs and frequently have

sloppy amplifiers that can emit spurious radiation all over the radio

spectrum.

And now a little discussion about the modem itself.  First of all, I'd

like to clerify a commonly misused term - BAUD.  The term "Baud" is

actually a man's name - J.M.E. Baudot (Pronounced:  Baw-doe) a French

Telegraphy expert. 1,200 and 2,400 Baud is NOT the same as 1,200 and 2,400

BPS (Bits Per Second). The usage of "Baud" to describe line speed in terms

of data through-put is incorrect.  1,200 and 2,400 BPS modems both operate

at 600 Baud.  Basically, without getting to technical, a Baud is a "blip"

of information.  1,200 BPS modems use four states per blip (or Baud) and

2,400 BPS modems use sixteen states per blip.  If you want more information

on what Baud and BPS mean and a full explanation of how data is actually

represented and transferred by the modem, please refer to PC Magazine

Volume 6, Number 9 (May 12, 1987).

Modems operating at 2,400 BPS are much more intolerant of line noise

than are modems operating at 1,200 BPS.  Conversely, modems capable of

2,400 BPS operate better at 1,200 BPS than do 1,200 BPS only modems.  If

you are being hopelessly attacked by noise at 2,400 BPS, trying calling

back at 1,200 BPS. It's very possible that the noise will be greatly

reduced or disappear altogether.  I know, you didn't buy a 2,400 BPS modem

just to retard it to 1,200 BPS.  The brand of the modem plays a part in the

immunity to line noise. Some modems can digest more noise (lower signal-to-

noise radio) than others. PC Magazine (same issue mentioned above) ran a

test on 87 different modems. You might check the results to see how your

modem ranks.  Most 2,400 BPS modems operating at 1,200 BPS have

approximately -8 to -10 db error threshold while the same modem has about -

16 to -20 db threshold operating at 2,400 BPS. For this reason, line

quality is much more critical at 2,400 BPS operation.

Additionally, a friend of mine who runs a bulletin board from their

office has been plagued with line noise problems at 2,400 BPS but very

little noise at 1,200 BPS.  The culprit is the office's centralized

telephone system. Many office buildings have a given number of trunks that

actually enter the building while there may be many, many more extension

within the building. These types of telephone systems have their own

controllers and line assignment devices and are frequently not as high in

quality as a hard-wired Ma Bell line.  The acceptable signal-to-noise ratio

in some of these inter-office phone controllers are lower than necessary

for reliable 2,400 BPS operation but not too low for 1,200 BPS.

If you gets transmission errors while downloading or uploading a file,

don't fret it.  The Xmodem (or whatever protocol) incorporates an error

checking/correction mechanism that automatically detects and corrects any

errors that may occur during transmission.  The very fact that Xmodem

reported the error in the first place means that he caught it and corrected

it.  The only errors you have to worry about are the ones that Xmodem does

NOT report. Any reported error has already be corrected.  Xmodem, especially

the CRC flavored one, is a very reliable file transfer protocol.  Even if

you got 100 errors during transmission, chances are still pretty slim that

the file got corrupted.  Occasionally, a file will be corrupted after

transfer, but many times this may be due to a bad ARCing of the file or

perhaps a disk error that may have occured sometime during the files' past.

I hope this text helped explain some facts about modems, line noise,

and file transfers.  If you have other, more specific questions, concerning

modems or communications in general, leave a C)omment to the SYSOP on the

PC Consultant.  I'll try to answer them.

the PC Consultant

SYSOP:  Robert K. Ricketts

P.O. BOX  42086

Houston, TX  77242-2086

Node 1  (713)270-7408  - Free and open to all.

Node 2  (713)270-8129  - Contributors only.

--end--

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Salted Slug Systems                      Strange              408-454-9368

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Tomorrow's 0rder of Magnitude            Finger_Man           415-961-9315

My Dog Bit Jesus                         Suzanne D'Fault      510-658-8078

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X