This is a second published pass at a IBM PC printer port FAQ; see the end

for some of the information I would like to have for future versions.

Let me know about any ftp archives or gopher servers or www nodes it goes

into, please.

==== Version 0.92  12/15/93   Zhahai Stewart   zstewart@nyx.cs.du.edu

Interfacing the IBM PC Parallel Printer Port

1. Overview

This document is called an FAQ because it answers many commonly asked

questions about the IBM parallel port, but it is formatted more as a brief

tutorial.  Read it twice before asking for more info; some stuff comes late.

Most of this I puzzled out for myself from various documention and

experiences.  The IBM documentation has some errors; I've had to cross

correlate various sources including schematics to get a consistent and

workable picture.  Since the first version, others have contributed

information, proofreading, and suggestions (see acknowledgements).

Starting with the original IBM PC, IBM defined a standard parallel printer

port which has become very widespread.  This port uses a female DB-25S

connector on the computer, and a special male DB-25P to Centronics male 36

pin "IBM Printer cable" is used to connect to standard Centronics parallel

printers.  This DB-25 connector is possible because about half of the

Centronics pins carry just electrical ground.

The original definition was embodied in the "IBM Printer Adaptor", and the

"IBM Monochrome Display and Printer Adaptor" cards.

2. Conventions used in this document.

An electrical "high" (on a pin or line) is TTL high, +2.4 to +5 volts.

An electrical "low" is TTL low, 0 to +0.8 volts.

A data high is a 1, a data low is a 0.

The connection between data (eg: a register bit) and pins is direct if a

data 1 is associated with an electical TTL high, and inverted if data 1

is associated with TTL low.  An overall connection (data to TTL to data)

is considered direct if outputing a 1 produces a 1 on input at the other

end, or inverted if outputting a 1 produces a 0 on the other end.

Pin labels, like "-Strobe" or "+Busy", are as defined by the printers and

by IBM.  The prefix "-" (or draw a line over the name) implies that the

signal is "active low"; that is, that when the signal is in its active state

when electically low.  The "+" prefix means "active high", just the opposite.

I have labeled the Data Out register bits as D0 to D7, with D0 being the

least significant bit and D7 the most signficant.  The Control Out bits are

labeled C0 to C3 (for the ones which go to pins) and C4 (for IRQ enable),

and maybe C5 (for bidirection ports only, controls direction).  The Status

In bits are labeled S3 to S7 (corresponding to data and CPU bit positions).

Often I will suffix C0 to C3 and S3 to S7 with a "+" or "-" as a reminder

of whether or not that register bit is inverted as compared to the output

or input pin it is associated with; "-" is for inverted, of course.  All

the Data Out bits are direct (not inverted); likewise data in for

bidirectional printer ports, but I have not bothered to suffix a "+".

Hexadecimal numbers are prededed by "0x", the C convention.

A "tristated" or disabled electical output basically disconnects the output

from the line or pin (high impedance), neither driving it high nor low.

3. Addresses, naming, BIOS and DOS

IBM defined three standard port base addresses (in 80x86 IO address space).

The Printer Adaptor could use base address 0x378, or later 0x278, while

the Monochrome Display and Printer Adaptor used base address 0x3BC.

The IBM BIOS defines RAM space for 4 parallel printer port base addresses,

stored as 4 16 bit words starting at main memory address 0x408.  During

bootup, the BIOS will check for parallel printer ports at base addresses

0x3BC, 0x378, and 0x278, in order, and store the base addresses of any

that are found in consecutive locations in this table.  Unused entries

may be 0, or some BIOSes fill them with the first port address found.

Some software may ignore this table, but it is used by at least the BIOS

itself (eg: INT 17, Printer I/O), and by DOS, as described below.

The BIOS detects these ports by writing 0xAA to the Data Out register (at

I/O address Base+0), reading the Data Feedback register (same address), and

deciding there is a port installed if it reads 0xAA.  This could be confused

if any lines are externally pulled up or down (or if the port defaulted to

tristate, or if there is another readback device register at that address).

The BIOS also counts the number of parallel ports it found and stores this

count in the upper two bits of the byte at 0x411 (yes, the table can hold

up to 4 entries, but the BIOS equipment flag printer count only goes to 3).

Warning: Just before this table there are 4 words at 0x400 which contain up

to 4 entries for base addresses for serial COM ports.  At least some serial

port software is known to store more than 4 entries, thus overlapping the

parallel port table.  Hopefully this is rare!

DOS (MSDOS and IBM DOS) maps these as LPTn devices.  Unlike COMn devices and

comm ports, the name mapping varies depending on whether or not there is a

Monochrome Display and Printer Adaptor card or not.  The first entry in

the BIOS table at 0x408 becomes LPT1, the second entry LPT2, and the third

entry LPT3 (if there are that many).  The DOS device "PRN" is really

software alias for another port, by default LPT1; use the MODE command to

change this aliasing.  The following table has "typical" assignments.

Note that by swapping the entries in the BIOS table at 0x408, you can change

which physical ports are assigned to LPT1, LPT2, etc.  Several "printer

swap" programs do just that.

Typical Assignments

Addr	MDPA   no MDPA

0x3BC	LPT1	n/a	Monochrome Display and Printer Adapter (MDPA)

0x378	LPT2	LPT1	Primary Printer Adapter

0x278	LPT3	LPT2	Secondary Printer Adapter

Name	MDPA	no MDPA

LPT1	0x3BC	 0x378

LPT2	0x378	 0x278

LPT3	0x278	  n/a

4. Registers addresses within the parallel printer port:

Port		R/W IOAddr	Bits	Function

----------	    ------	-----	----------------

Data Out	W   Base+0	D0-D7	8 LS TTL outputs

Status In	R   Base+1	S3-S7	5 LS TTL inputs

Control Out	W   Base+2	C0-C3	4 TTL Open Collector outputs

"		"     "         C4	internal, IRQ enable

"		"     "		C5	internal, Tristate data [PS/2]

Data Feedback	R   Base+0	D0-D7	matches Data Out

Control Feedbk	R   Base+2	C0-C3	matches Control Out

"		"     "		C4	internal, IRQ enable readback

The Feedback registers are for diagnostic purposes (except in bidirectional

ports, where Data Feedback is used for data input; and the IRQ enable C4).

5. Pin signals and register bits

<= in	DB25	Cent	Name of		Reg

=> out  pin	pin	Signal          Bit     Function Notes

------  ----    ----    --------        ---     -----------------------------

=>	 1	 1	-Strobe		C0-     Set Low pulse >0.5 us to send

=>	 2	 2	Data 0		D0      Set to least significant data

=>	 3	 3	Data 1		D1      ...

=>	 4	 4	Data 2		D2      ...

=>	 5	 5	Data 3		D3      ...

=>	 6	 6	Data 4		D4      ...

=>	 7	 7	Data 5		D5      ...

=>	 8	 8	Data 6		D6      ...

=>	 9	 9	Data 7		D7      Set to most significant data

<=	10	10	-Ack		S6+ IRQ	Low Pulse ~ 5 uS, after accept

<=	11	11	+Busy		S7-     High for Busy/Offline/Error

<=	12	12	+PaperEnd	S5+     High for out of paper

<=	13	13	+SelectIn	S4+     High for printer selected

=>	14	14	-AutoFd		C1-	Set Low to autofeed one line

<=	15	32	-Error		S3+	Low for Error/Offline/PaperEnd

=>	16	31	-Init		C2+	Set Low pulse > 50uS to init

=>	17	36	-Select		C3-	Set Low to select printer

==	18-25	19-30,	Ground

33,17,16

Note: Some cables, ports, or connectors may not connect all grounds.

Centronics pins 19-30 and 33 are "twisted pair return" grounds, while

17 is "chassis ground" and 16 is "logic ground".

"<= In" and "=> Out" are defined from the viewpoint of the PC, not the

printer.  The IRQ line (-Ack/S6+) is positive edge triggered, but only

enabled if C4 is 1.

Here's the same data grouped for ease of reference by Control Out and Status

In registers and pins.  (Data Out is straightforward in previous table).

<= in	DB25	Cent	Name of		Reg

=> out  pin	pin	Signal          Bit	Function Notes

------  ----    ----    --------        ---	------------------------------

=>	17	36	-Select		C3-	Set Low to select printer

=>	16	31	-Init		C2+	Set Low pulse > 50uS to init

=>	14	14	-AutoFd		C1-	Set Low to autofeed one line

=>	 1	 1	-Strobe		C0-	Set Low pulse > 0.5 us to send

<=	11	11	+Busy		S7-	High for Busy/Offline/Error

<=	10	10	-Ack		S6+ IRQ	Low Pulse ~ 5 uS, after accept

<=	12	12	+PaperEnd	S5+	High for out of paper

<=	13	13	+SelectIn	S4+	High for printer selected

<=	15	32	-Error		S3+	Low for Error/Offline/PaperEnd

6. Electrical

The Data Out pins were orginally driven by a 74LS374 octal latch, which could

source 2.6 mA and sink 24 mA.  There were 0.0022uF capacitors between each

line and ground to reduce transients.  The manual warns "It is essential that

the external device not try to pull these lines to ground", as this might

cause the 74LS374 to source more current than it could handle without damage.

The Feedback input for the Data Out register consisted of a 74LS244 latch; it

is uninverted.  Note that this port is only for diagnostics - if at any time

the feedback register for the data port does not match the data being output,

there is an error (eg: a line stuck high or low).  Exception: bidirectional

printer ports allow the 74LS374 (or equivalent) driver chip to be tristated,

which makes the data feedback port into a legitimate unlatched input port.

The Control Out pins were driven by 7405 inverting open collector buffers,

pulled to +5 volts via 4.7K resistors.  All data lines except C2 are inverted

before going to output pins; data line C2 is double inverted before going to

pin 16 (ie: is not inverted).

The Feedback input for the Control Out register also inverted all but D2,

which was passed through uninverted.  It is possible to use some or all of

the control out port bits for input, by programming the corresponding

control out to high (remembering the inversion on C0, C1, and C3), in which

case the open collector outputs are pulled high by the 4.7K resistors; any

externally applied high will retain the high state, while an externally

applied low will pull down the electical level to low.  This can be read

via the corresponding feedback bit(s).  If either the output from the control

register, or an externally applied signal, are low, then the input will be

low.  Remember the inversions between this electrical level and the bits, tho.

The Status In register is inverted for only pin 10, register bit S7.

ESD (from Steven M. Scharf <scharf@mirage.nsc.com>)

LSI implementations of the parallel port have often had serious ESD

(electrostatic discharge; includes static electricity) problems. National

and other high-end manufacturers have added extensive anti-ESD circuitry

to the parallel port signal lines; on cheap parallel port designs on some

other SuperIO clones ESD can easily destroy the parallel port circuit

when you turn the printer on when the computer is off or the printer is

on a switchbox.

7. Timing

The original IBM Printer Adapter which came out along with the PC (floppy

disks, pre XT) was built from TTL MSI parts.  They (and their clones) were

fast enough to work with the later AT bus, now called ISA, and still do.

I've yet to hear of a TTL printer adapter which is too slow for an ISA bus.

Partly this is because it was simple and fast in bus access (unlike the

serial port 8250 chip), and partly this is because even fast 486's slow

their ISA bus down to 8-11 MHz typically.

8. How to Print to a standard printer.

This is as defined for early IBM printers, and more or less compatible with

most others that use the Centronics 36 pin interface.

Wait for not +Busy (+Busy low)

Set Data Out bits

at least 0.5 uS delay

Pulse -Strobe low for at least 0.5 uS

hold Data Out for at least 0.5 uS after end of -Strobe pulse

Some time later, printer will pulse -Ack low for at least 5 uS

Printer may lower +Busy when it raises -Ack at end of pulse

Set other Control outputs or check Status inputs as desired.

9. BIOS Printer Support

The PC ROM BIOS detects and initializes the printer ports on bootup, and

provides printer support via INT 17 (and indirectly uses this in INT 5

"Print Screen").  The detection and building of a printer port base address

table is described in the Soft Addresses section above.

To control the printer via the BIOS, call INT 17 with the printer number in

DX (0 to 3, indexing into the four entry table at 0x408 mentioned in the

Soft Addresses section above), and the desired function in AH.  Functions:

AH = 0: print the character in AL

AH = 1: reinitialize the port, return status in AH

AH = 2: return status in AH

BIOS operation notes follow.

The BIOS normally keeps the Control register value as 0x0C:

C5 = 0  =>  output enabled (for bidirectional only)

C4 = 0  =>  IRQ disabled

C3 = 1  =>  -Select pin low    (Printer Selected)

C2 = 1  =>  -Init pin high     (Printer not being initialized)

C1 = 0  =>  -AutoFeed pin high (No Auto Feed)

C0 = 0  =>  -Strobe pin high   (No Stobe)

The BIOS prints a character (function AH = 0) as follows:

(Uses only  Status in S7/+Busy and Control out C0/-Strobe)

Write character to Data Out (D0-D7)

Wait for bit S7 to go to 1 (-Busy pin to go high/low?)

Timeout and return if this takes too long

Set C0 to 1 for a few microseconds (-Strobe pin pulsed low)

This assumes that the printer was already ready.

The BIOS handles reinitializing the printer (function AH = 1) thus:

Set C2 = 0 for a few hundred microseconds (-Init pin pulsed low)

The BIOS returns status in AH (functions AH = 1 or 2) is as follows:

__________________________________

| 7 | 6 | 5 | 4 | 3 | 2  | 1 | 0 |

|   |   |   |   |  not used  |___ 1 = "Time Out" (software)

|   |   |   |   |________________ 1 = "I/O Error" (inv C3+; -Error pin low)

|   |   |   |____________________ 1 = "Selected"  (C4+; +SelectIn pin high)

|   |   |________________________ 1 = "Paper Out" (C5+; +PaperEnd pin high)

|   |____________________________ 1 = "Acknowledge" (inv C6+; -Ack pin low)

|________________________________ 1 = "Not Busy"  (C7-; +Busy pin low)

The high 5 bits of this are essentially the Status In register from the

printer port, with bits 6 and 3 inverted (ie: XORed with 0x48).  The low

bit is generated by BIOS software after a timeout.  In the original PC,

timeout delays were generated by delay loops.

10. IRQ's

The primary IBM Printer Adapter (base 0x378) and the IBM Monochrome Display

and Printer Adapter (base 0x3BC) are allocated hardware interrupt 7 (IRQ 7),

which is wired to INT 0x0F.  If bit C4 of the Control Out register is high,

then input line "-ACK" (DB 25 pin 10, Status Register bit S6) will be wired

to the IRQ 7 bus line.  A rising edge (low to high) on -Acknowledge will be

cause a rising edge on IRQ 7, which will trigger an interrupt if that IRQ is

enabled in the 8259 interrupt controller chip.  (Note: the MCA bus is level

sensitive rather than edge triggered).  Both cards use the same interrupt,

and the ISA bus does not share interrupts correctly, so no more than one

of these ports should be enabled at the same time (or the 74LS125 chips

driving the PC bus will fight).

Secondary Printer Adapters (base 0x278) are supposed to use IRQ 5 (INT 0x0D)

for their interrupt; note that on the PC (pre-AT) this would have conflicted

with the floppy disk controller.  This IRQ is even more commonly "stolen"

for other usage: EGA, soundcards, network cards, etc.

The idea was apparently to use this IRQ as part of the printer driver, much

as the serial port IRQs can be used as part of comm drivers.  Unfortunately,

typical timing on printers did not allow this, so most (nearly all) DOS

software does not use the printer port IRQ for driving printers.  It's often

considered "up for grabs" and some soundcards like to use IRQ 7.  Some other

operating systems may use it for the printer port, as may some parallel port

transfer programs.

11. Bidirectional Ports

The IBM PS/2 series added one feature to the parallel ports: bidirectional

support.  This was done by using one more bit in the Control Out register

to control tristating of the Data Out port.  When bit C5 is 0, the Data

Out port operates as it did for earlier parallel ports (driver enabled);

when C5 is set to 1, though, the Data Out port is tristated, which means

that it is essentially electically disconnected from the pins (high

impedance state), not driving them high nor low.  This allows the data

feedback register to correctly read any externally applied TTL signals

on the corresponding data pins, effectively making this an input port.

Interestingly, the original IBM Printer Adapter, and many early clones,

had everything needed to become bidirectional.  The 74LS374 chip has a

chip enable pin which can tristate the 8 data outputs, but it is usually

wired to ground to always enable the output drivers.  The 74LS174

latch used for C0-C4 is actually a hex latch and processor data bus line

5 is attached to the sixth latch input; that bit IS latched when you

write to the Control Out register - but the output (Q6 on the 74LS174)

is not used by anything.  If you cut the trace from pin 1 (/OE) of the

74LS374 to ground, and connect this pin instead to pin 15 (Q6) of

the 74LS174, voila - you have a PS/2 compatible bidirectional parallel

port.  This only works on the older discrete TTL parallel ports, including

the IBM Printer Adapter and at least some clones of that era.  One theory

is that IBM had a bidirectional port in mind initially, but decided to

omit that aspect at the last moment (even while still latching the C5

bit from the processor into the LS174 hex latch).

There are now PS/2 compatible third party bidirectional parallel ports.

The cost for a TTL bidirectional port should have been identical, but now

that parallel ports are done with one LSI chip (or a fraction of one), this

will require an appropriate LSI chip.  Apparently some of them now have

bidirectional ports, some do not.

Note that some people have for many years been using the standard (not

bidirectional) data out port as an input port.  They output data 0xFF,

that is, all high.  Any pins which are externally pulled low via TTL or

switches will probably read as low (0) in the data feedback register,

because TTL low (sinking current) tends to be stronger than TTL high

(sourcing current), and tend to "win" when there two drivers fighting on

the same line.  An external high signal, or no signal, will allow the

pin (and data bit) to remain high (1).  This is NOT reccommended, as it

stresses the 74LS374 (or equivalent) beyond its safe margins and could

cause chip failure.  However, some people report doing this for years

without problem.  Note that all recent parallel ports are implemented

with single chip LSI controllers rather than the MSI TTL originals, which

means that they may have different drive capacities, electical margins,

and replacement costs, should you attempt this approach.  Sometimes the

LSI controller also includes other functions, such as serial ports or a

monochrome display port; if it overheats, more than just the parallel

port could be damaged.  Beware.  However, if you have an old TTL parallel

port (with a 74LS374 chip, preferably socketed), it may be easy and cheap

to replace if that chip should die.

I bought TTL parallel port card clones, fully socketed (all chips!), for

about $15 mail order about 5 years ago.  The bidirectional conversion was

a trace cut and a wire jumper.  The 74LS374 was easily replaced, if it

were damaged or if one wanted a better chip like the 74ACT374.  You could

even strap it for any base address (including non standard ones, so as not

to conflict).  I wish I could get more; there must have been scads of these

on the market.  You may find old TTL printer ports used; latch onto them

if you want to experiment with parallel port interfaces (pun acknowledged).

One last electrical note: if you use any sort of bidirectional parallel port

for inputs, be sure to tristate it whenever it is connected to external

devices which will drive it.  When I'm running it this way, I put a small

routine in my autoexec.bat which tristates the port in question on every

bootup.  Or use inline resistors to at least limit the current to a safe

value.

Steven M. Scharf <scharf@mirage.nsc.com> notes that some higher end parallel

port chips "lock" the direction control bit C5 to keep certain naive software

from accidentally changing the port direction.  In particular, he says that

"PTR bit 7" of the National Semiconductor SuperI/O chip must be 1 before you

can change the direction control bit.  I don't have documentation on this

chip, so I'm not sure what register PTR is; for now, consider this just a

warning.  He says that other chips may lock the direction control

differently.

12. Enhanced Ports

There are at least two specs for further enhanced parallel ports, the EPP

(Enhanced Parallel Port) and the ECP (Enhanced Capability Port), with the

latter being more ambitious.  I would appreciate any specs that can be

forwarded to me (or references) on either of these.

Steven M. Scharf <scharf@mirage.nsc.com> gave this table:

Different Parallel Ports on NSC Parts

--------------------------------------------------------

PC87311 Bidirectional Parallel Port (16450 UARTS)

PC87312 Bidirectional Parallel Port (16550 UARTS)

PC87322 Enhanced Parallel Port (EPP) (16550 UARTS)*

PC87332 Enhanced Capabilities Port (ECP) also EPP 1.7 and 1.9 (16550 UARTS)*

*Floppy controller signals can be redirected out the parallel port pins

via an internal multiplexor. This feature is used in some sub-notebooks

with external floppy drives

13. Copy Protection Dongles

(From info provided by Steven M. Scharf <scharf@mirage.nsc.com>)

Copy protection dongles typically watch the data lines for patterns of

data without any -strobe signal (pin 1, controlled by C0-).  That is, the

copy protection software writes various patterns to the data lines without

every pulsing the -strobe line.  Without a pulse on -strobe, any printer

also attached to the port should ignore this data.  Not all dongles work

on all ports; this strobeless method is a bit more temperamental than

regular printer output.

Steven M. Scharf: "I heard one dongle manufacturer complain to me about

a parallel port from Taiwan that tristated all the data lines between

every write to the data lines. This presented a pattern of FF to the dongle,

but had no effect on the printer since there was no strobe. In this case

the software was writing a sequence of bytes to the dongle and it didn't

work due to the FF in between each expected real byte."

"The dongle also draws power from the signal lines--a definite no-no.

Dongles should be designed to operate all the way down to the minimum TTL

low or at least to minimum Vout High of 2.4V. If your dongle doesn't work

but your printer works fine then it is almost certainly the fault of the

dongle--not the parallel port. Software with an incompatible dongle to the

parallel port on a machine will not be usable on that machine--one more

reason to not penalize the legitimate buyers of software."

14. Other devices:

From: geo@netcom.com (George Pontis)

"Another area that might be of interest in your document would be some

comment on the parallel port extenders. I have a xmit/rcv pair from LinkSys

that I bought from Fry's Electronics for about $70. They convert the parallel

signal to a serial data stream, using the signal and control lines for power.

My set was working fine until I added a hardware dongle for an expensive

Windows application. Then, printing ceased to work reliably. I took the

transmitter apart and partially traced the schematic. They have used 7

diodes to suck power from pins 13, 14, 15, 17, 1, 2, and 3.  Also, they

connected pins 15 (ERR) to 16 (INIT). The strobe line is coupled in to a

flip-flop, which starts clocking the parallel loaded data."

Comment from Zhahai - with both a dongle and a parallel port extender trying

to draw power from the port's data or control (not power) lines, it's not

surprising if things don't always work!  It must be really pushing the specs.

15. Transferring Data Via Ports

There are three basic ways to wire IBM printer ports together to transfer

data between them.  The most common is to wire D3-D7 from one port to S3 - S7

from the other port, and vice versa.  In this way, 4 bits of data can be put

out by one (D4 - D7) and read by the other (S4 - S7); the other bit (D3/S3)

can be used for synchronizing (eg: Data Ready).  If data is transferring one

way, the Dn/Sn path the other direction can be used for acknowledgements.  In

this scheme, you must remember that S3 - S6 read the inverse of the other

computer's D3 - D6 data bits, but S7 has the same value as D7 (also remember

that S6 and S7's mapping to pins is swapped in order from the others).

16. Transfer Modes and Cables

Mode 1A: nibble mode, using Data Out to Status In connection

This version works with all parallel ports; used in commercial xfer software.

Side 1	Pin     dir     Pin    Side 2   connection

------   ---     ---     ---    ------   ----------

D3	 5	 =>	15	S3+	direct

D4	 6	 =>	13	S4+	direct

D5	 7	 =>	12	S5+	direct

D6	 8	 =>	10	S6+	direct

D7	 9	 =>	11	S7-	inverted

S7-	11	<=	 9	D7	inverted

S6+	10	<=	 8	D6	direct

S5+	12	<=	 7	D5	direct

S4+	13	<=	 6	D4	direct

S3+	15	<=	 5	D3	direct

Mode 1B: nibble mode, using Data Out to Status In connection; Controls wired

for additional interfaces.  This version works with all parallel ports.

Side 1	Pin     dir     Pin    Side 2   connection

------   ---     ---     ---    ------   ----------

D3	 5	 =>	15	S3+	direct

D4	 6	 =>	13	S4+	direct

D5	 7	 =>	12	S5+	direct

D6	 8	 =>	10	S6+	direct

D7	 9	 =>	11	S7-	inverted

S7-	11	<=	 9	D7	inverted

S6+	10	<=	 8	D6	direct

S5+	12	<=	 7	D5	direct

S4+	13	<=	 6	D4	direct

S3+	15	<=	 5	D3	direct

C0-	 1	<=>*	 1	C0-	direct

C1-	14	<=>*	14	C1-	direct

C2+	16	<=>*	16	C2+	direct

C3-	17	<=>*	17	C3-	direct

* Note: Control Out bits on receiver set high (including inversion, ie:

C0,C1,C3=0; C2=1).  Control feedback on receiver can read control out from

sender.  Can use some lines each way, and could switch C0 - C2 and C1 - C3

for symmetry if we want two lines each way, or other variations.

Mode 2: 8 bits, using bidirectional parallel port

This version works only with bidirectional parallel port whose Data Out

can be tristated; the receiving side must tristate its Data Out port to

use its feedback register as an 8 bit input port.

Side 1	Pin     dir     Pin    Side 2   connection

------   ---     ---     ---    ------   ----------

D0	 2	<=>*	 2	D0	direct

D1	 3	<=>*	 3	D1	direct

D2	 4	<=>*	 4	D2	direct

D3	 5	<=>*	 5	D3	direct

D4	 6	<=>*	 6	D4	direct

D5	 7	<=>*	 7	D5	direct

D6	 8	<=>*	 8	D6	direct

D7	 9	<=>*	 9	D7	direct

C0-	 1	 =>	13	S4+	inverted

C1-	14	 =>	12	S5+	inverted

C2+	16	 =>	10	S6+	direct

C3-	17	 =>	11	S7-	direct

S4+	13	<=	 1	C0-	inverted

S5+	12	<=	14	C1-	inverted

S6+	10	<=	16	C2+	direct

S7-	11	<=	17	C3-	direct

* Note: bidirectional cards only; receiving side must tri-state with C5=1

If a two bidirectional ports are left connected in this fashion, and they

are both enabled (eg: after powerup or reset) with different data outputs,

then the 74LS374 driver chips could be "fighting".  Just to be careful, when

I created a cable like this (actually, a DB25 jumper box usually sold for

RS-232 jumpering, along with straight through 25 line DB-25 cables), I

used 8 10K resistors between the corresponding Data lines, to limit current

in this case.  (Actually, a DIP resistor pack fit perfectly on the PC board

inside the DB-25 jumper box).  The resistors are large enough to keep TTL

output from overstressing another one if both enabled, but when one is

disabled and the other enabled, the resistors are low enough to allow the TTL

output to drive a TTL input well enough.

Mode 3A: 8 bits, using Open Collector Control Outputs as inputs

This version uses 4 control outputs as inputs, plus 4 status inputs.

Side 1	Pin     dir     Pin    Side 2   connection

------   ---     ---     ---    ------   ----------

D0	 2	 =>*	 1	C0-	inverted

D1	 3	 =>*	14	C1-	inverted

D2	 4	 =>*	16	C2+	direct

D3	 5	 =>*	17	C3-	inverted

D4	 6	 =>	13	S4+	direct

D5	 7	 =>	12	S5+	direct

D6	 8	 =>	10	S6+	direct

D7	 9	 =>	11	S7-	inverted

C0-	 1	<=*	 2	D0	inverted

C1-	14	<=*	 3	D1	inverted

C2+	16	<=*	 4	D2	direct

C3-	17	<=*	 5	D3	inverted

S4+	13	<=	 6	D4	direct

S5+	12	<=	 7	D5	direct

S6+	10	<=	 8	D6	direct

S7-	11	<=	 9	D7	inverted

* Note: Control outputs used as inputs must be programmed high:

C0, C1, C3 = 0 and C2 = 1

Mode 3B: 8 bits, using Open Collector Control Outputs as inputs

This version uses 3 control outputs as inputs, plus 5 status inputs;

remaining control output is bidirectional - if left high by default,

either side can pull low (remember inverted logic).

Side 1	Pin     dir     Pin    Side 2   connection

------   ---     ---     ---    ------   ----------

D0	 2	 =>*	 1	C0-	inverted

D1	 3	 =>*	14	C1-	inverted

D2	 4	 =>*	16	C2+	direct

D3	 5	 =>*	15	S3+	direct

D4	 6	 =>	13	S4+	direct

D5	 7	 =>	12	S5+	direct

D6	 8	 =>	10	S6+	direct

D7	 9	 =>	11	S7-	inverted

C0-	 1	<=*	 2	D0	inverted

C1-	14	<=*	 3	D1	inverted

C2+	16	<=*	 4	D2	direct

S3+	15	<=*	 5	D3	direct

S4+	13	<=	 6	D4	direct

S5+	12	<=	 7	D5	direct

S6+	10	<=	 8	D6	direct

S7-	11	<=	 9	D7	inverted

C3-	17	<=>	17	C3-	direct (OC shared)

* Note: Control outputs used as inputs must be programmed high:

C0, C1, C3 = 0 and C2 = 1

[A future version of this document may sketch out the code to send and

receive data through these connections]

17. Capturing "printed" data from another machine

A computer with a bidirectional printer port, connected with a Mode 2

cable to any standard port, could potentially pretend to be a printer

so as to capture the "printed" information.  It would configure its

Data port for input, and set appropriate values on its Control Out to

mimic a printer on the other computer's Status In lines.  In the general

case, the problem would be detecting the very brief -Strobe pulse; this

would either require an external TTL latch triggered by -Strobe (either

edge), or some way to sense a quick pulse on that line.  In the latter

case, a revised connection (call it Mode 2B) could connect the "printing"

computer's -Strobe (C0-) line to the receiving computer's -Ack (S6+) line;

the trailing edge of the printing computer's -Stobe would generate an

interrupt on the receiving computer.  I have not tried this.  Of course,

the +Busy line would also be needed to avoid data overflow; perhaps it

could be kept high (busy) most of the time, but pulsed low after reading

the data (which would be handled by the IRQ routine).

[If anybody has had some success with this, let me know.]

18. Controlling Outputs

This can be easy; just use the Data Out TTL signals to control TTL level

items.  Unfortunately, they cannot source much current (providing positive

voltage on the pin, relative to ground) - be careful of the 2.6 mA limit.

Some LSI implementations might allow more or less than this (likely less).

Dn Out ------+

|+

Sourcing         Load (up to 2.6 mA @ 2.4 v)

|-

Ground ------+

If you have an external +5 volt supply, you have two options: use the Data

Out pins to sink up to 24 mA from your +5 volt supply, or use buffer chips

to control (source or sink) more current (or higher voltages).  I have

used an exteranl 5 Volt supply (regulated wall wart) plus optocoupled solid

state relays as the "load", to control AC voltages (keep the high AC voltages

away from any of this logic level stuff, obviously).

+------------------------------- (+5 v)

|+

Sinking         Load (up to 24 mA @ 4.2v)

|-				     Power Supply

Dn Out ------+

Ground -------------------------------------- ( Gnd)

Use limiting resistors if you need to limit the current.

If the load were an LED (or optocoupler) through which you wished to put

20 mA, do the calculations.  The Dn Output will probably be around 0.7

volts, so you have about 4.3 volts of drop; the LED will drop about 1.9 v

(check specs!), leaving 2.4 V to be dropped by a resister at 20 mA: 120

ohms.  Test and measure, adjust to fit.

You can also use the Control Out pins.  They can't source much of anything

(about 1 mA through the 4.7K resistors to +5), and can only sink about 7mA.

(The LS TTL gate actually sinks 8 mA, but one is taken up by the 4.7 K

resistor to +5).  Again, check on clones with different electical specs.

This can control TTL inputs fine, and might be able to run an optocoupler

or solid state relay in sink mode (depends on the device).

In one application, I used two 74ACT374 latches, which can source 48 mA

or sink 64 mA.  I connected the 8 inputs of each to the Data Out, and the

latch clocks to two Cn outputs.  In software, I put out 8 bits of data

on the Data Out port, pulsed a Cn bit to latch it into one 74ACT374,

put the next 8 bits out on Data Out, and used the other Cn bit to latch it

into the other 74ACT374 - voila, 16 bits of  64mA output control.  Of course,

this took a separate +5 V power supply ($5 surplus regulated wall wart).

If you can still find an old TTL parallel port (especially with sockets),

you can substitute the 74ACT374 chip for the original 74LS374 and get better

drive capability.  The back of magazine suppliers were selling *fully

socketed* TTL based parallel ports for about $15 a few years back; by cutting

a trace and soldering a jumper you could make them bidirectional; by plugging

in a $1 chip you could make them source/sink 48/64 mA.  You might still find

such TTL parallel port cards in old PCs.  Typically, one designed for the

original PC will still work fine on the ISA bus of your 486DX2-66, so don't

worry about that.

19. Sensing switches

There are several ways to sense external switches via a parallel printer

port.  If you are dealing with naked switches, the simplest is actually

to connect up to 4 switches between the control outputs (pins 1,14,16, and

17) and ground, program the control outputs high (counting inversions), and

use the control feedback register to read switch state (counting inversions).

This is because the pull-up resisters are already implemented for the open

collector Control Out pins.

Cn Out ------+		(Program output high; read feedback)

|

switch

|

Ground ------+

Another simple method would be to use up to 5 pullup resistors (4.7K) from

the Status inputs to +5 volts, and use switches to pull these down to ground.

One disadvantage of this is that it uses an external +5 supply.  TTL inputs

tend somewhat to float high, so you *might* get by without the pullup

resistors, but you are pushing it.

4.7K

Sn In -------+-^^^^^--- (+5 v)      (Read status register)

|

switch

|

Ground ------+---------- (gnd)

You could use this same scheme for 8 inputs on the data lines of a

bidirectional printer port - but you have to be sure that the outputs are

tristated beforehand, or your switches may damage the 74LS374 (or equiv).

This latter may be a problem after booting or rebooting.  One approach

is to use current limiting resistors in series "just in case".  You should

ideally still have pullup resistors too.  (The 1K value is chosen for LS TTL,

to keep the 2.4 V output at no more than 2.4 mA sourcing).

1K            4.7K

Dn Out --------^^^^^-----+----^^^^^------ (+5v)

|

Switch

|

Ground ------------------+--------------- (Gnd)

20. Optocoupled inputs

To be written.

21. Matrix scanning

In an upcoming project, I want to use a 4x4 keypad as an input.  In this

type of keypad (warning: there are other types), each key makes a connection

from one row to one column, when pressed.  My first design has the "columns"

connected to the 4 Control Out pins, while the "rows" are connected to 4 of

the Status Inputs.  Normally, all 4 control outputs were programmed high,

but to scan the keypad, I would lower one control output at a time, and

read the 4 status inputs at that time.  The 4 status inputs are each

pulled high to +5 volts via a 10K resistor, or left to float.  (Use Dn

plus resistors rather than +5 external supply?)  This scheme could be

expanded to as large as 4x5 (4 Cn outputs, 5 Sn inputs).

10 K

S4 ------------X X X X--^^^^^--+--- (+5)

S5 ------------X X X X--^^^^^--+

S6 ------------X X X X--^^^^^--+     (these pullups optional?)

S7 ------------X X X X--^^^^^--+

| | | |

C3 ------------+ | | |      Scan one Cn low at a time, read Sn each time

C2 --------------+ | |

C1 ----------------+ |

C0 ------------------+

Alternate scheme.  Use Dn's for scanning output rows, through diodes which

will isolate them from each other if two keys in the same column are pressed.

(Otherwise we have drivers fighting, and indeterminate voltage levels).

Germanium small signal diodes have less voltage drop.  Use the Cn pins

as inputs (with existing pullups) - program Control Out for high TTL levels,

and read the feedback register for input (counting inversions).  No extra

power supply is needed.  The diode between TTL output low and TTL input low

will push the TTL noise margins, but you can usually get by with it.  This

scheme could be expanded up to 8x4 (8 Dn outputs, 4 Cn used as inputs).  If

we know that only one key (or one key per column) will be pressed at a time,

the diodes can be omitted.

1N914

D0 ----|<|-----X X X X      (Set all Cn high),

D1 ----|<|-----X X X X	 Scan one Dn low at a time, read all Cn

D2 ----|<|-----X X X X

D3 ----|<|-----X X X X

| | | |

C3 ------------+ | | |

C2 --------------+ | |

C1 ----------------+ |

C0 ------------------+

Note that if we press three switches in the matrix which form 3 corners of

a box, it will appear as if all 4 corners are pressed; this is true of any

such matrix, unless each switch has its own diode.

I hope to have some results to share in a later revision of this doc.

22. Contributors and Information Needed for future revisions

My thanks for information, proofreading and suggestions from:

scharf@mirage.nsc.com (Steven M. Scharf)

cgd@ecmwf.co.uk (Dick Dixon)

rstevew@armory.com (Richard Steven Walz)

geo@netcom.com (George Pontis)

For the future:

I would like specs on the various enhanced printer ports and chipsets.

I would appreciate information on how parallel port connected Ethernet

interfaces, SCSI interfaces, tape backup units, sound cards, modems, and copy

protection devices work (which pins used how), including how "transparency"

is achieved (or not) on those with "passthrough".

I would appreciate info on data transfer programs, cables, and protocols.

I would like to hear about interrupt driven PC printer drivers, for DOS or

Unix or whatever (or why they don't work).

I would like to hear of "printer capture" software and hardware that uses

printer ports.

I would like to know about differences between "old standard" TTL parallel

ports and the LSI implementations (specific chip specs included).

I would like to hear of your interfacing projects, sucessful or not.

And of course, I'd like to hear about bugs and inaccuracies in this

document.  (Gently)

23. Differences

Version 0.92 corrected two minor typos which were kindly brought to my

attention (D2 in place of C2 in one place, and 7505 in place of 7405),

and reformatted some long lines.

==== End of version 0.92   12/15/93   Zhahai Stewart   zstewart@nyx.cs.du.edu

#