Sony Control Demo Program

Purpose:

This program is designed to fulfill two purposes:

First, it provides a finished program that can be used to remote

control a Sony Broadcast vtr from an IBM-PC or compatible comput-

er.

Second, for those interested in controlling a Sony machine in

some other application, it provides a sample of how this is done.

Hardware connections:

Serial control of the Sony machines is via RS-422 as specified by

SMPTE using a control protocol designed by Sony. The subset of

those commands used by SONYCTL are common to the entire family of

Sony broadcast vtr's.

In order to connect the Sony machine to the IBM computer, you can

either convert the standard IBM serial port to RS-422, or install

an IBM compatible RS-422 port in the computer.

Several companies produce RS-232 (the IBM standard) to RS-422

converters that can be used for this purpose.

If you use a converter, follow the instructions provided with the

converter when making up the interconnecting cables.

There are a number of companies that make RS-422/485 serial cards

that can be used with the program.

Of these, Antona Corp. makes an RS-422 compatible serial port

that is ideal for this application. This card provides two IBM

compatible serial ports. The two DB-9 output connectors are pin

compatible with Sony and Ampex machines. The card is supplied

with extensive documentation

Antona Corp.

1643 1/2 Westwood Blvd.

W. Los Angeles, CA 90024

(213) 473-8995

(213) 473-7112 FAX

ANC 1250 2 Channel Serial Base I/O Card

Install the card in the computer following the instructions pro-

vided with the card.

Connections between the ANTONA 1250 and the Sony vtr:

Antona                           VTR

DB 9 M                         DB 9 M

1  --------------------------  1   Gnd

2  --------------------------  2    T -

3  --------------------------  3    R +

7  --------------------------  7    T +

8  --------------------------  8    R -

Software:

The SONYCTL control program can be started in two ways:

If the vtr is connected to the serial port that is COM1: to DOS:

x>SONYCTL <enter>

Where x is the disc drive.

If the vtr's port is not the same as the DOS COM1: port, start

the program as follows:

x)SONYCTL z <enter>

Where z is the number 1 to 6, which converts to the IBM addresses

of com1: to com4:, and two special addresses.

1         com1      3F8h

2         com2      2F8h

3         com3      3E8h

4         com4      2E8h

5                   2C0h

6                   2C8h

When the program starts, it will initialize this port for 38.4kb,

even parity and 8 data bits (as defined by SMPTE). When the

program exits, it will reset the port to it's original settings.

Commands:

Figure 1 show the SONYCTL display.

The times at the top of the display show the in time, out time,

and duration (out time - in time), and the current time code

being read from the machine with it's time code type.

The 'Type' displays the type of vtr being controlled (BVU-800,

etc.).

The 'Status' display indicates the current status of the machine,

such as on or off line.

When the machine is in the search mode, the 'Search Speed' dis-

play indicates the direction the tape is moving, and it's approx-

imate speed.

At the bottom of the display are the command choices.

The current choice is indicated by the '*' to the left of the

display.

Pressing the 'Space Bar' issues stop to the vtr.

Pressing the 'P' key issues play to the vtr.

Pressing the 'F' key issues fast forward to the vtr.

Pressing the 'R' key issues fast rewind to the vtr.

Pressing the 'S' key places the machine in search.

When the vtr is in the 'Search' mode, use the left and right

arrow keys to control it's speed, and direction.

Pressing the 'Q' key issues cue up with data command to the

machine.

The vtr will cue to the time set in the 'IN' time display at

the top of the screen.

Pressing the 'D' key selects what type of time is requested from

the machine. The choices are time code, and control track.

Pressing the 'E' key ends the SONYCTL program and returns to DOS.

Pressing the 'I' key copies the current position to the IN regis-

ter and updates the duration display.

Pressing the 'O' key copies the current position to the OUT

register and updates the duration display

If there is no duration, or a negative duration, the display is

set to 0.

Figure 1:

Antona Corp.

Version X.XX

I = In         O = Out        Duration       Time Code

00:00:00:00    00:00:00:00    00:00:00:00    00:00:00:00 N

Type ********                 Status  Off-Line

<        Search Speed       >

| . . . . . . 0 . . . . . . |

*

*   = Stop

P = Play                           D = Display

F = Fast fwd                       E = Exit

R = Rewind                         I = In Time

S = Search                         O = Out Time

Q = Cue

For those wanting to do more, the source listing for SONYCTL has

been included.

The program is written in assembler, and can be assembled and

linked using Microsoft's MASM and LINK.

You can also use SLR Systems Optasm and Optlink (which were used

to develop the program).

SONYCTL links to the computers internal clock, and issues com-

mands, time code and status requests to the vtr 18.2 times a

second.

In more serious applications, it will probably be necessary to

use an interrupt locked to the an external sync generator.