Extracts from SUPER VGA ENHANCER USER'S MANUAL           1(2)

Standard VGA Modes

Mode  Type     Colors    Alpha   Buffer  Char   Screen

(hex)                    Format  Start   Size   Size

----------------------------------------------------------

0    text     16/256K   40x25   B8000   8x8    320x200

0*   text     16/256K   40x25   B8000   8x14   320x350

0+   text     16/256K   40x25   B8000   9x16   360x400

1    text     16/256K   40x25   B8000   8x8    320x200

1*   text     16/256K   40x25   B8000   8x14   320x350

1+   text     16/256K   40x25   B8000   9x16   360x400

2    text     16/256K   80x25   B8000   8x8    640x200

2*   text     16/256K   80x25   B8000   8x14   640x350

2+   text     16/256K   80x25   B8000   9x16   720x400

3    text     16/256K   80x25   B8000   8x8    640x200

3*   text     16/256K   80x25   B8000   8x14   640x350

3+   text     16/256K   80x25   B8000   9x16   720x400

4    graphic  4         40x25   B8000   8x8    320x200

5    graphic  4         40x25   B8000   8x8    320x200

6    graphic  2/256K    80x25   B8000   8x8    640x200

7    text     mono      80x25   B0000   9x14   720x350

7+   text     mono      80x25   B0000   9x16   720x400

D    graphic  16/256K   40x25   A0000   8x8    320x200

E    graphic  16/256K   80x25   A0000   8x8    640x200

F    graphic  mono      80x25   A0000   8x14   640x350

10    graphic  16/256K   80x25   A0000   8x14   640x350

11    graphic  2/256K    80x30   A0000   8x16   640x480

12    graphic  16/256K   80x30   A0000   8x16   640x480

13    graphic  256/256K  40x25   A0000   8x8    320x200

----------------------------------------------------------

* EGA text modes with 8x14 and 9x14 charachter sizes and 350

lines vertical resolution.

+ VGA text modes with 9x16 charchter size and 400 lines

vertical resolution.

NOTE : Default display modes are 3+ for color systems and 7+

for mono systems with analog monitors. Default display mode

is 3* for TTL monitors.

Extracts from SUPER VGA ENHANCER USER'S MANUAL           2(2)

Extended VGA Modes

Mode  Type     Colors    Alpha   Buffer  Char   Screen

(hex)                    Format  Start   Size   Size

----------------------------------------------------------

50    text     16/256K   80x30   B8000   8x16   640x480

51    text     16/256K   80x43   B8000   8x11   640x473

52    text     16/256K   80x60   B8000   8x8    640x480

53    text     16/256K   132x25  B8000   8x14   1056x350

54    text     16/256K   132x30  B8000   8x16   1056x480

55    text     16/256K   132x43  B8000   8x11   1056x473

56    text     16/256K   132x60  B8000   8x8    1056x480

57    text     16/256K   132x25  B8000   9x14   1188x350

58    text     16/256K   132x30  B8000   9x16   1188x480

59    text     16/256K   132x43  B8000   9x11   1188x473

5A    text     16/256K   132x60  B8000   9x8    1188x480

5B    graphic  16/256K   100x75  A0000   8x8    800x600

5C*   graphic  256/256K  80x25   A0000   8x16   640x400

5D*+  graphic  256/256K  80x30   A0000   8x16   640x480

5F+   graphic  16/256K   128x48  A0000   8x16   1024x768

----------------------------------------------------------

* Supports Set Mode but no scrolling function.

+ 5D and 5F need 512K DRAM. The SUPER VGA Graphics Adapter

needs to be set to the default switch setting in order to

use these modes (SW1 - SW6 ON).

How to use different graphic modes

To acess the modes available with your SUPER VGA Graphics

Adapter, use the following assembly routine :

MOV AX,xx      ;where xx is the mode number of the

;selected mode.

INT 10         ;Interrupt 10 initiates the video BIOS call.

INT 20         ;Interrupt 20 tells the program to exit back

;to DOS.

You can support the Print Screen command in the enhanced

modes (132 columns) by calling the "Alternate Select" BIOS

routine. If Interrupt 10 is called with AH=$12 and BL=$20

a generalized version of Print Screen will be initialized

that recognizes the extended dimensions of the screen.

Additional sources for serious VGA software :

*IBM Personal System/2 Display Adapter Technical Reference

April 1987 (IBM part number 68X2251 S68X-2251-0)

*IBM Personal System/2 and Personal Computer BIOS Interface

Technical Reference, April 1987

(IBM part number 68X2260 S68X-260-00)

Bjrn Ljungdahl 19891227