INT 10h,  10h (16)       Set Palette Registers                      PCjr,

Sets the palette and overscan (border) registers on the PCjr and EGA.

On entry:      AH         10h

AL         Subservice

0 - Set palette register

BL      Palette register

BH      Value to set

1 - Set overscan (border) color register

BH      Value to set

2 - Set palette registers and border register

ES:DX   Pointer to color list

3 - Toggle intensify/blinking bit

BL      0 - Enable intensify

BL      1 - Enable blinking

VGA Only:

7 - BL = Color register to get in BH

8 - Returns overscan value in BH

9 - Return list of palette registers & over-

scan value

ES:DX   Pointer to list

10h- Set color register

BX      Register to set

CH,CL,DL Green/Blue/Red

12h-

13h-

15h-

17h-

1Ah- Get color page info

BL      Mode

BH      Page

Registers destroyed:      AX, SP, BP, SI, DI

��������������������������������������������������������������������������

There are 16 palette registers on the EGA and PCjr displays and 1

overscan (border) register. This service gives the user control over

these registers. The four subservices are explained below.

Subservice  0  (Set Palette Register)

Each of the 16 palette registers can take on 1 of 64 values. the value

in BH is formatted as follows:

Bit 7 - Reserved

Bit 6 - Reserved

Bit 5 - Secondary Red

Bit 4 - Secondary Green

Bit 3 - Secondary Blue

Bit 2 - Red

Bit 1 - Green

Bit 0 - Blue

The secondary colors have an intensity of 1/3, while the normal colors

have an intensity of 2/3. For example, BH = 02h will produce green, BH

= 10h will produce a dim green, and BH = 12h will produce a bright

green.

The default values for the palette registers, which are the same as

the Color Graphics Adapter (CGA), are defined below:

��������������������������������������������������������������������������

Palette

Register        Color value         Color

00h               00h             Black

01h               01h             Blue

02h               02h             Green

03h               03h             Cyan

04h               04h             Red

05h               05h             Magenta

06h               14h             Brown

07h               07h             White

08h               38h             Dark Gray

09h               39h             Light Blue

0Ah               3Ah             Light Green

0Bh               3Bh             LIght Cyan

0Ch               3Ch             Light Red

0Dh               3Dh             Light Magenta

0Eh               3Eh             Yellow

0Fh               3Fh             Bright White

��������������������������������������������������������������������������

Subservice  1  (Set Overscan (Border) Register)

This subservice sets the border color. The Enhanced Color Display

supports overscan in 200-line modes only.

Subservice  2  (Set All Palette Registers and Overscan Register)

ES:DX points to a 17-byte color list, in which the first 16 bytes

consist of new values for the 16 palette registers and the last byte

is the new overscan register value.

Normally bit 7 of the attribute byte is used to indicate foreground

blinking. With this subservice, bit 7 can indicate foreground blinking

(BL=1) or background intensity (BL=0).

Subservice  3  (Toggle intensify/blinking bit)

VGA Only:

Subservice  7  (Get value of Color register)

Subservice  8  (Get Overscan value)

Subservice  9  (Load Color Registers into ...)

Subservice 10h (Set Color Register)

Subservice 12h (Change color registers ..)

Subservice 13h (Set page ....)

Subservice 15h (Read Color Register)

Subservice 17h (Load Color Registers into memory)

Subservice 1Ah (Get Color Page information)

INT 10h,  17h (23)       Set VGA RGB Registers

procedure SetVGApalette(var tp: PaletteType);

var regs: Registers;

begin { procedure SetVGApalette }

with regs do begin

AX:=$1012;

BX:=0; { first register to set }

CX:=256; { number of registers to set }

ES:=Seg(tp); DX:=Ofs(tp);

end;

Intr($10,regs);

end; { procedure SetVGApalette }

INT 10h,  17h (23)       Get VGA RGB Registers

procedure GetVGApalette(var tp: PaletteType);

var regs: Registers;

begin { procedure GetVGApalette }

with regs do begin

AX:=$1017;

BX:=0; { first register to get }

CX:=256; { number of registers to get }

ES:=Seg(tp); DX:=Ofs(tp);

end;

Intr($10,regs);

end; { procedure GetVGApalette }

Bj�rn Ljungdahl 1990