Special Programming Information

*******************************

This information is intended for advanced programmers writing applications or

drivers to make use of the extended capabilities of the 16 Bit VGA card. This

information is not needed for normal use of the 16 Bit VGA card. Use of the

information contained here requires familiarity with assembly language

programming and the workings of the IBM PC/AT environments.

Programmers requiring additional general information on programming VGA may

want to refer to the IBM PS/2 Technical Reference manuals as well as the

following book: Programmer's Guide to PC and PS/2 Video Systems,

by Richard Wilton, Microsoft Press, 1987. (ISBN 1-55615-103-9).

Programming Considerations

**************************

Refer to the Extended Video Mode table in the 16 Bit VGA Card Software Manual

for a description of the extended video modes available on your 16 Bit VGA.

132 Column Modes hex 54,55,56,57

********************************

If the multi-frequency monitor switch (sw 1) is turned on, the BIOS will use

the 8x8 font for the 43 row modes (54,56) and the 8x16 font for the 25 row

modes (55,57) and program the PVGA chip to use all 8 pixels. If on the other

hand, the multi-frequency monitor switch is off, the VGA board will be limited

to 7 pixels causing the 8x8 and 8x16 characters to run together. The VGA.EXE

utility compensates for this by loading a different font when the 132 column

modes are selected and the multi-frequency monitor switch is off. For those

programs that set the 132 column modes, the fonts used by the VGA.EXE utility

are provided on this disk:

FONT_25.I   ---- Mode 55 and 57.

FONT_43.I   ---- Mode 54 and 56.

The following program segment demonstrates how to determine if the fonts

should be used.

Read PR5 register (3CE index F):

mov     ax,007fh                ; Use extended setmode

mov     bh,1fh                  ; Look in the PR5 switches

int     10h

test    bl,80H                  ; Look for the switch 1 = on

jne     quit                    ; Switch is off do nothing

The switch was on. Pick the font needed based on the mode.

mov     ax,offset f5x7          ; load address of font for mode 54,56

mov     bh,9                    ; Load number of points

mov     ax,offset f7x16         ; load address of font for mode 55,57

mov     bh,16                   ; Set points

Setup for loading the font via BIOS call 11.

push    cs                      ; put cs in es

pop     es

push    ax                      ; Contains the offset

pop     bp

mov     al,00h                  ; Load user font

mov     ah,11h                  ; set bios call lcgen

mov     bl,00h                  ; Starting block

mov     cx,256                  ; number of characters

xor     dx,dx

int     10h

Extended 800x600 Graphics Modes hex 58,59

*****************************************

800 by 600 extended graphics modes require a multi-frequency monitor. No

checking is done in the BIOS or hardware to determine if such a monitor is

connected.  This mode is available independent of the setting of the

multi-frequency monitor special timing switch (switch lever 1).

Extended 256 Color Graphics Mode hex 5E

********************************************

The 640 by 400 extended graphics modes of the 16 Bit VGA card can be invoked

using the SetMode BIOS call.  The file 256COLOR.TXT on this disk describes in

detail how to access the additional memory required for this extended mode.

Extended BIOS Calls

Setmode Extensions

******************

The extended video modes of the 16 Bit VGA card can be set via

the standard SETMODE BIOS call or through the SET SPECIAL MODE BIOS call.

Additionally support is provided for setting and locking non-VGA modes

and reading and writing the Paradise Registers through the EXTENDED

FUNCTIONS Setmode BIOS call.

Setmode (BIOS call AH=00) has been extended as follows:

AH = 00h    SETMODE

AL = extended video mode

This call will set the extended 16 Bit VGA card video modes:

54h, 55h, 56h, 57h, 58h, 59h, 5Eh.

AX = 007E   SET SPECIAL MODE

This call sets modes by describing the mode desired.

BX = The horizontal dimension of the mode desired

(in pixels for graphics modes, columns for alpha modes)

CX = The vertical dimension of the mode desired

(in pixels for graphics modes, rows for alpha modes)

DX = The number of colors of the mode desired

(use 0 for monochrome modes)

On exit, the BH register will equal 7E if successful.

AX = 007F   EXTENDED FUNCTIONS

This call supports the following functions:

BH = 00  SET VGA OPERATION

BH = 01  SET NON-VGA OPERATION

Board must be set to a valid mode for non-VGA operation.

Color modes (0,1,2,3,4,5,6) will set non-VGA CGA operation.

Monochrome mode 7 will set non-VGA MDA/Hercules operation.

BH = 02  QUERY MODE STATUS

On exit the following is returned:

BL = 0 if operating in VGA mode.

BL = 1 if operating in non-VGA mode

CH = Total video RAM size in 64k byte units.

CL = Video RAM used by the current mode.

BH = 03  LOCKS CURRENT MODE

Allows current mode (VGA or non-VGA) to survive re-boot.

PR Register Extended Function Calls

BH = 09  WRITE PR0A REGISTER (3CE index 9)

BL = Value to set in PR0A.

BH = 0A  WRITE PR0B REGISTER (3CE index A)

BL = Value to set in PR0B.

BH = 0B  WRITE PR1 REGISTER (3CE index B)

BL = Value to set in PR1.

BH = 0C  WRITE PR2 REGISTER (3CE index C)

BL = Value to set in PR2.

BH = 0D  WRITE PR3 REGISTER (3CE index D)

BL = Value to set in PR3.

BH = 0E  WRITE PR4 REGISTER (3CE index E)

BL = Value to set in PR4.

BH = 0F  WRITE PR5 REGISTER (3CE index F)

BL = Value to set in PR5.

The high order 4 bits cannot be changed (SWITCHES).

BH = 19  READ PR0A REGISTER (3CE index 9)

On exit the following is returned:

BL = Value of PR0A.

BH = 1A  READ PR0B REGISTER (3CE index A)

On exit the following is returned:

BL = Value of PR0B.

BH = 1B  READ PR1 REGISTER (3CE index B)

On exit the following is returned:

BL = Value of PR1.

BH = 1C  READ PR2 REGISTER (3CE index C)

On exit the following is returned:

BL = Value of PR2.

BH = 1D  READ PR3 REGISTER (3CE index D)

On exit the following is returned:

BL = Value of the PR3.

BH = 1E  READ PR4 REGISTER (3CE index E)

On exit the following is returned:

BL = Value of the PR4.

BH = 1F  READ PR5 REGISTER (3CE index F)

On exit the following is returned:

BL = Value of PR5.

On exit, the BH register will equal 7F if successful.

Note: The state of PR Registers cannot be assumed between standard

Setmode calls, and should not be assumed in general.

*******************************************************************************

* This information is intended for advanced programmers writing applications or

* drivers to make use of the extended capabilities of the 16 Bit VGA card. This

* information is not needed for normal use of the 16 Bit VGA card. Use of the

* information contained here requires familiarity with assembly language

* programming and the workings of the IBM PC/AT environments.

*******************************************************************************

Extended Memory Mapping

The memory map for IBM PCs and compatibles allots 128 Kbytes of the

one Megabyte system total to video RAM. This area begins at A0000

and extends to BFFFF, but "standard" video modes only use:

ADDRESS   SIZE      USED IN MODES

A0000     64K      D,E,F,10,11,12,13

B0000     32K      7

B0000     64K      Hercules

B8000     32K      0,1,2,3,4,5,6

If two video cards coexist, they must be set to modes using different

VRAM areas. All of the modes listed above are designed to use no more

than 64K of the video memory map - even modes D-12 conform to this

limit by organizing their extra memory into "planes".

Mode 13 is in a class by itself, it can provide 256 colors with a

resolution of 320x200 pels. Each pel requires one byte of VRAM, so

it takes 64,000 bytes to store a display screen. Using 128K at

A0000, it would be possible to support 640x200 or 320x400 resolution;

but these modes would not be able to coexist with secondary video

cards.

The 16 Bit VGA is designed to provide 256 color modes with resolutions

as high as 640x400 pels. Although this requires 250K of VRAM, a kind

of bankswitching allows this mode to operate without violating the

64K limit at any time.

Before giving examples of how to read and write extended VRAM, the

necessary memory and I/O conventions will be described. In a 256 color

mode the memory assigns one byte for each pel - in addition, the byte's

offset from the beginning of the VRAM is a function of the X-Y coordinates

of the pel's position on the display. By convention, the leftmost

pel of the top scan line is assigned coordinates (0,0) - the X coordinates

increase moving to the right and Y coordinates increase moving downward.

The rightmost pel in the last scan line of a 640x400 display has the

position (639,399). Letting WIDTH represent the number of pels in

one horizontal scan line (usually 320 or 640), a given pel's relative

location in memory is:

OFFSET = (WIDTH*Y)+X

The problem of extended VRAM control is the conversion of this OFFSET

to an address that can be accessed by an 8088 MOV instruction. In

mode 13, this is simple - the true address is equal to A0000+OFFSET.

There is no problem because OFFSET is always less than 64K (since

WIDTH=320, X<320 and Y<200). In extended modes, the video memory must

be thought of in two distinct ways - the 16 Bit VGA's point of view (VGA) and

the SYSTEM's. The 16 Bit VGA addressing can be thought of as the correspondence

between display coordinates and memory location given in the formula

for OFFSET. It represents the "true" pel address. The SYSTEM addressing

must be set up by the software prior to the memory access itself.

This setup is carried out by writing to certain I/O registers in the

VGA. Note that this is not an unusual situation, a similar preparation

is required to work with standard "planed" memory.

Before describing the SYSTEM memory I/O registers in detail, a model

of the relationship between VGA and SYSTEM memory treatment will

be presented. All memory addresses, unless stated otherwise, are hexadecimal.

The VGA can control up to one megabyte of video RAM, so the value

calculated by OFFSET requires twenty bits or five hex digits. It is

useful to split the five digit address into two parts - the two high

order digits (HI_OFFSET) and the three low order digits (LO_OFFSET).

For example, OFFSET=1E7A4 has 1E as a HI_OFFSET and 7A4 as its LO_OFFSET.

Because SYSTEM video memory addresses begin at A0000 and cannot exceed

AFFFF, a full five digit OFFSET will go out of range if A0000 is added

(like for mode 13). To allow the necessary adjustment, the VGA provides

two offset registers - PR0A and PR0B. These registers can be used

to map regions of the VRAM into the SYSTEM memory area. There are

two options:

1. Use PR0A alone to access a 64K VRAM window. This window

begins at A0000 in SYSTEM memory.

2. Use PR0A and PR0B to access two 32K VRAM windows. The

PR0A window is mapped to A0000 and PR0B to A8000. Even

though the windows are separate in SYSTEM memory, they

can overlap in VGA memory if the values written to PR0A

and PR0B specify it.

PR0A and PR0B both take one byte values. Assuming ADDRESS is a twenty

bit number in the video range A0000-AFFFF, the correspondence between

VGA and SYSTEM addressing is given as follows:

SYSTEM to VGA is straightforward -

OFFSET = (ADDRESS-A0000)+VALUE(PR0A)*1000H

and OFFSET = (ADDRESS-A0000)+VALUE(PR0B)*1000H

VGA to SYSTEM is more ambiguous because the same OFFSET

can be derived from different combinations of ADDRESS and

VALUE(PR0A). The recommended settings are -

ADDRESS     = A0000+LO_OFFSET

VALUE(PR0A) = HI_OFFSET

and ADDRESS     = A8000+LO_OFFSET

VALUE(PR0B) = HI_OFFSET-8

Adherance to this recommendation maximizes the size of the VGA memory

blocks that can be modified without worrying about address

wraparound in SYSTEM memory:

e.g. take OFFSET=2FFFF

Case 1: set DS:SI=A000:FFF and VALUE(PR0A)=2F

Case 2: set DS:SI=A000:FFFF and VALUE(PR0A)=20

Both cases yield the correct OFFSET, but if SI is incremented, the

case 1 OFFSET is 30000 and the case 2 OFFSET has wrapped back to 20000.

Because LO_OFFSET has a maximum value of FFF (=4K-1), it is safe to

operate with 60K (=64-4) blocks if PR0A alone is used, and 28K (=32-4)

if PR0A and PR0B are both activated.

A close check of the VGA-SYSTEM conversion formulas should reveal

that the PR0B variants aren't quite as symmetric as those for PR0A.

In particular, to generate a SYSTEM address that accesses VGA OFFSETs

in the range 00000-07FFF, it is necessary to set PR0B to a value in

the vicinity of

(Total 4K units of VRAM used)-8

For example, 640x400 mode requires 256K of video memory which is equivalent

to a maximum OFFSET of 40000. This value has a HI_OFFSET equal to

40 which also represents the "Total 4K units of VRAM used". So, by

setting PR0B to 38 (=40-8) and accessing SYSTEM address A8000, software

can modify OFFSET 00000. Although this technique allows PR0B to access

any VRAM that PR0A can, the implementation might be clumsy since the

"Total 4K units" varies according to the mode's resolution. An alternative

is to check the value of HI_OFFSET-8 prior to setting PR0A or PR0B

and making sure that PR0B receives a valid setting. In other words,

if the source and destination OFFSETs are both less than 08000, use

PR0A only; otherwise set up PR0A to handle the lower OFFSET and PR0B

to access the higher OFFSET.

For anyone familiar with the 8088 architecture, PR0A and PR0B can

be thought of as being analogous to the segment registers DS and ES.

Both are used to modify a sixteen bit address (e.g. [SI]) to a full

twenty bit address required by the hardware. The principal difference

is the granularity - incrementing a segment register by one advances

the true address by one paragraph (=16 bytes), and incrementing PR0A

or PR0B jumps the pointer into PVGA memory by 4K (=4096 bytes).

The following diagram illustrates the VGA-SYSTEM addressing relationship

in PR0A-only and PR0A-PR0B operation. The numbers represent the beginning

addresses of 4K memory blocks, so 02000 should be read as "the memory

from 2000 to 2FFF inclusive":

Example 1                          Example 2

PR0A=12                            PR0A=00

PR0B inactive                      PR0B=0E=16-8

VGA      SYSTEM WINDOW            VGA      SYSTEM WINDOWS

00000                              00000     A0000 (PR0A)

01000                              01000     A1000 (PR0A)

02000                              02000     A2000 (PR0A)

03000                              03000     A3000 (PR0A)

04000                              04000     A4000 (PR0A)

05000                              05000     A5000 (PR0A)

06000                              06000     A6000 (PR0A)

07000                              07000     A7000 (PR0A)

08000                              08000

09000                              09000

0A000                              0A000

0B000                              0B000

0C000                              0C000

0D000                              0D000

0E000                              0E000

0F000                              0F000

10000                              10000

11000                              11000

12000     A0000 (PR0A)             12000

13000     A1000 (PR0A)             13000

14000     A2000 (PR0A)             14000

15000     A3000 (PR0A)             15000

16000     A4000 (PR0A)             16000     A8000 (PR0B)

17000     A5000 (PR0A)             17000     A9000 (PR0B)

18000     A6000 (PR0A)             18000     AA000 (PR0B)

19000     A7000 (PR0A)             19000     AB000 (PR0B)

1A000     A8000 (PR0A)             1A000     AC000 (PR0B)

1B000     A9000 (PR0A)             1B000     AD000 (PR0B)

1C000     AA000 (PR0A)             1C000     AE000 (PR0B)

1D000     AB000 (PR0A)             1D000     AF000 (PR0B)

1E000     AC000 (PR0A)             1E000

1F000     AD000 (PR0A)             1F000

20000     AE000 (PR0A)             20000

21000     AF000 (PR0A)             21000

22000                              22000

23000                              23000

.                                  .

.                                  .

.                                  .

Now that the operation of PR0A and PR0B has been made abundantly clear,

their enabling and setting procedures will be outlined. Control is

handled by a variety of I/O registers provided by the VGA: PR0A,

PR0B, PR1, PR2, PR3, PR4 and PR5. These registers are described in

the PVGA User's Guide; of them, PR0A, PR0B, PR1 and PR5 are associated

with extended memory control. (So is PR4, but it is set automatically

whenever an extended memory mode has been activated and should be

left alone thereafter.)

These registers are implemented as additions to the Graphics Controller

portion of the VGA. Software communicates with the Graphics Controller

through two eight-bit I/O ports - the Index Port at 3CE and the Data

Port at 3CF. Both ports can be written to with a standard OUT instruction,

an IN instruction returns the last value sent to the port. The Index

Port is used to select a specific Graphics Controller register, the

following indices are assigned within the VGA:

Index          Name

0-8           Standard VGA Graphics Controller Registers

9            PR0A

0A            PR0B

0B            PR1

0C            PR2

0D            PR3

0E            PR4

0F            PR5

Since PR0A through PR5 are not standard VGA registers, the VGA chip

offers optional protection from accidental modification. The PR5 register

handles this protection in the following manner:

Example - writing the value 2E to PR0B

MOV  DX,3CE         ; GRAPHICS CONTROLLER INDEX PORT ADDRESS

MOV  AL,0F          ; PR5 INDEX VALUE

MOV  AH,5           ; THE VALUE 5 "UNLOCKS" PR0A TO PR4

OUT  DX,AX          ; UNLOCK

MOV  AL,0A          ; PR0B INDEX VALUE

MOV  AH,2E          ; DESIRED SETTING

OUT  DX,AX          ; SET PR0B

MOV  AL,0F          ; PR5 INDEX VALUE

MOV  AH,0           ; VALUES OTHER THAN 5 "LOCK" PR0A TO PR4

OUT  DX,AX          ; RELOCK

It is not necessary to relock after setting a register, but this is

a safe convention. PR5 values are recognized modulo 8 (e.g. A5 is

equivalent to 05).

PR0A is modified in the same way as PR0B. It is probably desirable

to zero these registers upon completion of an extended memory access

- this forces consistency between diferent applications. The following

example shows how to enable PR0B by setting the 8 bit (bit 3) of PR1:

Example - enabling and disabling PR0B

MOV  DX,3CE         ; GRAPHICS CONTROLLER INDEX PORT ADDRESS

MOV  AL,0F          ; PR5 INDEX VALUE

MOV  AH,5           ; THE VALUE 5 "UNLOCKS" PR0A TO PR4

OUT  DX,AX          ; UNLOCK

MOV  AL,0B          ; PR1 INDEX VALUE

OUT  DX,AL          ; SET INDEX ONLY

INC  DX             ; GRAPHICS CONTROLLER DATA PORT ADDRESS

IN   AL,DX          ; READ CURRENT CONTENTS

OR   AL,8           ; SET BIT TO ENABLE PR0B

OUT  DX,AL          ; ENABLE PR0B

---                 ; INSERT CODE TO MODIFY MEMORY,

; LEAVING REGISTERS AND PORTS UNCHANGED

AND  AL,0F7         ; SET BIT TO DISABLE PR0B

OUT  DX,AL          ; DISABLE PR0B

DEC  DX             ; GRAPHICS CONTROLLER INDEX PORT ADDRESS

MOV  AL,0F          ; PR5 INDEX VALUE

MOV  AH,0           ; VALUES OTHER THAN 5 "LOCK" PR0A TO PR4

OUT  DX,AX          ; RELOCK

It is also possible to read and write PR0A and PR0B using the extended

BIOS calls described in the document PROGRAM.TXT.  When using these BIOS

calls it is not necessary to unlock and lock the extended registers since

this is performed automatically by BIOS routines.

Example - writing the value 2E to PROB using BIOS

MOV  AX,007F	; Extended function BIOS call

MOV  BH,0A		; Write to PR0B

MOV  BL,2E		; Value to write to PR0B

INT  10		; Video BIOS software interrupt

Example - enabling and disabling PROB using BIOS

MOV  AX,007F	; Extended function BIOS call

MOV  BH,1B		; Read contents of PR1

INT  10		; Video BIOS software interrupt

OR   BL,8		; Set bit of PR1 to enable PR0B

MOV  BH,0B		; Write value to PR1 using BIOS call

INT  10		; Video BIOS software interrupt

___		; Insert code to modify memory

; Leaving registers and ports unchanged

AND  BL,0F7	; Set bit of PR1 to disable PR0B

MOV  BH,0B		; Write value to PR1 using BIOS call

INT 10		; Video BIOS software interrupt

In summary, control through the PR0A and PR0B registers allows software

to access any part of extended video memory without violating the

PC and VGA memory map standards. There are two access options - PR0A

is most useful for simple memory fills and for moving VRAM blocks

that lie within 64K of each other. But if the lowest OFFSET of one

block is more than 64K bytes from the highest OFFSET of the other,

it is necessary to use both PR0A and PR0B. To avoid address wrapping

or conflict with other software, the following conventions are recommended:

1. Upon completion of a task, make sure that:

a. PR0A and PR0B are zeroed

b. PR0B is disabled

c. PR0A through PR4 are locked by setting PR5

2. When converting an OFFSET to its equivalent SYSTEM address,

use the PR0A and PR0B registers to their greatest advantage

by splitting OFFSET into HI_OFFSET and LO_OFFSET as described

earlier.

3. Use 32K as a maximum memory block size for PR0A operations,

and use 16K for PR0A-PR0B transfers. In conjunction with

convention 2, this avoids any address wrap problems. Also,

as multiples of 4K, these block sizes are convenient to deal

with in loops.

Sample program 1

Task: zero 256K of video memory starting at OFFSET 00000

Assuming: registers unlocked, PR0A enabled, PR0B disabled

MOV  AX,0A000

MOV  ES,AX          ; SET VIDEO RAM SEGMENT

MOV  DX,3CE         ; GRAPHICS CONTROLLER INDEX PORT

MOV  BL,0           ; INITIAL VALUE FOR HI_OFFSET

LOOP1:                        ; OUTER LOOP FILLS 32K BLOCKS

MOV  AL,9           ; INDEX FOR PR0A

MOV  AH,BL          ; HI_OFFSET FOR CURRENT 32K

OUT  DX,AX          ; SET IT

CLD

MOV  DI,0           ; SET LO_OFFSET

MOV  AL,0           ; DATA VALUE

MOV  CX,8000H       ; 32K BYTES

REP  STOSB          ; FILL 32K WITH ZERO

ADD  BL,8           ; ADVANCE HI_OFFSET BY 32K

CMP  BL,40          ; TEST IF HI_OFFSET OF 256K

JNZ  LOOP1

Sample program 2

Task: Move the contents of a source VRAM block to a

destination VRAM block.

Assuming: registers unlocked

PR0A enabled

PR0B enabled

pointer and size variables are in code segment

08000 = destination OFFSET

source, destination blocks disjoint

SIZE+either OFFSET = highest possible OFFSET

DB   SOURCE_HI      ; HI_OFFSET FOR SOURCE BLOCK

DW   SOURCE_LO      ; LO_OFFSET FOR SOURCE BLOCK

DB   DESTINATION_HI ; HI_OFFSET FOR DESTINATION

DW   DESTINATION_LO ; LO_OFFSET FOR DESTINATION

DD   SIZE           ; BLOCK SIZE IN BYTES

MOV  AX,0A000

MOV  DS,AX          ; SET SOURCE VRAM SEGMENT

MOV  BL,CS:BYTE PTR SOURCE_HI

MOV  AX,0A800

MOV  ES,AX          ; SET DESTINATION VRAM SEGMENT

MOV  BH,CS:BYTE PTR DESTINATION_HI

SUB  BH,8           ; PR0B ADJUSTMENT OF HI_OFFSET

MOV  DX,3CE         ; GRAPHICS CONTROLLER INDEX PORT

LOOP1:                        ; OUTER LOOP MOVES 16K BLOCKS

MOV  CX,4000        ; DEFAULT BLOCK SIZE OF 16K

CMP  CS:WORD PTR SIZE+2,0

JNZ  MORE           ; SIZE64K, CONTINUE

CMP  CX,CS:WORD PTR SIZE

JBE  MORE           ; SIZE=16K, CONTINUE

MOV  CX,CS:WORD PTR SIZE      ; SET COUNT=MIN(16K,SIZE)

JCXZ EXIT           ; SIZE=0, DONE

MORE:     MOV  AL,9           ; INDEX FOR PR0A

MOV  AH,BL          ; HI_OFFSET FOR SOURCE

OUT  DX,AX          ; SET IT

MOV  AL,0A          ; INDEX FOR PR0B

MOV  AH,BH          ; HI_OFFSET FOR DESTINATION

OUT  DX,AX          ; SET IT

MOV  SI,CS:WORD PTR SOURCE_LO ; SET LO_OFFSETS

MOV  DI,CS:WORD PTR DESTINATION_LO

PUSH CX             ; SAVE SUB-BLOCK SIZE

CLD

REP  MOVSB          ; TRANSFER BLOCK PORTION (16K MAX)

ADD  BL,4           ; ADVANCE SOURCE BY 16K

ADD  BH,4           ; ADVANCE DESTINATION BY 16K

POP  CX             ; RECOVER SUB-BLOCK SIZE

SUB  CS:WORD PTR SIZE,CX      ; SUBTRACT PART MOVED

SBB  CS:WORD PTR SIZE+2,0     ; BORROW FROM HI ORDER

JMP  SHORT LOOP1

EXIT:     ---