Video BIOS Data Areas

Video Display Data Area

Address    Name         Type  Description

0040:0049  CRT_MODE     Byte  Current BIOS video mode number

0040:004A  CRT_COLS     Word  Number of displayed character columns

0040:004C  CRT_LEN      Word  Size of video buffer in bytes

0040:004E  CRT_START    Word  Offset of start of video buffer

0040:0050  CURSOR_POSN  Word  Array of eight words containing the cursor

position for each of eight possible video

pages. The high-order byte of each word

contains the character row, the low-order

byte the character column.

0040:0060  CURSOR_MODE  Word  Starting and ending lines for alphanumeric

cursor. The high-order byte contains the

starting (top) line; the low-order byte

contains the ending (bottom) line.

0040:0062  ACTIVE_PAGE  Byte  Currently displayed video page number

0040:0063  ADDR_6845    Word  I/O port address ofCRT Controller's Address

register (3B4h for monochrome, 3D4h for

color).

0040:0065  CRT_MODE_SET Byte  Current value for Mode Control register (3B8h

on MDA, 3D8h on CGA). On the EGA and VGA, the

values emulate those used on the MDA and CGA.

0040:0066  CRT_PALETTE  Byte  Current value for the CGA Color Select

register (3D9h). On the EGA and VGA, the

value emulates those used on the MDA and CGA.

0040:0084  ROWS         Byte  Number of displayed character rows-1

0040:0085  POINTS       Word  Height of character matrix

0040:0087  INFO         Byte  EGA and VGA Video Display Data Area

0040:0088  INFO_3       Byte  EGA and VGA Video Display Data Area

0040:0089  Flags        Byte  Miscellaneous flags

0040:008A  DCC          Byte  Display combination code table index

0040:00A8  SAVE_PTR     Dword Pointer to BIOS Save Area

Mapping 0040:0087 INFO

Bit 7    Reflect bit 7 of video mode number passed to INT10h function 00h

6-5  Amount of video RAM

00b - 64K

01b - 128K

10b - 192K

11b - 256K

4    Reserved

3    1 - video subsystem is inactive

2    Reserved

1    1 - video subsystem is attached to monochrome display

0    1 - alphanumeric cursor emulation is enabled

Mapping 0040:0088 INFO_3

Bits 4 through 7 reflect the power on status of the EGA feature connector.

Bit 0 through 3 reflect the setting of the four EGA configuration switches

(the switch values are emulated by the VGA BIOS, depending on the type of

display attached).

Bit 7 Input from feature connector FEAT1 (bit 6 of input status register 0)

in response to output on FC1 (bit 1 of Feature Control register)

6 Input from feature connector FEAT0 (bit 5 of input status register 0)

in response to output on FC1 (bit 1 of Feature Control register)

5 Input from feature connector FEAT1 (bit 6 of input status register 0)

in response to output on FC0 (bit 0 of Feature Control register)

4 Input from feature connector FEAT0 (bit 5 of input status register 0)

in response to output on FC0 (bit 0 of Feature Control register)

3 Configuration switch 4 (1 - off, 0 - on)

2 Configuration switch 3 (1 - off, 0 - on)

1 Configuration switch 2 (1 - off, 0 - on)

0 Configuration switch 1 (1 - off, 0 - on)

Mapping 0040:0089 Flags

Bit 7 Alphanumeric scan lines (with bit 4)

bit 7  bit 4

0      0      350-line mode

0      1      400-line mode

1      0      200-line mode

1      1      Reserved

6 1 - display switching is disabled

0 - display switching is enabled

5 Reserved

4 (see bit 7)

3 1 - default palette loading is disabled

0 - default palette loading is disabled

2 1 - using monochrome monitor

0 - using color monitor

1 1 - gray scale summing is enabled

0 - gray scale summing is disabled

0 1 - VGA active

0 - VGA not active

��������������������Ŀ

� SAVE_PTR           �

����������������������

�

V

��������������������Ŀ

� SAVE POINTER table �->  Address of video parameter table

�                    �->  Address of parameter save area (EGA, VGA only)

�                    �->  Address of alphanumeric character set override

�                    �->  Address of graphics characrer set override

����������������������

�

V

��������������������Ŀ

�   SECONDARY SAVE   �->Address of display combination code table

�   POINTER table    �->Address of 2nd alphanumeric character set override

�     (VGA only)     �->Address of user palette profile table

����������������������

SAVE POINTER table (EGA, MCGA, VGA)

Offset  Type   Description

00h     Dword  Address of video parameter table

04h     Dword  Address of parameter save area (EGA, VGA only)

08h     Dword  Address of alphanumeric character set override

0Ch     Dword  Address of graphics characrer set override

10h     Dword  Address of SECONDARY SAVE POINTER table (VGA only)

14h     Dword  Reserved

18h     Dword  Reserved

SECONDARY SAVE POINTER table (VGA only)

Offset  Type   Description

00h     Dword  Length of SECONDARY SAVE POINTER table in bytes

02h     Dword  Address of display combination code table

06h     Dword  Address of second alphanumeric character set override

0Ah     Dword  Address of user palette profile table

0Eh     Dword  Reserved

12h     Dword  Reserved

16h     Dword  Reserved

Format of a VGA Video Parameter Table entry. The VGA Video Parameter Table

contains 29 such entries.

Offset  Type     Description

00h     Byte     Value for CRT_COLS

01h     Byte     Value for ROWS

02h     Byte     Value for POINTS

03h     Word     Value for CRT_LEN

05h     4-byte   Values for sequencer registers 1-4

09h     Byte     Value for miscellaneous output register

0Ah     25-byte  Values for CRTC registers 00h-18h

23h     20-byte  Values for attribute controller registers 00h-13h

37h     9-byte   Values for graphics controller registers 00h-08h

Parameter Save Area

Offset   Type     Description

00h      16-byte  Current contents of graphics controller palette registers

10h      Byte     Current contents of graphics controller overscan register

11h-FFh           Reserved

Alphanumeric Character Set Overrride

Offset  Type        Description

00h     Byte        Length in bytes of each character definition

01h     Byte        Character generator RAM bank

02h     Word        Number of characters defined

04h     Word        First character code in table

06h     Dword       Address of character definition table

0Ah     Byte        Number of displayed character rows

0Bh     Byte-array  Applicable video modes

Byte        FFh end of list of video modes

Graphics Character Set Overrride

Offset  Type        Description

00h     Byte        Number of displayed character rows

01h     Word        Length in bytes of each character definition

03h     Dword       Address of character definition table

07h     Byte-array  Applicable video modes

Byte        FFh end of list of video modes

Display Combination Table (see INT10 function 1Ah)

Offset  Type        Description

00h     Byte        Number of entries in table

01h     Byte        DCC table version number

02h     Byte        Maximum display type code

03h     Byte        Reserved

04h     Word-array  Each pair of bytes in the array describes a valid

display combination

User Palette Profile Table

Offset  Type        Description

00h     Byte        Underlining 1 - enable in all alphanumeric modes

0 - enable in monochrome alphanumeric mode

-1 - Disable in all alphanumeric modes

01h     Byte        Reserved

02h     Word        Reserved

04h     Word        Number of attribute controller registers in table

06h     Word        First attribute controller register number

08h     Dword       Address of attribute controller register table

0Ch     Word        Number of video DAC color registers in table

0Eh     Word        First video DAC color register number

10h     Dword       Address of video DAC color register table

14h     Byte-array  Applicable video modes

Byte        FFh end of list of video modes