Function 00h (0)        Set Video Mode

On entry:      AH         00h

AL         Video mode

Returns:       None

Registers destroyed:      AX, SP, BP, SI, DI

��������������������������������������������������������������������������

Table of Video Modes

Buffer

Mode     Type        Resolution      Adapter(s)      Colors       Address

00h     Text          40 x 25      All but MDA       16 gray       B8000

01h     Text          40 x 25      All but MDA   16 fore/8 back    B8000

02h     Text          80 x 25      All but MDA       16 gray       B8000

03h     Text          80 x 25      All but MDA   16 fore/8 back    B8000

04h   Graphics       320 x 200     All but MDA         4           B8000

05h   Graphics       320 x 200     All but MDA       4 gray        B8000

06h   Graphics       640 x 200     All but MDA         2           B8000

07h     Text          80 x 25        MDA,EGA          b/w          B0000

08h   Graphics       160 x 200        PCjr            16           B0000

09h   Graphics       320 x 200        PCjr            16           B0000

0Ah   Graphics       640 x 200        PCjr             4           B0000

0Bh   Reserved    (internal to EGA)

0Ch   Reserved    (internal to EGA)

0Dh   Graphics       320 x 200        EGA             16           A0000

0Eh   Graphics       640 x 200        EGA             16           A0000

0Fh   Graphics       640 x 350        EGA             b/w          A0000

10h   Graphics       640 x 350        EGA             16           A0000

��������������������������������������������������������������������������

Notes:         When the mode is changed, the screen is cleared.

However, do not use this service as a "clear-screen"

service, since this can take a little longer than

the normal screen-clearing routines (Service 06h or

07h).

To avoid screen clearing (PCjr, EGA, and PC

Convertible only), call with the high bit in AL set.

The cursor is not displayed in graphics mode.

Changing the video mode also changes the cursor

size.

For the PCjr, CGA, and EGA, modes 0, 2, and 5 are

identical to modes 1, 3 and 4 except color burst is

not enabled. ("Color burst" controls the display of

color on composite monitors: When color burst is on,

color is displayed; when it's off, color is not

displayed. RGB displays are not affected by the

color burst status.)

Use INT 10h, Service 0Fh to get the current video

mode.

Function 01h (1)        Set Cursor Size

Adjusts the size of the cursor by setting its start and end lines.

On entry:      AH         01h

CH         Cursor start line (0 - 31)

CL         Cursor end line   (0 - 31)

Returns:       None

Registers destroyed:      AX, SP, BP, SI, DI

��������������������������������������������������������������������������

Cursor Start and End line values (CH and CL)

Legal               Default

Adapter            Start  End         Start      End

CGA                 0     7            6         7

MDA                 0    13           11        12

EGA                 0    13          6(11)     7(12)  Depends on mode

��������������������������������������������������������������������������

Notes:         Line numbering starts at the top of the character

block (0) and increments downward. If you specify an

end line lower than the start line, a two-part

"wraparound" cursor is displayed.

There is only one cursor size for all video pages.

The cursor blink may not be disabled (it's caused by

the hardware), but you can make the cursor

invisible. To do this, position the cursor--with

Service 02h--offscreen in a nonprintable location

(such as row 25).

Setting bits 5 or 6 will cause erratic results, from

strange blinking to no cursor. Do not use this

technique to make the cursor invisible. It may not

work on some machines.

This service affects text-mode displays only. There

is no cursor display in any graphics mode.

Service 03h returns the current cursor size.

Function 02h (2)        Set Cursor Position

Sets the cursor position (row and column) for a specified display

page.

On entry:      AH         02h

BH         Display page number

DH         Row

DL         Column

Returns:       None

Registers destroyed:      AX, SP, BP, SI, DI

��������������������������������������������������������������������������

Notes:         This service can position the cursor on the active

display page or on an inactive page.

In graphics modes, this service sets the logical

position of the cursor, even though the cursor

itself is not displayed (there is no cursor display

in any graphics mode).

To make the cursor invisible, position it on row 25.

The upper left corner of the display is DH = 0h, DL

= 0h, while the lower right corner is DH = 18h (24),

DL = 27h (39) for 40-column mode or 4Fh (79) for 80-

column mode.

See Service 05h for a list of legal page numbers for

the different display modes. Because the service

does no value checking, erratic results can occur if

you specify an illegal page number. The page number

for graphics modes is 0.

Function 03h (3)        Read Cursor Position and Size

Reports the cursor position (row and column) and size for a specified

display page.

On entry:      AH         03h

BH         Display page number

Returns:       CH         Cursor start line

CL         Cursor end line

DH         Row

DL         Column

Registers destroyed:      AX, SP, BP, SI, DI

��������������������������������������������������������������������������

Cursor Start and End line values (CH and CL)

Legal               Default

Adapter            Start  End         Start      End

CGA                 0     7            6         7

MDA                 0    13           11        12

EGA                 0    13          6(11)     7(12)  Depends on mode

��������������������������������������������������������������������������

Notes:         This service can report the position of the cursor

on the active display page or on an inactive page.

Since there is only one hardware cursor, the cursor

start line (CH) and cursor end line (CL) (and hence

the size of the cursor) will be the same no matter

what page number is specified.

In graphics modes, this service reports the logical

position of the cursor, even though the cursor

itself is not displayed.

The upper left corner of the display is DH = 0h,

DL = 0h, while the lower right corner is DH = 18h

(24), DL = 27h (39) for 40-column mode or 4Fh (79)

for 80-column mode.

See Service 05h for a list of legal page numbers for

the different display modes. Because the service

does no value checking, erratic results can occur if

you specify an illegal page number. The page number

for graphics modes is 0.

Use Service 02h to set the current cursor position.

Use Service 01h to set the current cursor size.

Early versions of the Monochrome Display Adapter

(MDA) and the Compaq display adapter had a bug. They

incorrectly reported the cursor start and end scan

lines after the video adapters were initialized (the

mode had changed or the machine was powered on). The

MDA would report the cursor size as CH = 06h, CL =

07h, instead of CH = 0Bh, CL = 0Ch. The Compaq

reported the cursor size as CH = 00h, CL = 67h, for

any display mode. Note that this is a problem only

after the mode has changed or after power-up. Once

the cursor size is set (using service 01h), then

this service will report the correct cursor size.

Function 04h (4)        Read Light-Pen Position

Function 05h (5)        Set Active Display Page

Function 06h (6)        Scroll Window Up

Function 07h (7)        Scroll Window Down

Function 08h (8)        Read Character and Attribute At Cursor

Function 09h (9)        Write Character and Attribute at Cursor

Function 0Ah (10)       Write Character at Cursor

Function 0Bh (11)       Set Color Palette

Function 0Ch (12)       Write Pixel

Function 0Dh (13)       Read Pixel

Function 0Eh (14)       Write Character in Teletype (TTY) Mode

Function 0Fh (15)       Get Current Video Mode

Function 10h (16)       Set Palette Registers                      PCjr, EGA

Function 11h (17)       Character Generator                              EGA

Function 12h (18)       Alternate Select                                 EGA

Function 13h (19)       Write Character String                          many

Function 14h (20)       LCD Handler                              Convertible

Function 15h (21)       Return Physical Display Characteristics  Convertible

Function 16h (22)       Get/Set type of dislpay                          VGA

Function 1Ah

Reference

Programmer's Guide to PC & PS/2 Video Systems,

Maximum performance from the EGA, VGA, HGC and MCGA

Richard Wilton / Microsoft Press