Voice Mode + Ultra High Speed 16800 bps FOR

U-1496E and U-1496E PLUS

****************************************************

Q & A List

==============

Q-1, What is ZyXEL proprietary ultra high speed 16800 bps ?

-------------------------------------------------------------

Ans : ZyXEL 16800 bps scheme is full duplex, not like USR'HST design

which is only one way.

ZyXEL 16800 bos mode is an extesion of V.32bis, it has fast

speed shifting downward to and upward from V.32bis according to

line condition.

Q-2, May we know more detail on U-1496E Plus model ?

----------------------------------------------------------------

Ans : U-1496E Plus model has a 68000-16 running at 20MHz.  It

has larger SRAM space for digital signal processor (DSP). This

provides the power and flexibility for higher 19200 bps speed.

The possibility of higher speed depends on the complexity of

the scheme, computation power of the modem, and architecture

flexibility of the modem. ZyXEL can support 16800 bps on all

U-1496 series models now (except U1496B which does not have

1M sockets installed), for higher speed , we think the modem

needs more computation power so that not to degrade the performance.

We need more DSP memory to accommodate a more complicated scheme.

Q-3, Can user upgrade U1496E model into U1496E Plus model ?

------------------------------------------------------------------

Ans : Due to some hardware difference, ZyXEL current U1496E is not

upgradeable to PLUS model.  It will be too messy to do the

hardware modification.  The firware for the U1496E and E PLUS

is not changeable.

Q-4, What is your new feature --Caller ID in ZyXEL modem ?

------------------------------------------------------------

Ans : Normally called Caller Number Delivery (CND), this is a service

feature available from phone companies in some parts of USA and

in some other countries. With this service, the phone company

sends calling party information along with ring signal to the

called party.  The major part of this information is the telephone

number of the calling party for caller ID.  ZyXEL modems can detect

and report caller ID.

Q-5, What is Distinctive Ring in your ZyXEL product ?

---------------------------------------------------------

Ans :This is a service feature available from some phone companies

in some area of USA and other countries. You can have several

phone numbers on the same subscribe phone line. The phone company

sends a different ring signal for each phone number assigned to the

same line.

You can have your phone equipments answer only to a certain type of ring

corresponding to a certain phone number.  ZyXEL modems can be set to

answer on one of or a combination of 4 types of ring signal. There is a

problem that each country may have a different specification for

different ring types.

Q-6, What is Digitized Voice ? and CELP technology ?

--------------------------------------------------------------------------

Ans : Modem with digitzed voice capability can digitize the incoming

voice message and the computer can store it as file.  The modem can

also playback a recorded digital voice message either locally or to

the line as an announcement.

Digital voice message needs speech compression to reduce the storage

requirement. This speech compression is done in real time by the modem

and it does the decompression in playback.  A simple voice capability

using ADPCM algorithm can reduce speech data rate to 1/2 or 1/3.  We call

it advanced voice capability when it can reduce speech data rate even

further with advanced algorithm like CELP (Code Excited Linear Prediction).

Q-7, May we know more detail on VOICE mode ? Can it be used as an

answering machine ?

--------------------------------------------------------------------

Ans : In VOICE mode, the modem will digitize and compress the incoming

voice message from the telephone line and send it to the computer for

storage or forward.  It will also playable the recorded voice message

for local listening or as announcement to the phone line.

It will also detect DTMF tone signal during recording that the computer

program can detect and respond to the tone signal from a push-button phone.

The simplest use is as a digital answering machine or used in voice mail

system.  With voice mode, you combine modem, fax, and voice capability in

one signal device.

Q-8, When will V.fast standard be included in ZyXEL modems ? How fast will

the standard be ?

-----------------------------------------------------------------------------

Ans : We do not know when the V.fast standard will be settled and ZyXEL can not

promise when we will implement it.  We must have the detail of the

standard before we can have an estimation.  There are only talking paper

presentation but not a draft standard about what speed will be included

in V.fast.  Proposed speed include 24k bps, 25.6k bps, and 28.8k bps.

You need a perfect line to get that high speed , must probably you can

only get 19.2k bps speed on the real line connection.

Q-9, Will it be possible to upgrade ZyXEL to V.fast just with changing the

firmware ?

----------------------------------------------------------------------------

Ans : Please never claim, promise, or imply this possibility. We can not

promise a thing that we do not know what it really is.  V.fast may

become too complicated that we need to develop a new model for it.

Q-10, How to process U-1496 series modem firmware upgrade with 1Mbit EPROM ?

----------------------------------------------------------------------------

Ans : The 1Mbit EPROM has 32pins while the 512K EPROM has only 28 pins.

Among the four additional pins the 1M bit EPROM has, only pin 2 and

pin 32 need signal correction, pin 1 and pin 31 have no connection

inside the chip. If you put 1Mbit EPROM chip in front of you with

pin 1 on the lower left corner, the 28 pins on the right have the

same signal connection as the 512K EPROM chip.

There are three cases for 1Mbit firmware upgrade:

(1) There are already a pair of 32pin sockets installed on the modem main

board.

Just install the 1Mbit EPROMS in the 32-pin sockets.  Note that there

are no pin sockets for pin 1 and pin 31, no connection is needed for

those two pins.

(2) 28-pin sockets on the modem board but pin holes on the board exist for

pin 2 and pin 32.

Solder in two single-pin sockets for pin 2 and pin 32 and you can

install the 1M bit EPROM.  You need some skill to solder the single-pin

socket.

It is better to use a SIP 8-pin single-row pin socket as an alignment tool.

Plug the single-row pin socket into the existing socket and the single-pin

socket you want to solder and it will hold the single-pin socket in place

while you solder.

(3) 28 pin sockets on the modem board and no pin hole exists on the board for

pin 2 and pin 32.

We do NOT recommend to install 1Mbit EPROM on this case.  You can do the

following hardware modifications but it will void the warranty and it is

difficult to remove and reprogram the EPROMS.

(a) Solder a jump-wire to short pin 32 and pin 30 of the EPROM.

(b) Install the 1Mbit EPROMS on the 28-pin sockets with the left 4pins

outside the sockets.

(c) Solder a wire to connect pin 2 of the EPROM to pin 44 of the MC68000

and solder a wire to short pin 30 and pin 32 of EPROM.

(file :qa)