ESQ-1.DOC

The following is a spec sheet on the ENSONIQ ESQ-1 Digital Wave

Synthesizer, downloaded from Performing Artists Network.

- Sysop Ray Reach

Name: 79) ESQ-1 SPEC SHEET

Type: TEXT

Date: 27-MAY-1986 23:19 by ENSONIQRLW

Size: 5601

esq1 specs

Keywords: GENERAL, ESQ-1, SPEC SHEET,

ENSONIQ

ACTION> (Next, Down, Xm, List)

5/26/86

ESQ-1 DIGITAL WAVE SYNTHESIZER

Here  comes  New  Faithful.    The  Ensoniq  ESQ-1  Digital  Wave

Synthesizer  is a powerful complex-waveform synth and multi-track

MIDI  sequencer  in one compact and  great  sounding  instrument.

It's extensive voicing,  sequencing and editing capabilities make

it  the  ideal central instrument for live performance  and  MIDI

studio work.

The ESQ-1 is an 8-voice polyphonic, polytimbral synth with a rich

and  full sound that can only be accomplished with 3  oscillators

per  voice.  There is a choice of 32 multi-sampled and  synthetic

waveforms  for a nearly unlimited variety of sounds and  effects.

Included  are  sampled  waqveforms of piano,  voice and  bells  -

creating a synth unlike any other in character and realism.

Choosing  programs  is  greatly simplified with  the  ESQ-1's  80

character  fluorescent display.   Easily readable in  nearly  all

lighting conditions, the display shows 10 programs - by name, not

just  number - at any one time.   There are 40 programs on  board

with  an additional 80 cartridge programs available,  giving  you

instant access to 120 distinct sounds.

Multi-track recording is simple and straightforward with the ESQ-

1's on-board sequencer.  It features 8 discreet polyphonic tracks

and  the  ability to assign each track its own program  and  MIDI

channel.   It  also features auto-correct to 1/32 note  triplets,

metronome, auto-locate, and mixdown facility for balancing tracks

and stereo panning.  The internal storage capacity is 2400 notes,

cartridge expandable to 10,000 notes.

The  ESQ-1  is the perfect start for a MIDI studio,  the  perfect

addition  to  a current studio,  and a versatile  and  responsive

performance instrument.

------------------------------------------------------------

ESQ-1 Digital Digital Wave Synthesizer

SPECIFICATIONS

KEYBOARD

[] 61  note  (C-C)  weighted-action  keyboard  with  polyphonic

velocity sensitivity

[] Programmable split point

[] Sound layering on either or both keyboard halves

[] Polyphonic glide, fingered mono glide

VOICE ARCHITECTURE

[] 8-voice polytimbral

[] 3 digital waveform oscillators per voice

[] 32 multi-sampled and synthetic waveforms

[] 15 routable voice modulation sources

[] 3  multi-waveform  LFO's  per  voice  with  humanized  random

variation

[] 4 complex envelope generators per voice, velocity controlled

[] Programmable panning and chorusing

[] Hard sync and ring modulation (AM)

[] Analog filters, 4-pole low-pass with variable resonance

PROGRAM PARAMETERS

[] 80-character fluorescent display,  readable in  all  lighting

conditions

[] 10 program names simultaneously displayed

[] Multiple display pages for simple yet flexible programming

[] 40 internal programs, direct access

[] 80 external cartridge programs,  direct access with cartridge

installed

[] Analog  filters,   4-pole  low-pass  filters  with   variable

resonance

[] Compare mode for comparing programs

SEQUENCER

[] 8  polyphonic  tracks,  each with separate program  and  MIDI

channel

[] Tracks  can  play  internal  voices  and/or  external   MIDI

instruments

[] Up to 8 voices per track, dynamically assigned

[] Post-quantization (auto-correct to 1/32 note triplets)

[] Built in auto-locater and metronome

[] Mixdown facility for balancing individual tracks

[] 30 separate sequences, chainable into 10 songs

[] Internal storage - 2400 notes, expandable to 10,000 notes

[] External storage - on audio tape or on 3 1/2" diskettes  using

Mirage Digital Sampling Keyboard or Digital Multi-Sampler

[] sync to tape

MIDI

[] Poly, Omni, Multi and Mono modes

[] "MIDI Overflow Mode" permits slaving additional units for  16

or more voices

[] 8 simultaneous polyphonic channels with separate programs

[] Global  controllers  in mono mode for use  with  MIDI  guitar

controllers, etc.

[] MIDI song position for use with SMPTE auto-locators

[] MIDI standard remote programming

INPUTS/OUTPUTS

[] Left/Mono and Right channel audio outputs allow voices to  be

panned

[] Pedal/Control Voltage Input (allows modulation of voices from

an external source)

[] Sustain Pedal,  Sequencer Footswitch, Tape In/Out for external

sequence storage

[] MIDI In, MIDI Out

STANDARD ACCESSORIES

[] Standard  accessories  include:   Owner's/Programming  Guide,

detachable power cord, Footswitch

OPTIONAL ACCESSORIES

[] ENSONIQ  EEPPROM  Cartridge - for storing up  to  80  player-

created programs

[] ENSONIQ VOICE Cartridge - 80 factory-programmed sounds

[] Sequencer  Expander Cartridge - adds 7,600 notes to  on-board

sequencer for a total capacity of 10,000 notes

[] Sustain  Pedal/Footswitch  - for voice  sustain  or  sequence

triggering

[] CV Pedal - for voice modulation

DIMENSIONS

[] 38 3/4" (98cm) wide X 3 1/2" (9cm) high X 13 1/2" (34cm) deep

[] Weight: 28 pounds (12.7 kilograms)

WARRANTY

[] One year, parts and labor

SUGGESTED LIST PRICE-US  $1,395

Prices  and  specifications  subject to  change  without  notice.

ENSONIQ,  ESQ-1,  Mirage  Digital  Sampling Keyboard  and  Mirage

Digital Multi-Sampler are trademarks of Ensoniq Corp.

C 1986 ENSONIQ Corp

##########################################