APPENDIX C - MIRAGE MASOS MIDI IMPLEMENTATION 1.1

This  is the Mirage implemention available after booting from  the

MASOS disk.  The Real Time messages are eliminated and  additional

commands  are added to support the MASOS funtions when  used  with

the Visual Editing System.

1.0 TRANSMIT DATA.

1.1 Channel Information.

status      second      third     Description

------      ------      -----     -----------

1000NNNN    0KKKKKKK    0VVVVVVV   Note off:K[36..96],V[1..127]

1001NNNN    0KKKKKKK    0VVVVVVV   Note on: K[36..96],V[1..127]

1011NNNN    01000000    01111111   Sustain pedal on. *1,*2

1011NNNN    01000000    00000000   Sustain pedal off. *1,*2

1011NNNN    00000001    0VVVVVVV   Mod wheel amount:V[0..127].*2

1110NNNN    00000000    0VVVVVVV   Pitch bender:[0..127]

1.2 NOTES

*1 Only sent if pedal is in Sustain Pedal Mode, parameter[89].

*2 Only sent if enabled by Midi Controller Enable. par.[84].

On power up Mirage defaults to Omni on (mode 1)

and will transmit on Channel number 1 (NNNN=0)

Mirage is always in poly mode.

Mirage can be set to Omni off mode (mode 3)

2.0 RECOGNIZED RECEIVE DATA.

2.1 Channel Information.

status     second     third     Description

------     ------     -----     -----------

1000NNNN    0KKKKKKK   0VVVVVVV   Note off:K[36..96],V[127]

V=0 will be treated as V=64

1001NNNN    0KKKKKKK   0VVVVVVV   Note on:(V not=0)K[36..96],

V[127];V=0treated as note off

with V=64

1011NNNN    01000000   01111111   Sustain pedal on

1011NNNN    01000000   00000000   Sustain pedal off

1011NNNN    00000001   0VVVVVVV   Mod wheel amount:V[0..127]*1

1110NNNN    0XXXXXXX   0VVVVVVV   Pitch bend:V[0..127] *1

X is ignored.

1011NNNN    01111011   00000000   All notes off.  *2

1011NNNN    01111100   00000000   Omni mode off. All notes off.

*2

1011NNNN    01111101   00000000   Omni mode on. All notes off.

*2

1011NNNN    01111110   00000000   Mono mode on. All notes off.

*2

Mono mode off is ignored but

all notes are turned off.

1011NNNN    01111111   00000000   Poly mode on. All notes off.

*2

2.2 Notes

*1 Only recognized when enabled by Midi Controller Enable[84]

*2 These funtions as a key release for all notes. The notes

will not stop sounding instantly unless the envelope =0

The Mirage powers up in Omni on,Poly mode. In this case,

channel data is recognized on all channels.

When the Mirage is in Omni off mode, channel data is only

recognized for the channel selected by the Midi Channel

Select, parameter[82].

Controllers only affect notes played by MIDI key commands.

A MIDI Pitch Bend or Mod Wheel command will only affect notes

which sounded as a result of MIDI key commands. Notes played

on  the  local  keyboard or from the  sequencer  will  not  be

affected.

3.0 SYSTEM EXCLUSIVE FORMAT

3.1 Receive Data.

3.1.1 Mirage Command Code (front panel commands).

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00000001                         Mirage command code.

0NNNNNNN                         Up to 5 bytes of front panel

.                             message(Table 3.3)Must be a

.                             complete message, i.e.terminated

.                        with ENTER if the command requires it

01111111                         End of Command marker.

3.1.2 Program Dump Request.

This command instructs the Mirage to dump its current Programs

for the upper or lower keyboard.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

000N0011                         Program dump request;

N=0:lower,N=1:upper.

3.1.3 Configuration Parameters Dump Request.

This command instructs the Mirage to dump its current

configuration parameters.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00000000                         Configuration parameters

dump request.

3.1.4 Wavesample Dump Request.

Used to ask the Mirage to dump the current wavesample as

selected by Wavesample Select, parameter[26].

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00000100                         Wavesample dump request.

3.1.5 Wavesample Dump Absolute Request.

Used to ask the Mirage to send waveform data from the absolute

addresses specified in the message.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00001011                         Wavesample dump absolute request

000NLLLL                         N=0:lower,N=1:upper.

----

0000LLLL                         LLLLLLLL=Low byte of start addr.

----

0000HHHH

----

0000HHHH                         HHHHHHHH=High byte of start addr.

----

0000LLLL

----

0000LLLL                         LLLLLLLL=Low byte of end address

----

0000HHHH

----

0000HHHH                         HHHHHHHH=High byte of end address

----

3.1.6 Wavesample Dump Absolute Data.

Used to send or receive data to or from the Mirage

from the pointers passed in the message.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00001100                    Wavesample dump absolute data message

000NLLLL                         N=0:lower,N=1:upper.

----

0000LLLL                         LLLLLLLL=Low byte of start addr.

----

0000HHHH

----

0000HHHH                         HHHHHHHH=High byte of start addr.

----

0000LLLL

----

0000LLLL                         LLLLLLLL=Low byte of end address

----

0000HHHH

----

0000HHHH                         HHHHHHHH=High byte of end address

----

Each data byte sent as 2 nybbles with

the MS nybble clear. Low nybble is sent

first, i.e. 0000LLLL then 0000HHHH.

0CCCCCCC                Checksum: formed as a modulo 128

add of all data nybbles plus the nybbles

of the start and end addresses.

3.1.7 Program Dump Data.

Used to send upper or lower programs to the Mirage.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

000N0101                         N=0:lower,N=1:upper

.                            Program data(table 3.4)

.

.

.

3.1.8 Wavesample Dump Data.

Used to transfer data into the currently selected wavesample

on the Mirage. Note: a Wavesample should not be transmitted

and received simultaneously. as this will have unpredictable

results.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00000110                         Wavesample Dump message

0000CCCC                         Page count LS nybble.

0000CCCC                         Page count MS nybble.

.                     Each data byte sent as 2 nybbles with

.                     the MS nybble clear. Low nybble is sent

.                     first,i.e. 0000LLLL then 0000HHHH.

.

0CCCCCCC                 Checksum: formed as a modulo 128

add of each nybble and the page count

3.1.9 Perform Wavesample Manipulation Command.

Used to perform MASOS wavesample manipulation functions.

When the fuction is completed an acknowledge is transmitted

by the Mirage.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00001111                         Manipulation function

command code.

000N0FFF                         N=0:lower,N=1:upper.

FFF=Command function code.*1

0000LLLL                         Source Start address

----

0000LLLL                         LS nybble through MS nybble.

0000HHHH                         Address=HHHHHHHH LLLLLLLL.

----                                     ----     ----

0000HHHH

0000LLLL                         Source End address

----

0000LLLL                         LS nybble through MS nybble.

0000HHHH                         Address=HHHHHHHH LLLLLLLL.

----                                     ----     ----

0000HHHH

0000FFFF                         Scale Start Factor(FFFFFFFF).

----                                                ----

0000FFFF                    Always transmitted whether used or not

0000FFFF                         Scale End Factor(FFFFFFFF).

----                                              ----

0000FFFF                    Always transmitted whether used or not

0000LLLL                    Destination Start address.

----

0000LLLL                    Always transmitted whether used or not

0000HHHH                    Address=HHHHHHHH LLLLLLLL.

----                                ----     ----

000NHHHH                    N=1:dest.upper,N=0:dest.lower

3.1.10 Notes

*1 Function Code definitions:

0 = Copy Source to Destination.

1 = Scale Source according to Scale Factors.

2 = Add Source to Destination, place result in Destination.

3 = Invert Source.

4 = Reverse Source.

5 = Replicate Source page.

6 = Rotate Current Wavesample Left.

7 = Rotate Current Wavesample Right.

3.2 TRANSMIT DATA.

3.2.1 Program Parameter Message.

This message is sent whenever a parameter is changed from

the front panel on the Mirage. *1

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00001101                         Program parameter message

000N00PP                         N=0:lower,N=1:upper,PP=Prog#

0XXXXXXX                         Parameter #

0000VVVV                         Value LS nybble.

0000VVVV                         Value MS nybble.

3.2.2 Wavesample Parameter Message

This message is sent whenever a wavesample parameter is changed

from the front panel of the Mirage. *1

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00001110                         Wavesample parameter message

N=0:lower,N=1:upper.

000N0SSS                         SSS=wavesample #

0XXXXXXX                         Parameter #

0000VVVV                         Value LS nybble.

0000VVVV                         Value MS nybble.

3.2.3 Program Status Message.

Sent by Mirage when a Program number is changed from the front

panel.  *1

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00000111                         Prog. change status message

000N00PP                         N=0:lower,N=1:upper,PP=Prog#

3.2.4 Wavesample Status Message.

Sent by Mirage when a new wavesample is selected

from the front panel.  *1

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00001000                         Wavesample Status Message

N=0:lower,N=1:upper:upper

000N0SSS                         SSS=wavesample #

3.2.5 Wavesample Dump Received without Error (Ack).

Sent by the Mirage when the checksum of a received wavesample

dump is good. Also sent when a wavesample function is

completed.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00001001                         Wavesample Ack message

3.2.6 Wavesample Dump Received with Error (NAK)

Sent by the Mirage when the checksum of a received wavesample

dump is bad.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00001010                         Wavesample Nak message

3.2.7 Program Dump Data.

Used to send the upper or lower program from the Mirage.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

000N0101                         Program dump message.

N=0:lower,N=1:upper.

.                             Program data(Table 3.4)

.                             sent as two nybbles with MS

.                             nybble clear, low nybble first,

.                             i.e.0000LLLL then 0000HHHH.

3.2.8 Configuration Dump Data.

Used to send the configuration parameters from the Mirage.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00000010                         Configuration dump data message

.                             Configuration dump data(Table3.5)

.                             sent as two nybbles with MS

.                             nybble clear, low nybble first,

.                             i.e.0000LLLL then 0000HHHH.

3.2.9 Wave Table Dump Data.

Used to send the current wavesample from the Mirage.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00000110                         Wave table dump message

0000CCCC                         Page count LS nybble

0000CCCC                         Page count MS nybble

.

.                             Wavesample data

.                             sent as two nybbles with MS

.                             nybble clear, low nybble first,

.                             i.e.0000LLLL then 0000HHHH.

0CCCCCCC                         Checksum of wavesample dump.

Formed as a modulo 128 add of

the data nybble and the page

count

3.2.10 Notes

*1  Only  transmitted when enabled by  External  Computer  Port

enable parameter [91].

3.3 Keypad Number Decoding Table.

Hex Code  Dec Code  Key Label    Hex Code  Dec Code  Key Label

00        00      0/PROG         14        20      REC SEQ

01        01      1              15        21      PLAY SEQ

02        02      2              16        22      LOAD SEQ

03        03      3              17        23      SAVE SEQ

04        04      4              10        16      LOAD UPPER

05        05      5              11        17      LOAD LOWER

06        06      6              12        18      SAMPLE UPPER

07        07      7              13        19      SAMPLE LOWER

08        08      8              0C        12      PARAM

09        09      9              0D        13      VALUE

0A        10      ENTER/START    0E        14      (UP ARROW)

0B        11      CANCEL/STOP    0F        15      (DOWN ARROW)

3.4 Program Dump Table Format.

The Program dump contains eight Wavesample Control Blocks, eight

Segment Lists and four Program Parameter Blocks. The data is

transferred in nybbles and reassembled to form a block of bytes.

Byte Offset     Lenght   Description

0             1       Sound Rev Level

1             24      Wavesample Control Block 1

25            24      Wavesample Control Block 2

49            24      Wavesample Control Block 3

73            24      Wavesample Control Block 4

97            24      Wavesample Control Block 5

121           24      Wavesample Control Block 6

145           24      Wavesample Control Block 7

169           24      Wavesample Control Block 8

193           32      Segment List, Wavesample 1

225           32      Segment List, Wavesample 2

257           32      Segment List, Wavesample 3

289           32      Segment List, Wavesample 4

321           32      Segment List, Wavesample 5

353           32      Segment List, Wavesample 6

385           32      Segment List, Wavesample 7

417           32      Segment List, Wavesample 8

449           32      Segment List, spare

481           36      Parameter Block, Program 1

517           36      Parameter Block, Program 2

553           36      Parameter Block, Program 3

589           36      Parameter Block, Program 4

Total Lenght:  625 bytes

WAVESAMPLE CONTROL BLOCK  Lenght: 24 bytes

Each Wavesample Control Block contains pointers which are used

internally by the Mirage operating system.

WARNING: no wavesample parameters should be changed

externally. The Mirage performs internal housekeeping tasks

when any of these values are changed. Use front panel command

equivalents to change them. Consider these control blocks to

be READ ONLY.

Byte Offset    Parameter #   Parameter Name         Range

0                          SAMPLE START POINTER

2                          SAMPLE END POINTER

4                          LOOP START POINTER

6                          LOOP END POINTER

8               [65]       LOOP SWITCH            0..1

9               [67]       COARSE TUNE            0..7

10              [68]       FINE TUNE              0..255

11              [69]       RELATIVE AMPLITUDE     0..63

12              [70]       RELATIVE FILTER FREQ.  0..198

13              [71]       MAX. FILTER FREQ.      0..198

14              [72]       TOP KEY                0..60

15              [60]       WAVESAMPLE START       0..FE

16              [61]       WAVESAMPLE END         1..FF

17              [62]       LOOP START             0..FE

18              [63]       LOOP END               1..FF

19              [64]       LOOP END FINE          0..FF

20                         FREE RUN FLAG          0..1

21                         spare 3 bytes          0

SEGMENT LIST           Lenght: 32 bytes

The Segment List is a set of operating system variables which

control the digital oscillators.

As with the Wavesample Control Blocks, the Segment Lists should

not be changed externally and sent back to the Mirage.

PROGRAM PARAMETER BLOCK   Lenght: 36 bytes

Byte Offset    Parameter #   Parameter Name         Range

0              [28]        Mono Mode Switch       0..1

1              [31]        LFO Frequency          0..99

2              [32]        LFO Depth              0..99

3              [33]        Osc Detune             0..99

4              [34]        Osc Mix                0..252

5              [35]        Mix Vel Sens.          0..124

6              [36]        Filter Cutoff Freq.    0..198

7              [37]        Resonance              0..160

8              [38]        Filter Kybd Tracking   0..4

9                          spare                  0

10             [27]        Intial Wavesample      0..7

11             [28]        Mix Mode Switch        0..1

12             [40]        A Filter Envelope      0..31

13             [41]        P                      0..31

14             [42]        D                      0..31

15             [43]        S                      0..31

16             [44]        R                      0..31

17             [45]        Av                     0..124

18             [46]        Pv                     0..124

19             [47]        Dk                     0..124

20             [48]        Sv                     0..124

21             [49]        Rv                     0..124

22             [50]        A Amplitude Envelope   0..31

23             [51]        P                      0..31

24             [52]        D                      0..31

25             [53]        S                      0..31

26             [54]        R                      0..31

27             [55]        Av                     0..124

28             [56]        Pv                     0..124

29             [57]        Dk                     0..124

30             [58]        Sv                     0..124

31             [59]        Rv                     0..124

32                         spare 4 byte           0

3.5 Configuration Dump Table.

Byte Offset    Parameter #   Parameter Name         Range

0                          Dummy byte must be zero

1              [21]        Master Tune            0..99

2              [22]        Pitch Bend Range       0..34

3              [23]        Velocity Sensitivity   0..63

4              [24]        Upper/Lower Balance    0..126 *1

5              [25]        Program Link Switch    0..1

6              [73]        Sample Time Adjust     20..99

7              [74]        Input Filter Freq.     0..198 *1

8              [75]        Mic/Line Switch        0..1

9              [76]        Sampling Threshold     0..126 *1

10             [77]        Multisample Switch     0..1

11             [81]        Midi Omni-mode Flag    0..1

12             [82]        Midi Channel           0..15

13             [83]        Midi Thru-mode switch  0..1

14             [84]        Midi Mod Enable Switch 0..1

15             [85]        Source Start MSB       0..255

16             [86]        Source Start LSB       0..255

17             [87]        Source End MSB         0..255

18             [88]        Source End LSB         0..255

19             [89]        Destination  MSB       0..255

20             [90]        Destination LSB        0..255

21             [94]        Destination Bank       0..1

22             [95]        Scale Start Factor     0..255

23             [96]        Scale End Factor       0..255

24             [91]        External Comp. Switch  0..1

25             [92]        Baud Rate Switch       0..1

26             [93]        Cartridge Filter Freq. 0..25

27             [97]        Spare                  0..255

3.5.1 Notes

*1 All parameters with this mark are displayed divided by 2.

They must always have their least significant bit reset.

APPENDIX D - MIRAGE MIDI IMPLEMENTATION 1.1

This is the normal Mirage MIDI implementation which is available

when the Mirage is booted from a Sound Disk or Blank Formatted

Diskette.

1.0 TRANSMIT DATA.

1.1 Channel Information.

status      second      third     Description

------      ------      -----     -----------

1000NNNN    0KKKKKKK    0VVVVVVV   Note off:K[36..96],V[1..127]

1001NNNN    0KKKKKKK    0VVVVVVV   Note on: K[36..96],V[1..127]

1011NNNN    01000000    01111111   Sustain pedal on. *1,*2

1011NNNN    01000000    00000000   Sustain pedal off. *1,*2

1011NNNN    00000001    0VVVVVVV   Mod wheel amount:V[0..127].*2

1110NNNN    00000000    0VVVVVVV   Pitch bender:[0..127]

1011NNNN    01111011    00000000   All Notes Off, transmitted at

end of sequence.

1.2 Real Time Data

11111000                  Timing clock sent when internal

seq. clock is selected or when MIDI

clock is selected and a MIDI clock is

received. Also sent when external

clock is selected and an external

clock occurs.

1.3 Note

*1 Only sent if pedal is in Sustain Pedal Mode, parameter[89].

*2 Only sent if enabled by Midi Controller Enable. par.[84].

On power up Mirage defaults to Omni on (mode 1)

and will transmit on Channel number 1 (NNNN=0)

Mirage is always in poly mode.

Mirage can be set to Omni off mode (mode 3)

2.0 RECOGNIZED RECEIVE DATA.

2.1 Channel Information.

status     second     third     Description

------     ------     -----     -----------

1000NNNN    0KKKKKKK   0VVVVVVV   Note off:K[36..96],V[127]

V=0 will be treated as V=64

1001NNNN    0KKKKKKK   0VVVVVVV   Note on:(V not=0)K[36..96],

V[127];V=0treated as note off

with V=64

1011NNNN    01000000   01111111   Sustain pedal on

1011NNNN    01000000   00000000   Sustain pedal off

1011NNNN    00000001   0VVVVVVV   Mod wheel amount:V[0..127]*1

1110NNNN    0XXXXXXX   0VVVVVVV   Pitch bend:V[0..127] *1

x is ignored.

1011NNNN    01111011   00000000   All notes off.  *2

1011NNNN    01111100   00000000   Omni mode off. All notes off.

*2

1011NNNN    01111101   00000000   Omni mode on. All notes off.

*2

1011NNNN    01111110   00000000   Mono mode on. All notes off.

*2

Mono mode off is ignored but

all notes are turned off.

1011NNNN    01111111   00000000   Poly mode on. All notes off.

*2

2.2 Real Time Data.

11111000                 Timing clock recognized when MIDI

clock is selected.

11111010                 Start sequencer.

11111011                 Continue sequencer.

11111100                 Stop sequencer.

2.3 Notes

*1 Only recognized when enabled by Midi Controller Enable[84]

*2 These funtions as a key release for all notes. The notes

will not stop sounding instantly unless the envelope =0

The Mirage powers up in Omni on,Poly mode. In this case,

channel data is recognized on all channels.

When the Mirage is in Omni off mode, channel data is only

recognized for the channel selected by the Midi Channel

Select, parameter[82].

Controllers only affect notes played by MIDI key commands.

A MIDI Pitch Bend or Mod Wheel command will only affect notes

which sounded as a result of MIDI key commands. Notes played

on  the  local  keyboard or from the  sequencer  will  not  be

affected.

3.0 SYSTEM EXCLUSIVE FORMAT

3.1 Receive Data.

3.1.1 Mirage Command Code (front panel commands).

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00000001                         Mirage command code.

0NNNNNNN                         Up to 5 bytes of front panel

.                             message(Table 3.3)Must be a

.                             complete message, i.e.terminated

.                        with ENTER if the command requires it

01111111                         End of Command marker.

3.1.2 Program Dump Request.

This command instructs the Mirage to dump its current Programs

for the upper or lower keyboard.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

000N0011                         Program dump request;

N=0:lower,N=1:upper.

3.1.3 Wavesample Dump Request.

Used to ask the Mirage to dump the current wavesample as

selected by Wavesample Select, parameter[26].

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00000100                         Wavesample dump request.

3.1.4 Program Dump Data.

Used to send upper or lower programs to the Mirage.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

000N0101                         N=0:lower,N=1:upper

.                            Program data(table 3.4)

.

.

.

3.1.5 Wavesample Dump Data.

Used to transfer data into the currently selected wavesample

on the Mirage. Note: a Wavesample should not be transmitted

and received simultaneously. as this will have unpredictable

results.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00000110                         Wavesample Dump message

0000CCCC                         Page count LS nybble.

0000CCCC                         Page count MS nybble.

.                     Each data byte sent as 2 nybbles with

.                     the MS nybble clear. Low nybble is sent

.                     first,i.e. 0000LLLL then 0000HHHH.

.

0CCCCCCC                 Checksum: formed as a modulo 128

add of each nybble and the page count

3.2 TRANSMIT DATA.

3.2.1 Wavesample Dump Received without Error (Ack).

Sent by the Mirage when the checksum of a received wavesample

dump is good. Also sent when a wavesample function is

completed.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00001001                         Wavesample Ack message

3.2.2 Wavesample Dump Received with Error (NAK)

Sent by the Mirage when the checksum of a received wavesample

dump is bad.

11110000                         System Exclusive.

00001111                         ENSONIC Code.

00000001                         Mirage Code.

00001010                         Wavesample Nak message

3.3 Keypad Number Decoding Table.

Hex Code  Dec Code  Key Label    Hex Code  Dec Code  Key Label

00        00      0/PROG         14        20      REC SEQ

01        01      1              15        21      PLAY SEQ

02        02      2              16        22      LOAD SEQ

03        03      3              17        23      SAVE SEQ

04        04      4              10        16      LOAD UPPER

05        05      5              11        17      LOAD LOWER

06        06      6              12        18      SAMPLE UPPER

07        07      7              13        19      SAMPLE LOWER

08        08      8              0C        12      PARAM

09        09      9              0D        13      VALUE

0A        10      ENTER/START    0E        14      (UP ARROW)

0B        11      CANCEL/STOP    0F        15      (DOWN ARROW)

3.4 Program Dump Table Format.

The Program dump contains eight Wavesample Control Blocks, eight

Segment Lists and four Program Parameter Blocks. The data is

transferred in nybbles and reassembled to form a block of bytes.

Byte Offset     Lenght   Description

0             1       Sound Rev Level

1             24      Wavesample Control Block 1

25            24      Wavesample Control Block 2

49            24      Wavesample Control Block 3

73            24      Wavesample Control Block 4

97            24      Wavesample Control Block 5

121           24      Wavesample Control Block 6

145           24      Wavesample Control Block 7

169           24      Wavesample Control Block 8

193           32      Segment List, Wavesample 1

225           32      Segment List, Wavesample 2

257           32      Segment List, Wavesample 3

289           32      Segment List, Wavesample 4

321           32      Segment List, Wavesample 5

353           32      Segment List, Wavesample 6

385           32      Segment List, Wavesample 7

417           32      Segment List, Wavesample 8

449           32      Segment List, spare

481           36      Parameter Block, Program 1

517           36      Parameter Block, Program 2

553           36      Parameter Block, Program 3

589           36      Parameter Block, Program 4

Total Lenght:  625 bytes

WAVESAMPLE CONTROL BLOCK  Lenght: 24 bytes

Each Wavesample Control Block contains pointers which are used

internally by the Mirage operating system.

WARNING: no wavesample parameters should be changed

externally. The Mirage performs internal housekeeping tasks

when any of these values are changed. Use front panel command

equivalents to change them. Consider these control blocks to

be READ ONLY.

Byte Offset    Parameter #   Parameter Name         Range

0                          SAMPLE START POINTER

2                          SAMPLE END POINTER

4                          LOOP START POINTER

6                          LOOP END POINTER

8               [65]       LOOP SWITCH            0..1

8               [65]       LOOP SWITCH            0..1

9               [67]       COARSE TUNE            0..7

10              [68]       FINE TUNE              0..255

11              [69]       RELATIVE AMPLITUDE     0..63

12              [70]       RELATIVE FILTER FREQ.  0..198

13              [71]       MAX. FILTER FREQ.      0..198

14              [72]       TOP KEY                0..60

15              [60]       WAVESAMPLE START       0..FE

16              [61]       WAVESAMPLE END         1..FF

17              [62]       LOOP START             0..FE

18              [63]       LOOP END               1..FF

19              [64]       LOOP END FINE          0..FF

20                         FREE RUN FLAG          0..1

21                         spare 3 bytes          0

SEGMENT LIST           Lenght: 32 bytes

The Segment List is a set of operating system variables which

control the digital oscillators.

As with the Wavesample Control Blocks, the Segment Lists should

not be changed externally and sent back to the Mirage.

PROGRAM PARAMETER BLOCK   Lenght: 36 bytes

Byte Offset    Parameter #   Parameter Name         Range

0              [28]        Mono Mode Switch       0..1

1              [31]        LFO Frequency          0..99

2              [32]        LFO Depth              0..99

3              [33]        Osc Detune             0..99

4              [34]        Osc Mix                0..252

5              [35]        Mix Vel Sens.          0..124

6              [36]        Filter Cutoff Freq.    0..198

7              [37]        Resonance              0..160

8              [38]        Filter Kybd Tracking   0..4

9                          spare                  0

10             [27]        Intial Wavesample      0..7

11             [28]        Mix Mode Switch        0..1

12             [40]        A Filter Envelope      0..31

13             [41]        P                      0..31

14             [42]        D                      0..31

15             [43]        S                      0..31

16             [44]        R                      0..31

17             [45]        Av                     0..124

18             [46]        Pv                     0..124

19             [47]        Dk                     0..124

20             [48]        Sv                     0..124

21             [49]        Rv                     0..124

22             [50]        A Amplitude Envelope   0..31

23             [51]        P                      0..31

24             [52]        D                      0..31

25             [53]        S                      0..31

26             [54]        R                      0..31

27             [55]        Av                     0..124

28             [56]        Pv                     0..124

29             [57]        Dk                     0..124

30             [58]        Sv                     0..124

31             [59]        Rv                     0..124

32                         spare 4 byte           0