Computers and Music

By Robert A. Moog

[Robert A. Moog is the inventor of the practical music synthesizer and

president of Big Briar, Inc., a Leicester, North Carolina, firm specializing

in the design of custom electronic instruments.]

For some of us, the idea of an electronic muse is scary; after all, music is

an essentially human activity, while electronic equipment, especially the

computer, is "mechanical" and "unnatural."  Throughout history, however, music

has been closely linked to technology.  Except for the human voice, the

instruments of music-making have always been "high-tech" in their time.

THE ELECTRONIC MUSE

-------------------

The violin, pipe organ, and trumpet are complex constructions that were as

"unnatural" when they were first developed as the computer is today.  The

piano and saxophone, those vital elements of our musical experience, were

triumphs of manufacturing technology a century ago.  Musical instrument

designers have always employed the most advanced technology of their time.

Now, in our time, electronic and computer technologies are preferred for new

musical instrument development.

But this is not to say that musicians are embracing electronics just because

it's the "latest thing."  As a group, musicians favor instruments that

a) sound good and b) offer musically useful ways of manipulating sound.

Increasingly, musicians are drawn to electronic instruments -- not because

they're easy to play or sound like traditional acoustic instruments, but

because they offer new tone colors and new ways of making music.

What's more, musicians have been experimenting with electronic instruments

ever since the first vacuum tube was invented three-quarters of a century ago.

Even before that, musicians and musical instrument builders were collaborating

to harness the forerunners of electronics and computers to the service of the

muse.

For a growing number of musicians, computer technology is the greatest

advance since the invention of catgut.  Music is a form of communication -- of

organizing and transmitting data.  The "alphabet" of music consists of notes.

Melodies, chords, and rhythmic patterns are the "words" and "phrases" of

music.  Just as computers can generate "characters" to make text or a graphic

design, they can also process a stream of numbers that represent a sound

waveform.  And just as word processing programs endear computers to

wordsmiths, today's composers, performers and music teachers are all exploring

the computer's ability to handle musical information.

If you understand the general principles of computer operation and if you like

to listen to music, you'll have no trouble following the many ways that

digital technology and computers can be used to make music.  Just keep in

mind that computer music is a natural extension of traditional music and uses

programs that are only slightly different from your basic word processor or

data handler.  As we shall see, simulating a multitrack recording studio on

your monitor screen is done with software that is directly related to the

program used to "compose" this article.

MUSICAL DIGITS

--------------

All sounds, musical or otherwise, are vibrations of the air, at rates of

roughly 20 to 20,000 times a second.  If the vibrations repeat regularly, the

sounds are pitched (like a guitar or clarinet tone).  If a sound vibration

does not repeat regularly, then it sounds pitchless or "noisy" (like a cymbal

crash).  In a pitched sound, the rate of repetition is called its frequency;

the greater the frequency, the higher the musical pitch of the tone.  The

strength of the vibration is called its amplitude; the greater a sound's

amplitude, the louder it is.

The shape of a vibration is called the waveform.  You can think of the

waveform of a sound as the graph of the air pressure at a particular point

over time.  The waveform is an abstraction that we use to describe the sound.

It happens to be n abstraction that has a lot to do with the tone's perceived

quality.

A loudspeaker (speaker, for short) is a device that converts electronic

vibrations into sound.  In talking about electronic music and computers, we

generally refer to electrical waveforms that exist inside an instrument's

circuitry.  When we refer to these waveforms as if they were sounds, we assume

there's a speaker somewhere and that we're using it to produce the sounds.

A personal computer may contain its own small speaker (e.g. the Apple ][), may

use the speaker of the TV to which it is connected (e.g. the Atari or

Commodore 64) or may require connection to an external sound system like most

high-quality music synthesizers.  Most electronic pianos, organs, and

synthesizers use "analog" circuits that produce smooth waveforms.  Digital

computer circuits, on the other hand, work by switching on and off.

How does a computer produce a musical tone?  In most computers, you can turn

the speaker on and off as is it were, say, a memory location.  You can produce

a tone by writing a simple program to  a) turn the speaker current on, b) wait

a very short time, c) turn the speaker off, d) wait again, and e) repeat the

above steps a specified number of times.  The waiting time determines the

pitch of the tone, while the number of cycles determines its duration.

If the "speaker off" and "speaker on" times are the same, the resultant

waveform is called a square wave and the musical quality is somewhat hollow,

like that of a clarinet.  If the "speaker on" and "speaker off" times are not

the same, the waveform is called "rectangular" and the quality may be

saxophone- or oboe-like.  If the "speaker on" and "speaker off" times are

programmed to change randomly, the resultant sound is a pitchless noise.

While any other computer can produce square and rectangular waves, only those

equipped with sound and synthesizer circuits can produce more complex

waveforms.  Some sound synthesizers are built on single integrated circuit

chips that can be programmed to produce a wide variety of waveforms and

envelopes.  (The envelope of a sound is its outline as it builds up, sustains

and dies out.)

Other synthesizers are built on circuit cards that plug into the computer or

may be completely separate or peripherals.  Computer programs enable musicians

to design their own sounds.  Musicians think of this type of programming as

"building an instrument": the "instruments" exist as data that define

waveforms and envelopes--and may therefore be stored in "libraries" on disk or

tape.

Computer-controlled sound synthesizers may be all-digital (the waveform itself

is generated from digital data), all-analog (waveforms are produced

continuously by analog circuitry that responds to digital instructions) or a

combination of the two (waveforms are converted from digital to analog form,

then passed through analog circuitry).  Digital circuits that produce waveforms

are made up of steps that are often audible. Both methods of synthesis have

their advantages and limitations; some musicians prefer the smooth,

distortion-free analog waveforms, while others favor the accuracy and

versatility of digital generators.

PLAYING THE PC

--------------

There are simple programs for most personal computers to make scales and

melodies through the computer's speaker.  To use a typical program of this

kind, you type in codes for the pitches and durations of the notes.

More sophisticated programs enable you to vary the rectangular wave tone

color, adjust the overall tempo, produce trills and glides, and store tunes

that you have programmed on disk or cassette tape.  Music Maker, a software

package for the Apple ][, produces the illusion of two notes being played

simultaneously, generates sound effects as well as musical tones, and displays

a colorful animated video pattern in time with the music.  Programs like Music

Maker don't produce complex or high-quality tones; their main uses are

educational and recreational -- you can learn a good deal about programming,

train your ear and have a lot of fun, for a very small investment in addition

to your computer.

By using a computer with a built-in sound synthesizer, or adding a digitally

controlled synthesizer peripheral, you can make music with a wide variety of

interesting tone colors.  The Commodore 64 has one of the most versatile

built-in synthesizers of any currently available personal computer.  The "64"

uses a proprietary chip that produces three tones with programmable waveform

and envelope.  The chip also contains an analog filter, a device that changes

the tone color by emphasizing some of the sound's overtones and cutting out

others.  The resulting range and quality of sound rival that of some of the

analog keyboard synthesizers available in musical instrument stores.

Some of the most musically advanced computer programs are designed around the

Mountain Computer Musicsystem, an eight-voice digital tone generator for the

Apple ][.  Among the more popular are the Alpha Syntauri and the Soundchaser

systems.  Both use the Musicsystem in combination with a professional-style

four- or five-octave music keyboard and their own operating software.

With either of these systems you can make up your own sounds, play them from

the music keyboard and record the keyboard performance.  Since one part of the

software sets the Musicsystem up to produce the desired tone colors and

another part captures and stores the keyboard performance, you can play back

your keyboard performance with a variety of tone colors, pitch ranges, and

speeds.  Both the Alpha Syntauri and the Soundchaser can implement the basic

functions of a multitrack recording studio.  You can record a keyboard

performance on one "track," then play that track back while recording

subsequent tracks.  The Syntauri Metatrak program, for instance, lets you

record up to sixteen tracks, then play them back simultaneously.  Fast

Forward, Rewind, Record, and Erase functions are implemented by typing one or

two characters on the computer keyboard.

To a musician, using Metatrak (or the Soundchaser Turbotracks program) is

closely akin to using a conventional tape recorder.  To the average computer

user, programs that implement a multitrack recorder are actually file

management systems with real-time merging capability.  Whichever way you look

at it, Metatrak, Turbotracks, and related programs offer potent musical

resources to pro musicians -- and a lot of musical enjoyment to amateurs.

In addition to simulating multitrack recorders, computer-based music systems

offer other functions that are important to musicians.  Music-teaching

programs are available for both the Soundchaser and the Alpha Syntauri.

Soundchaser's Musictutor package contains an array of ear-training exercises

that not only sharpen your ears, but keep track of your musical progress.

Syntauri's Simply Music program will teach you how to play a keyboard

instrument in a variety of styles and at a pace that suits you.  Once your

keyboard chops are in good shape, you can convert your keyboard performances

directly to a printed score with Syntauri's Composer's assistant, a software

package that enables a dot-matrix printer to produce conventional music

notation.

COMPUTER CONTROL

----------------

The Roland Compumusic CMU-800R is an example of an analog musical sound

generator designed for computer control.  The Compumusic uses electronic

piano, organ, and synthesizer circuits to produce realistic percussion, bass,

"rhythm" guitar and melody voices through your sound system.  Using ROland-

supplied software, you program the melody, harmony, and rhythm from the

computer keyboard.  Then you "mix" the sounds by manipulating the volume

sliders on the Compumusic unit while the computer "plays" the complete piece

of music that you've programmed.  The computer is not able to program the

Compumusic waveforms since these are determined by the unit's analog

circuitry.  The advantages of Compumusic are in its high sound quality and

hands-on-the-knobs control.

Musicians have expressed the desire to control a regular electronic keyboard

by means of a computer.  An increasing number of electronic pianos, organs,

and synthesizers are being adapted for computer control.  For this purpose,

the musical instrument industry has developed an interface called MIDI, the

Musical Instrument Digital Interface.  MIDI allows electronic instruments,

computers, and similar devices to be connected with a minimum of fuss.  This

means that, if your computer itself is equipped with a MIDI peripheral and the

necessary software, you can use your computer to control any MIDI-equipped

musical instrument.  You can even combine instruments into a computer-

controlled "orchestra."

Will computers ever completely replace human musicians?  A number of

traditional instrumentalists, upon seeing entire string and horn sections

replaced by synthesizers and other digital instruments, have asked this

question.  The answer lies in the fact that music is and always will be an

aesthetic and emotional experience for humans and not for computers.  There

will always be musicians as long as there is a song in our hearts.

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X