The mouse and the Psion Series 3(a) .. by Marko Schuster 1993

-------------------------------------------------------------

(Translated into english in 1994)

Files in Archive: (files are german words..)

Maus3.*      - Example for Series3

Maus3a.*     - Example for Series3a

Kreuz.pic    - Sprite for S3a, must be in M:\pic\sprites (see Maus3a.opl!)

Note to the prg: drawing by pressing the left mouse-button

A mouse on the S3? This should work? Of course.. Of course the prg. must

be programmed to accept a mouse as input. That means, you can only

use a mouse if a programmer writes his prg. so that you can use the

mouse. Assembler-hackers may try to get into the system-keyboard-interrupt

and make a prg. to emulate the cursor-keys for example and so be able

to use the mouse in all applications (mousebuttons may be ESC/MENU/ENTER..)

The mouse .. it must be a serial PC-mouse. Mouses for Amiga/C64/Atari/..

will not work. To use a mouse, it must be connected with the serial

link of the S3(a). But between both you need a (null)modem-adapter

(changes SD&RD, DTR&DSR, RTS&CTS). An other solution is to make your

own cable or to change the pins directly on the S3-Link-cable (for example,

i have cut the 9pin-connector and have made a 9pin-connecter which i can

directly connect with my mouse. Note if you do this: be sure that the

other connector is NOT connected to your PC! It wouldn't work.. if

all Serial links have the same colors on their pins, such a new 9pin-

connecter must be make as followed:

Pin Color

1  empty

2  brown

3  red

4  black

5  blue & shield

6  green

7  yellow

8  orange

9  empty

!! I make no warranty if this is correct. I make no warrenty if you damage

your PSION !!

The problem:

A serial mouse works with 1200 Baud, so we must open the serial with

1200 Baud. 2. There seems to be different mouses. I have an Amiga and a

PC-Card, so i don't know much about PC-mouses, but on my mouse i can

choose between "2-buttons (Microsoft-Mouse)" and "3-buttons (Mouse System-

Mouse)". All informations I know are related to the last one! You must have

a 3-button-mouse if you want to try my prgs.!

Actually I don't know how to check if there is a char on the serial port,

so the prg. cannot get keypresses/events if it waits for an input on

the serial port !!

The mouse sends for each movement 5 Bytes. Each movement may contain

more pixels & more directions. The movements are in the last 4 Bytes,

in the first byte is the status of the mouse-buttons. These codes

(all decimal!) can be found at the first byte:

128 : all mouse buttons pressed

129 : left and middle button pressed

130 : left and right pressed

131 : left pressed

132 : middle and right pressed

133 : middle pressed

134 : right pressed

135 : no button pressed

Directions&Movements: look in the OPL-Source for these informations.

..written by .. Marko Schuster 12/93, translated to english 2/94

EMAIL to: ns103@fim.uni-erlangen.de