X-15, WORLD'S FIRST HYPERSONIC RESEARCH AIRCRAFT

The X-15 was a small rocket-powered aircraft, 50 feet long

with a wingspan of 22 feet.  It had a conventional fuselage, but

an unusual wedge-shaped vertical tail, thin stubby wings and

unique side fairings that extended along the fuselage.  The X-15

weighed about 14,000 pounds empty and approximately 34,000 pounds

at launch.  The rocket engine, which was controlled by the pilot,

was capable of developing 60,000 pounds of thrust.

The X-15 research aircraft was developed to provide inflight

information and data on aero-thermodynamics, aerodynamics,

structures, flight controls and the physiological aspects of

high-speed, high-altitude flight.  A follow-on program utilized

the aircraft as a testbed to carry various scientific experiments

beyond the Earth's atmosphere on a repeated basis.

For flight in the dense air of the usable atmosphere, the X-

15 utilized conventional aerodynamic controls.  For flight

outside of the appreciable Earth's atmosphere, the X-15 used a

ballistic control system.  Eight hydrogen peroxide thrust

rockets, located on the nose of the aircraft controlled pitch and

yaw. Four other rockets were located on the wings for roll

control.

Because of the rapid fuel consumption, the X-15 was air-

launched from under the wing of a B-52 aircraft at 45,000 feet at

a speed of about 500 miles per hour.  Depending on the mission

and engine throttle setting, the rocket engine provided thrust

for the first 80 to 120 seconds of flight.  The remainder of the

normal 10-11 minute flight was powerless and ended with a 200

mile-per-hour glide landing.

Usually, one of two types of X-15 flight profiles were

flown:  a high-altitude flight plan that called for the pilot to

climb steeply after launch or a speed profile that called for the

pilot to push over and maintain a level altitude.  First flown

June 8, 1959, the three X-15 aircraft made 199 powered flights,

concluding with the last flight on Oct. 24, 1968.  Flight

maximums of 354,200 feet in altitude and a speed of 4,520 miles

per hour were obtained.

The airframe manufacturer was North American Rockwell,

Inc.  Thiokol Chemical Corp. manufactured the power plant.  The

program was a joint NASA-USAF-USN effort.

Three X-15 aircraft were built.  The number 1 aircraft now

is displayed in the Smithsonian's National Air and Space Museum,

Washington, D.C.  Aircraft number 2 is on display at the Air

Force Museum at Wright-Patterson Air Force Base, Ohio.  The third

aircraft was destroyed in a fatal accident in November 1967.

Total cost of the X-15 program, including development of the

three aircraft, was about $300 million.

- end -

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Salted Slug Systems                      Strange              408-454-9368

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Tomorrow's 0rder of Magnitude            Finger_Man           408-961-9315

My Dog Bit Jesus                         Suzanne D'Fault      510-658-8078

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X