HOMOPOLAR "FREE-ENERGY" GENERATOR TEST

Robert Kincheloe

Professor of Electrical Engineering (Emeritus)

Stanford University

Paper presented at the 1986 meeting

of the

Society for Scientific Exploration

San Francisco

June 21, 1986

Revised February 1, 1987

--------------------------------------------------------------------

HOMOPOLAR "FREE-ENERGY" GENERATOR TEST

Robert Kincheloe

ABSTRACT

Known for over  150  years, the Faraday homopolar generator has

been claimed to provide a basis for so-called  "free-energy"

generation, in that under certain conditions the extraction of

electrical output energy is not reflected as a corresponding

mechanical  load  to  the driving source.

During 1985 I  was invited to test such a machine.  While it did

not perform as claimed, repeatable data  showed  anomalous

results that did not seem to conform to traditional theory.

In particular, under certain assumptions about internally

generated output voltage, the increase in input power when power

was extracted from the generator  over that measured due to

frictional losses with the generator unexcited seemed to  be

either about 13% or 20% of the maximum computed generated power,

depending on interpretation.

The paper briefly  reviews  the homopolar generator,  describes  the

tests on this particular machine, summarizes and presents

tentative conclusions from the resulting data.

THE SUNBURST HOMOPOLAR GENERATOR

In July, 1985, I became aware of and was invited to examine and

test a so-called free-energy generator known as the Sunburst N

Machine.

This device, shown  in Figs 1a and 1b, was proposed by Bruce

DePalma and constructed by Charya Bernard of the Sunburst

Community in Santa Barbara, CA, about 1979.

The term "free-energy" refers to  the  claim  by  DePalma  [1]

(and others [2]) that it was capable of producing electrical

output power that was not reflected as a mechanical load to the

driving mechanism but derived from presumed latent spatial

energy.

Apart from mechanical frictional and electrical losses  inherent

in the particular construction,  the  technique employed was

claimed to provide a basis for constructing a generator which

could supply the energy to provide not only its own motive power

but also additional energy for external use.  From August  1985

to  April 1986 I made a series of measurements on this particular

machine to test these claims.

GENERATOR DESCRIPTION

Details of the generator construction are shown in Figs. 2 and 3.

It consists essentially of an electromagnet formed by a coil of

3605 turns of #10  copper  wire  around  a soft iron core  which

can  be rotated with the  magnetic  field parallel to and

symmetrical around the axis of rotation.

At each end of the magnet are conducting  bronze cylindrical

plates, on one of  which  are  arranged  (as shown in Fig.  3)

one  set  of graphite brushes for extracting output current

between the shaft and the outer circumference  and  a  second

set of metering brushes for independently measuring the induced

voltage between these locations.

A third pair of brushes and slip  rings  supply  the current for

the electromagnet.  A thick   sheath  of  epoxy-impregnated

fiberglass windings allow the magnet to be rotated at high speed.

The generator may be recognized as a so-called homopolar, or

acyclic machine, a device   first  investigated  and  described

by  Michael Faraday [3] in 1831 (Figs. 4,5) and shown

schematically in Fig. 6.

It consists of a cylindrical conducting  disk  immersed  in an

axial magnetic field, and  can  be  operated as a generator  with

sliding brushes extracting current  from  the  voltage  induced

between the inner and outer regions of the disk  when  the

rotational energy is supplied by an external driving source.

The magnitude of  the  incremental  radial  generated   voltage

is proportional to both  the  strength  of  the  magnetic field

and the tangential velocity, so that in a  uniform  magnetic

field the total voltage is proportional to the product of speed

times the difference between the squares of the inner and outer

brush radii.

The device may  also  be  used  as a motor when an external

voltage produces an radial current between the sliding brushes.

There have been  a  number  of  commercial applications of

homopolar motors and generators, particularly early in this

century [4], and their operating principles are described in a

number of texts [5].

The usual technique is to use a stationary  magnet  to  produce

the magnetic field in  which  the  conducting  disk  (or

cylinder)  is rotated.

Faraday found, however,  (Fig 7) that it does not matter whether

the magnet itself is stationary or rotating with the disk as long

as the conductor is moving in the field, but that rotating the

magnet with the conducting disk stationary did not produce an

induced voltage.

He concluded that a magnetic field  is  a  property of space

itself, not attached to the magnet which serves to induce the

field [6].

DePalma stated [7] that when the conducting disk  is  attached

to a rotating magnet, the interaction of the primary magnetic

field with that produced by the radial output current results in

torque between the disk and the magnet structure which is not

reflected back to the mechanical driving source.

Lenz's law therefore  does  not  apply, and the extraction of

output energy does not  require additional  driving  power.

This  is  the claimed basis for extracting "free" energy.

Discussions of the torque experienced by a rotating magnet are also

discussed in the literature [8].

Because the simple  form  shown  in  Fig.  6  has  essentially

one conducting path, such  a  homopolar  device  is characterized

by low voltage and high current requiring a large magnetic field

for useful operation.

Various homopolar devices have been used for specialized

applications [9] (such as generators for developing large

currents for welding, ship degaussing, liquid metal

magnetohydrodynamic pumps for nuclear reactor  cooling,

torquemotors  for propulsion,  etc.), some involving quite high

power.

These have been  extensively  discussed  in  the literature,

dealing with such problems as developing the  high  magnetic

fields required (sometimes using superconducting  magnets  in

air   to  avoid  iron saturation effects), the  development of

brushes that can handle the very high currents and have low

voltage  drop  because  of  the low output voltage generated,

and with counteracting armature  reaction which otherwise would

reduce  the  output  voltage  because  of the magnetic field

distortion resulting from the high currents.

From the standpoint  of  prior  art,  the  design  of  the

Sunburst generator is inefficient and not suitable for power

generation:

1. The magnetic field is concentrated near the axis where

the tangential velocity is low, reducing the generated

voltage.

2. Approximately 4 kilowatts of power are required to

energize the magnet, developing enough heat so that the

device can only be operated for limited periods of time.

3. The graphite brushes used have a voltage drop almost

equal to the total induced voltage, so that almost all of

the generated power is consumed in heating the brushes.

4. The large contacting area (over 30 square inches) of

the brushes needed for the high output current creates

considerable friction loss.

Since this machine was not intended as a practical  generator but

as a means for  testing  the  free energy principle, however,

from this point of view  efficiency  in  producing   external

power  was  not required or relevant.

DEPALMA'S RESULTS WITH THE SUNBURST HOMOPOLAR GENERATOR

In 1980 DePalma   conducted  tests  with  the  Sunburst

generator, describing his measurement technique and results in an

unpublished report [10].

The generator was  driven  by a 3 phase a-c 40 horsepower motor

by a belt coupling sufficiently long that  magnetic  fields  of

the motor and generator would not interact.  A table from this

report  giving his data and results is shown in Fig. 8.

For a rotational speed of 6000 rpm an output power of 7560 watts

was claimed to require an increase of 268 watts of drive power

over that required to supply losses due to friction, windage,

etc. as measured with the output switch open.

If valid, this  would  mean that the output power was 28.2 times

the incremental input power needed to  produce  it.  Several

assumptions were made in this analysis:

1. The drive motor input power was assumed to be the product

of the line voltage and current times the appropriate factor

for a three-phase machine and an assumed constant 70% power

factor.

There was apparently no consideration of phase angle

change as the motor load increased.  This gives optimistic

results, since consideration of phase angle is necessary

for calculating power in an a-c circuit, particularly with

induction motors.

It might  also  be  noted that the measured incremental line

current increase of 0.5 ampere  (3.3%)  as obtained with the

analog clamp-on  a-c ammeter that was used  was  of  limited

accuracy.

2. The output power of the generator was taken to be the

product of the measured output current and the internally

generated voltage in the disk less the voltage drop due only

to internal  disk  resistance.   Armature  reaction was thus

neglected or assumed not to be significant.

3. The generated voltage which produced the current in the main

output brushes was assumed  to  be the same as that measured

at the metering brushes, and the decrease in metered voltage

from 1.5 to 1.05 volts when the output switch  is closed was

assumed to  be  due  to  the internal voltage drop resulting

from the output current flowing  through  the  internal disk

resistance that  is  common  to  both sets  of  brushes  and

calculated to 62.5 microohms.

Of these, the first assumption seems the most serious, and it is my

opinion that the results of this particular test were inaccurate.

Tim Wilhelm of Stelle, Illinois, who witnessed tests of the Sunburst

generator in 1981, had a similar opinion [11].

RECENT TESTS OF THE SUNBURST GENERATOR

Being intrigued by DePalma's hypothesis, I accepted the offer by

Mr. Norman Paulsen, founder  of the Sunburst Community, to

conduct tests on the generator which apparently  had not been

used since the tests by DePalma and Bernard in 1979.

Experimental Setup

A schematic diagram of the test arrangement is shown in Fig. 9,

with the physical equipment  shown  in Fig. 10.  The generator

is  shown coupled by a  long  belt to the drive motor behind it,

together with the power supplies and metering both  contained

within and external to the Sunburst power and metering cabinet.

Figure 10b shows the panel of the test cabinet which  provided

power for the generator magnet and motor field.  The 4-1/2 digit

meters on the panel were  not  functional  and  were not used;

external meters were supplied.

I decided to  use  an  avaiable  shunt-field   d-c  drive  motor

to facilitate load tests  at different speeds and to simplify

accurate motor input power measurements.

Page 5

Referring to Figure  9,  variacs  and  full-wave  bridge

rectifiers provided variable d-c supplies for  the motor armature

and field and the homopolar generator magnet.

Voltages and currents were measured with Micronta model 11-191

3-1/2 digit meters calibrated  to  better  than  0.1%  against  a

Hewlett Packard 740B Voltage  Standard that by itself was

accurate to better than .005%.

Standard meter shunts together with the digital voltmeters were

used to measure the  various  currents.    With   this

arrangement  the generator speed could be varied smoothly from 0

to  over  7000  rpm, with accurate measurement  of  motor  input

power, metered generator output voltage Vg and generator output

current Ig.

Speed was measured with a General  Radio  model 1531 Strobotac

which had a calibration  accuracy of better than 2% (as  verified

with  a frequency counter) and which allowed determination of

relative speed changes of a few rpm of less.

Small changes in  either  load  or  input power were clearly

evident because of the  sensitivity  of  the  Strobotac  speed

measurement, allowing the motor  input  power  to be adjusted

with  the  armature voltage variac to   obtain   the  desired

constant  speed  with  no acceleration or deceleration before

taking readings from the various meters.

Generator Tests

Various tests were conducted with  the output switch open to

confirm that generated voltage at both the output brushes (Vbr)

and metering brushes (Vg) were proportional to speed and magnetic

field, with the polarity reversing when magnetic field or

direction of rotation were reversed.

Tracking of Vbr and Vg with variation of magnetic field  is shown

in Fig. 11, in  which it is seen that the output voltages are not

quite linearly related to magnet current, probably due to core

saturation.

The more rapid departure of Vg from  linearity  may  be  due  to

the different brush locations  as  seen  on  Fig 3, differences

in  the magnetic field at the different brush locations, or other

causes not evident.  An expanded  plot  of  this voltage

difference is shown in Fig. 12, and is seen to considerably

exceed meter error tolerances.

Figure 11 also shows an approximate 300 watt increase in drive

motor armature power as  the magnet field  was  increased  from

0  to  19 amperes.

(The scatter of input power measurements shown in the upper curve

of Fig. 11 resulted  from the great sensitivity of the  motor

armature current to small fluctuations in power line voltage,

since the large rotary inertia of  the  400  pound  generator did

not allow speed to rapidly follow line voltage changes).

At first it was thought that this  power  loss  might  be due to

the fact that the outer output brushes were arranged  in  a

rectangular array as shown in Fig. 3.

Since they were  connected  in parallel but not equidistant from

the axis the different generated voltages  would  presumably

result  in circulating currents and additional power dissipation.

Measurement of the  generated  voltage  as  a  function   of

radial distance from the  axis  as  shown  in Fig. 13, however,

showed that almost all of the voltage differential occurred

between 5 and 12 cm, presumably because this was the region  of

greatest  magnetic field due to the centralized iron core.

The voltage in the region of the outer brushes was  almost

constant, with a measured variation of only 3.7% between the

extremes, so that this did not seem to explain the increase in

input power.  The other likely explanation seems to be that there

are internal losses in the core and other  parts  of  the metal

structure due to eddy currents, since these are also moving

conductors in the field.

In any event, the increase in drive power was only about 10% for

the maximum magnet current of 19 amperes.

Figure 14 typifies  a  number of measurements  of  input  power

and generator performance as a function of speed and various

generator conditions.

Since the generator output knife switch procedure was very stiff

and difficult to operate the procedure used was to make a

complete speed run from zero to the maximum speed and descending

again to zero with the switch open,  taking readings at each

speed increment  with  the magnet power both off and on.

The procedure was  then  repeated  with  the switch closed.  (It

was noted that during the descending speed run the input power

was a few percent lower than for the same speed  during  the

earlier ascending speed run; this  was  presumably  due  to

reduced  friction  as  the brushes and/or bearings  became

heated.   In  plotting the data the losses for both runs were

averaged  which gave a conservative result since the losses

shown  in  the figures exceed the  minimum  values measured).

The upper curve  (a)  shows  the  motor  armature input power

with a constant motor field current of 6  amperes  as  the  speed

is varied with no generator magnet excitation and is seen to

reach  a maximum of 4782 watts as the speed is increased to 6500

rpm.

This presumably represents  the  power required to overcome

friction and windage losses in the motor, generator,  and drive

belt, and are assumed to remain  essentially  constant  whether

the  generator  is producing power or not [12].

Curve 14b shows  the  increase  of motor armature power over that

of curve (a) that results from energizing  the  generator magnet

with a current of 16 amperes but with the generator output

switch  open so that there is   no   output  current  (and  hence

no  output  power dissippation).

This component of power (which is  related  to the increase of

drive motor power with increased magnet current as shown  in Fig.

11  as discussed above) might  also be present whether or not the

generator is producing output current and power, although this is

not so evident since the  output  current  may  affect  the

magnetic field distribution.

Curve 14c shows the further increase  of  motor armature input

power over that of curves (a) plus (b) that results when the

output switch is closed, the generator magnet is energized and

output  current  is produced.

It is certainly not zero or negligible but rises to a maximum of

802 watts at 6500 rpm.  The total motor armature input power

under these conditions is thus  the  sum  of  (a),  (b),  and

(c) and reaches a maximum of 6028 watts at 6500 rpm.

The big question has to do with the  generated  output  power.

The measured output current at 6500 rpm was 4776 amperes; the

voltage at the metering brushes was 1.07 volts.

Using a correction factor derived from Fig. 12 and assuming a

common internal voltage drop  due  to  a calculated disk

resistance  of  38 microohms, a computed  internal generated

potential of 1.28 volts is obtained which if  multiplied  by

the   measured   output  current indicates a generated power of

6113 watts.

All of this  power  is  presumably  dissipated in the  internal

and external circuit resistances,  the  brush loss due both to

the brush resistance and the voltage drops at the contact

surfaces between the brushes and the disk (essentially an arc

discharge), and the power dissipated in the 31.25 microohm meter

shunt.

It still represents power generated  by  the  machine,  however,

and exceeds the 802 watts of increased motor drive power  due

solely to closing the generator  output  switch  and causing

output current to flow by a factor of 7.6 to 1.

If the 444  watts  of  increased  input  power  that  resulted

from energizing the magnet with the output switch open is assumed

to have been converted to  generated  output  power  and  hence

should  be included as part  of  the total increased drive motor

power required to produce generated output, the  computed  6113

watts of generated power still exceeds  the  total input power of

444  watts  plus  802 watts by a factor of 4.9 to 1.

The computed output  power  even  slightly  exceeds  the total

motor armature input power including all  frictional and windage

losses of 6028 watts under  these  conditions  (although  the

total   system effeciency is still less than 100% because of the

generator magnet power of approximately 2300 watts and motor

field power of about 144 watts which must  be  added  to  the

motor armature power to obtain total system input power).

It would thus  seem that if the above  assumptions  are  valid

that DePalma correctly predicted  that much of the generated

power  with this kind of  machine  is  not  reflected back to the

motive source. Figure 15 summarizes the data discussed above.

To further examine  the question  of  the  equivalence  between

the internally generated voltage  at  the main output brushes and

that measured at the  metering  brushes,  a  test was made of the

metered voltage as a function of speed with the generator magnet

energized with a current  of  20  amperes both with the output

switch open and closed.  The resulting data is shown in Fig. 16.

The voltage rises to about 1.32 volts  at  6000  rpm with the

switch open (which is  close to that obtained by DePalma)  and

drops  0.14 volts when the  switch  is closed and the measured

output current is 3755 amperes, corresponding to an  effective

internal resistance of 37 microohms.

Even if this were due to other causes, such as armature reaction,

it does not seem  likely  that  there would be a large  potential

drop between the output   and  metering  brushes  because  of

the  small distance, low magnetic field (and  radial differential

voltage), and large mass of conducting disk material.

Internal currents many times the measured output current  of

almost 4000 amperes would  be  required  for the voltage

difference between the outer metering  and  output   brushes   to

be  significant  and invalidate the conclusions reached above.

A further method  of  testing the validity of the assumed

generated output potential involved  an examination of the

voltage drop across the graphite brushes themselves.

Many texts on  electrical  machinery   discuss  the  brush  drop

in machines with commutators or slip rings.

All of those examined agree that graphite brushes typically have

a voltage drop that is essentially constant at approximately one

volt per brush contact when the current density rises above 10-15

amperes per square centimeter.

To compare this  with the Sunburst machine the total  brush

voltage was calculated by  subtracting the IR drop due to the

output current in the known (meter shunt) and calculated  (disk,

shaft,  and brush lead) resistances from  the  assumed

internally  generated   output voltage.  The result  in  Fig. 17

shows that the brush drop obtained in this way is even less than

that  usually  assumed, as typified by the superimposed curve

taken from one text.

It thus seems   probable   that   the  generated  voltage   is

not significantly less than that obtained from the metering

brushes, and hence the appropriateness of the computed output

power is supported.

CONCLUSIONS

We are therefore  faced  with  the  apparent  result that the

output power obtained when  the  generator   magnet  is

energized  greatly exceeds the increase  in  drive  power  over

that needed  to  supply losses with the  magnet  not energized.

This is certainly anomalous in terms of convential theory.

Possible explanations?

1. There could be a large error in the measurements resulting

from some factor such as noise which caused the digital

meters to read incorrectly or grossly inaccurate current

shunt resistances.

If the measured results had shown that the computed generated

output power exceeded the input drive power by only a few percent

this explanation would be reasonable and would suggest that more

careful calibration and measurements might show that the results

described above were due to measurement error.

With the data showing such a large ratio of generated power to

input power increase, however,  in  my  opinion  this

explanation  of the results seems unlikely.

(A later test showed that the digital  meters  are  insensitive

to a large a-c ripple superimposed on the measured d-c,  but

within their rated accuracy of 0.1% give a true average value).

2. There could be a large difference between the measured

voltage at the metering brushes and the actual generated

voltage in   the   output  brush  circuit  due  to  armature

reaction, differences in  the  external  metering and output

circuit geometry, or other unexplained causes.

As discussed above the various data do not seem to support this

possibility.

3. DePalma may have been right in that there is indeed a

situation here whereby energy is being obtained from a

previously unknown and unexplained source.

This is a conclusion that most scientists and engineers would

reject out of hand as being a violation of accepted laws of

physics, and if true has incredible implications.

4. Perhaps other possibilities will occur to the reader.

The data obtained so far seems to have shown that while DePalma's

numbers were high, his basic premise has not been disproved.

While the Sunburst generator does not produce useful output power

because of the internal  losses  inherent  in  the  design,   a

number  of techniques could be used to reduce the friction

losses, increase the total generated voltage   and   the

fraction  of  generated  power delivered to an external load.

DePalma's claim of  free energy generation  could  perhaps  then

be examined.

I should mention, however, that the obvious application of using

the output of a "free-energy" generator to provide its own motive

power, and thus truly  produce a source of free energy, has

occured  to  a number of people and several such machines have

been built.

At least one  of  these  known to me [13], using what seemed to

be a good design techniques, was unsuccessful.

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

FOOTNOTES

1. DePalma, 1979a,b,c, 1981, 1983, 1984, etc.

2. For example, Satelite News, 1981, Marinov, 1984, etc.

3. Martin, 1932, vol. 1, p.381.

4. Das Gupta, 1961, 1962; Lamme, 1912, etc.

5. See, for example, Bumby, 1983; Bewley, 1952; Kosow, 1964; Nasar,

1970.

6. There has been much discussion on this point in the

literature, and about interpretation of flux lines.  Bewley,

1949; Cohn, 1949a,b; Crooks, 1978; Cullwick, 1957; Savage,

1949.

7. DePalma, op. cit.

. Kimball, 1926; Zeleny, 1924.

9. Bumby, Das Gupta, op. cit.

10. DePalma, 1980.

11. Wilhelm, 1980, and personal communication.

12. The increase in  motor losses with increased load are

neglected in this discussion because of  a  lack  of accurate

values for armature and brush  resistances, magnetic  field

distortion resulting from  armature reaction,  etc.   Such

losses, while small, would  be appreciable,  however;  their

inclusion  would further increase the ratio of generated to

drive  power  so that the results described are conservative.

13. Wilhelm, 1981, and personal communication.

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

REFERENCES

[Bewley, 1949] - L. V. Bewley, letter re [Cohn, 1949a]; ELECTRICAL

ENGINEERING, Dec. 1949, p.1113-4.  (Claims error in Cohn's paper)

[Bewley, 1952] - L. V. Bewley, FLUX LINKAGES & ELECTROMAGNETIC

INDUCTION, Macmillan,   NY,   1952.    (Explanation  of  induction

phenomena and the Faraday generator)

[Bumby, 1983] - J. R. Bumby, SUPERCONDUCTING ROTATING ELECTRICAL

MACHINES, Claredon Press, 1983.  (Homopolar designs, high current

brushes including liquid metal)

[Cohn, 1949a] - George I. Cohn, "Electromagnetic Induction",

ELECTRICAL ENGINEERING, May 1949, p441-7.  (Unipolar generator as

paradox)

[Cohn, 1949b] - George Cohn, letter re [Savage, 1949]; ELECTRICAL

ENGINEERING, Nov 1949, p1018.  (Responds to criticism by Savage)

[Crooks, 1978] - M. J. Crooks et al, "One-piece Faraday generator:

A paradoxical experiment from  1851",  Am.  J.  Phys.  46(7), July

1978, p729-31.   (Derives  Faraday  generator  performance   using

Maxwell's equations)

[Cullwick, 1957] - E. G. Cullwick, ELECTROMAGNETISM AND RELATIVITY,

Longmans &   Green,   London,  1957.   (Chapter  10,  "A  Rotating

Conducting Magnet", pp.141-60, discusses question of flux rotation

with magnet)

[Das Gupta, 1961]  - A. K. Das Gupta,  "Design  of  self-compensated

high current  comparatively higher voltage homopolar  generators",

AIEE Trans.   Oct  1961,  p567-73.   (Discusses  very high current

homopolar generator design)

[Das Gupta, 1962] - A. K. Das Gupta, "Commutatorless D-C generators

capable to supply currents more than one million amperes, etc"

AIEE Trans.  Oct 1962, p399-402.  (Discusses very high current low

voltage Faraday generators)

[DePalma, 1979a] - Bruce DePalma, EXTRACTION OF ELECTRICAL ENERGY

DIRECTLY FROM SPACE:  THE N-NACHINE, Simularity Institute, Santa

Barbara CA,  6  Mar  1979.   (Discusses  homopolar generator or N-

Machine as free-energy source)

[DePalma, 1979b] - Bruce DePalma,  "The  N-Machine",  Paper given at

the World Symposium on Humanity, Pasadena, CA, 12 April 1979.

(Describes background, development of "free-energy" theories)

[DePalma, 1979c] -   Bruce   DePalma,  ROTATION  OF   A   MAGNETIZED

GYROSCOPE, Simularity   Institute   Report   #33,  16  July  1979.

(Describes design of Sunburst homopolar generator)

[DePalma, 1980] - Bruce DePalma, "Performance of the Sunburst N

Machine", Simularity Institute,  Santa  Barbara,  CA,  17 December

1980.  (Description of tests and results)

[DePalma, 1981] - Bruce DePalma, "Studies on rotation leading to the

N-Machine", DePalma   Institute,   1981  (transcript   of   talk?)

(Discusses experiments  with  gravity  that  led to development of

idea of free-energy machine)

[DePalma, 1983] -  Bruce DePalma,  THE  ROTATION  OF  THE  UNIVERSE,

DePalma Institute  Report #83, Santa Barbara, CA,  25  July  1983.

(Uses Faraday disc to discuss universal principles).

[DePalma, 1984] - Bruce DePalma, THE SECRET OF THE FARADAY DISC,

DePalma Institute, Santa Barbara, CA, 2 Feb 1984.  (Claims

explanation of Faraday disc as a free-energy device)

[Kimball, 1926] -   A.   L.   Kimball,  Jr.,  "Torque  on  revolving

cylindrical magnet",  PHYS.  REV.   v.28,   Dec   1928,  p.1302-8.

(Alternative analysis of torque in a homopolar device  to  that of

Zeleny and Page, 1924)

[Kosow, 1964] - Irving L. Kosow, ELECTRICAL MACHINERY & CONTROL,

Prentice-Hall, 1964.  (Discusses high current homopolar (acyclic)

generators)

[Lamme, 1912] - B. G. Lamme, "Development of a successful direct-

current 2000-kW unipolar generator", AIEE Trans. 28 June 1912,

p1811-40.  (Early discussion of design of high power homopolar

generator)

[Marinov, 1984]- Stefan Marinov, THE THORNY WAY OF TRUTH, Part II;

Graz, Austria,  1984  (Advertisement  in  NATURE).   (Claims free-

energy generator proved by DePalma, Newman)

[Martin, 1932] - Thomas Martin (ed), FARADAY'S DIARY, Bell, 1932,

in 5 vols.  (Transcription and publication of Faraday's original

diaries)

[Nasar, 1970] - S. Nasar, ELECTROMAGNETIC ENERGY CONVERSION DEVICES

& SYSTEMS, Prentice-Hall, 1970.  (Discusses principles and

applications of acyclic (homopolar) machines)

[Satellite News, 1981] - "Researchers see long-life satellite power

systems in  19th  century  experiment",  Research  news, SATELLITE

NEWS, 15  June 1981.  (Reports  DePalma's  claim  for  free-energy

generator)

[Savage, 1949] - Norton Savage, letter re [Cohn, 1949a]; ELECTRICAL

ENGINEERING, July 1949, p645.  (Claims error in Cohn's paper)

[Wilhelm, 1980] - Timothy J. Wilhelm, INVESTIGATIONS OF THE N-EFFECT

ONE-PIECE HOMOPOLAR DYNAMOS, ETC. (Phase I), Stelle, IL, 12 Sept

1980.  (Discusses tests on DePalma's N-Machine)

[Wilhelm, 1981] - Timothy J. Wilhelm, INVESTIGATIONS OF THE N-EFFECT

ONE-PIECE HOMOPOLAR DYNAMOS, ETC. (Phase II), Stelle, IL, 10 June

1981.  (Design and tests of improved homopolar generator/motor)

[Zeleny, 1924] - John Zeleny & Leigh Page, "Torque on a cylindrical

magnet through which a current is passing", PHYS.  REV.  v.24,  14

July 1924,  p.544-59.   (Theory  and  experiment  on  torque  in a

homopolar device)

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

(Sysop note:  The following figure also had an accompanying drawing)

Figure 5 - Transcription of the first  experiment showing generation

of electrical  power  in  a moving conductor  by  Michael

Faraday

99*. Made many expts. with a copper revolving plate, about 12 inches

in diameter  and  about  1/5  of inch thick, mounted on a brass

axle.

To concentrate the polar action two small magnets 6 or 7 inches

long, about 1 inch wide and half an inch thick were put against

the front of the large poles, transverse to them and with their

flat sides against them, and  the  ends  pushed  forward  until

sufficiently near; the bars were prevented from  slipping  down

by jars and shakes by means of string tied round them.

100. The edge of the plate was inserted more of less between the two

concentrated poles  thus formed.  It was also well amalgamated,

and then contact was made with this edge in different places by

conductors formed from equally  thick copper plate and with the

extreme end edges grooved and amalgamated so  as  to  fit on to

and have  contact  with  the  edges of the plate.  Two of these

were attached to a piece of card board by thread at such

*[99]

(Sysop note:  a sketch appeared in this area)

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

(Sysop note:  The following figure also had an accompanying drawing)

Figure 7 - Test of a rotating magnet  by  Michael  Faraday, December

26, 1831.

255.  A copper disc was cemented on the top of a cylinder magnet,

paper intervening,  the  top being the marked pole; the magnet

supported so as to rotate by means of string, and the wires of

the galvanometer connected with  the  edge and the axis of the

copper plate.   When  the  magnet  and  disc together  rotated

unscrew the  marked  end  of  the  needle went west.  When the

magnet and disc rotated screw  the  marked  end  of the needle

went east.

256.  This direction is the same as that which would have resulted

if the  copper  had  moved and the magnet been  still.   Hence

moving the  magnet  causes  no  difference provided the copper

moves.  A  rotating and a stationary  magnet  cause  the  same

effect.

257.  The disc was then loosed from the magnet and held still

whilst the magnet itself was revolved; but now  no effect upon

the galvanometer.  Hence it appears that, of the metal circuit

in which  the  current  is  to be formed, different parts must

move with different angular  velocities.  If with the same, no

current is produced, i.e. when both parts are  external to the

magnet.

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

(Sysop note:  The following figure also had an accompanying drawing)

Figure 8 - Test data from report by Bruce DePalma

PERFORMANCE OF THE SUNBURST HOMOPOLAR GENERATOR

machine speed:                        6000 r.p.m.

drive motor current no load           15 amperes

drive motor current increase

when N machine is loaded                    1/2 ampere max.

Voltage output of N generator no load       1.5 volts d.c.

Voltage output of N generator loaded        1.05 v.d.c.

Current output of N generator               7200 amperes

(225 m.v. across shunt @ 50 m.v./1600 amp.)

Power output of N machine                   7560 watts = 10.03 H.p.

Incremental power ratio =  7560/268         28.2  watts out/watts in

Internal resistance of generator            62.5 micro-phms

Reduction of the above data gives as the equivalent circuit for the

machine:

(Sysop note: a drawing            R(internal) =  62.5 micro-ohms

appeared in this area)            R(brush)    = 114.25  "    "

R(shunt)    =  31.25  "    "

BRUCE DEPALMA

17 DECEMBER 1980

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

Page 14

Figure 15 - Summary of test results at 6500 rpm

I              II            III

MAGNET POWER              OFF              ON             ON

OUTPUT SWITCH             OPEN           OPEN         CLOSED

SPEED                     6500           6500           6500    RPM

MAGNET CURRENT               0             16             16

AMPERES

MOTOR ARMATURE POWER      4782           5226           6028

WATTS

INCREMENT                       444            802

WATTS

METER BRUSH VOLTAGE       .005          1.231          1.070

VOLTS

OUTPUT CURRENT               0              0           4776

AMPERES

GENERATED VOLTAGE                       1.280         (1.280)

VOLTS

GENERATED POWER              0              0          (6113)

WATTS

HOMOPOLAR GENERATOR TEST - BIG SPRINGS RANCH     APRIL 26, 1986

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X