Sega Programming FAQ February 5, 1994, Third Edition

=============================================================

This FAQ (Frequently Asked Questions) is about how to program

the Sega Genesis and Mega Drive Video Games Systems.

This file IS Public Domain, and can be used by all, HOWEVER

if you want to use this in a Data Base, or put it up on Gopher,

etc el then you MUST get my permission. You may copy and post

this file elsewhere without my permission.

If you want to add anything, especially game code or

information on the hardware please E-mail me. My E-mail

and real address are below and at the end of this FAQ.

Disclaimer : In no way I am indorsing any products, systems,

etc. What I am doing is providing the information that I

have found throughout internet and other network systems and

reporting them here, with out reverse engineering. All

information is reported as is- I will not accept any

responsibility if you fry your system trying something out.

All copyrights are owned by their respective owners, no

infringements intended.

Henry Rieke

<U6c16@Wvnvm.Wvnet.Edu>

US Mail:

Henry Rieke

Attn: Sega FAQ

161 Poplar Dr

Morgantown, Wv 26505-2340

=============================================================

0.1 Changes, Updates and Coming Soon

1) I've changed the part on the hardware description to be

smaller and easier to understand. If you still want the

whole file on the Sega Hardware internals then I suggest

that you get the file called Hardware.Txt by FTP from

Ftp.spd.louisville.edu under the directory pub\sega

2) This FAQ has a home now (yea!) at Ftp.spd.louisville.edu

(pub\sega) and it can also be found at Busop.cit.wayne.edu

(pub\megadev\incoming) and soon coming to WWW graphic database

in England (more on that later!).

3) I am going to be writing letters soon to all the companies

on the list (See Chapter 3.0, Companies) asking for more

information concerning their products, and details. I will also

be enquiring to Sega about CD programming and Saturn information.

=============================================================

1.0 Questions

Q) What are the differences between Mega Drive and Genesis?

What are the differences between Mega CD and Sega CD?

A) Physically, on the outside, the only difference is the

plastic case, the cartridge shape is slightly different on

the American version, smaller in size. On the inside there

are four jumpers labeled J1 though J4. J1 and J2 is the

language switch, between English and Japanese, J3 and J4

select the output between PAL and NTSC. You can build a

switch that can allow you to select between these options.

The diagram is included in this document (Chapter 5.0,

called "Language Switch"). The reason for the language

switch is to keep people from getting the game before it

is officially released elsewhere in the world. The newer

Sega Machines do not have the jumpers, and hence you can

not build a switch (as of yet).

The difference between Mega CD and Sega CD are internal.

In side the machine there is a ROM BIOS chip which is changed

for different markets. Sega has a version of the BIOS which

will play any system's games, but this is for Game developing.

There is a device called Pro-CDX which will allow you to by-pass

the lockout.

Q) Where to find files on the Sega Genesis/Mega Drive

hardware?

A) You can find the files on the Sega hardware from one of

the following FTP sites. A brief section describing memory

and cartridge ROMs are included in this FAQ. The author

Rick McTeague, is from Louisville and the most current copy

can be found there first.

FTP Ftp.spd.louisville.edu

Pub\sega\hardware.txt

FTP Busop.cit.wayne.edu

Pub\megadev\incoming\hardware.txt

There are also many other files here as well as a few

68000 ASM.

Q) What does it take to program the Genesis?

A) Development kits can be bought from many companies, a list

is included in the section called "Companies". These kits

require an IBM or Amiga computer to compile the code,

which is usually assembly, but can be in C as long as it

can be compiled into binary. SEGA's own kit is extremely

expensive and they are rumored to have slow response time

to answer questions. The price of the kits cost from $1000

to $4000.

Q) Is IBM or Amiga the only two platforms for game

development?

A) As far as I know, yes. The Apple MacIntosh might have

something, but I have not heard anything on this. There is

a system for the Apple IIgs, called Sluggo from Parsons

Engineering. As for the Mac, if you know, please pass it

on. It is rumored that Electronic Arts uses a priority

system on the MacIntosh for porting the games.

Q) How do I program the SEGA CD Rom?

A) I have no idea on programming this device, if you do have

some information please pass it along to me so I can

include it in the next update.

Q) What is Saturn, and how do I program that?

A) Saturn is Sega's next generation game machine, which

is based on Sega's S-1 arcade Machine's motherboard.

Rumor has it that it will allow programming in C as

well as Assembly.

There is good news and bad news as well.

Good news, Microsoft will be writing the OS.

Bad new, Microsoft will be writing the OS.

Q) What else is there on Internet?

A) Well there is the Sega Programming forum, which you can

join by subscribing to it with this command:

Send a message to Listserv@Busop.Cit.Wayne.Edu with the

first line reading SUBSCRIBE MEGADEV

To unsubscribe, do the same above, except the first line

should read UNSUBSCRIBE MEGADEV

There is also Rec.Games.Video.Programmer and

Rec.Games.Video.Sega where you can ask your questions.

There are also files on how to use the old SEGA 3D glasses

with your IBM compatible computer which can be found all

over the place (including the above FTP sites).

=============================================================

2.0 Hardware : This is the inner workings of the Sega system

ROM and Memory. For a better understanding Sega Hardware I

suggest reading Hardware.txt. An up-to-date version of this

file can be gotten by anonymous ftp from Ftp.spd.louisville.edu,

in the file "Pub/sega/Hardware.txt".

Genesis - this information comes from Rick McTeague.

Please send corrections or comments to:

Rick McTeague

Sysrick@starbase.spd.louisville.edu

Disclaimers:

The following information was extracted without the use of

official information from Sega Enterprises, Ltd. I therefore

offer no warranty on the correctness of this information.

Use it at your own risk.

This information was gathered as a personal project, and is

not the responsibility of my employer.

Different versions of the Genesis exist, so I won't suggest

that any of this information is right for anybody's Genesis

but my own.

Sega, Genesis, Sonic the Hedgehog, and Ecco the Dolphin are

trademarks of Sega Enterprises, Ltd. Other trademarks are the

property of their respective companies.

The Cartridge ROM

-----------------

As others on the net have observed, a standard Genesis

cartridge is nothing more than a small printed circuit board

with a ROM on it. There are some cartridges which have

battery-backed RAM for saving high scores, configurations,

etc., but I haven't seen these, so I don't know about them.

If you know of a particular cartridge that has RAM, let me

know so I can go rent one and take it apart (shhhh!) to see

what's in it.

A cartridge contains a word-wide (16 bit) ROM with up to 2

Meg words (4 Mbytes). This is based on the signal !C_CE, which

is used as the cartridge ROM's chip enable on the cartridges

I've looked at, and which is active at addresses $000000-$3fffff.

It may be possible to have more than 4 Mbyte cartridges; if

so, either they ignore the !C_CE line and do their own

address decoding, or there's a register somewhere in the

Genesis which changes the default address decoding scheme for

!_CE.

Note that cartridges are advertised as being "8 Meg"; as far

as I know, is 8 Meg _bits_, not bytes. 1 Mbyte is still

pretty impressive for a game machine.

SONIC uses a 40-pin 256Kx16 (512Kx8) ROM in word mode.

Ecco the Dolphin uses a 42-pin 512Kx16 (1024Kx8) ROM in word

mode. This one confused me because I'd never seen a 42-pin

DIP before, so I thought it was 40 pins. No wonder the

signals didn't look right...

Putting together a little bit of information from a Sharp

data sheet for a 512 Kword ROM with a bit of conjecture based

on the Sonic and Ecco cartridges, this is what I can come up

with for pinouts:

Sonic                              Ecco

(256 Kword)                       (512 Kword)

----u----

----u----               A18    1=|       |=42   VCC?

A17    1=|       |=40   A 8      A17    2=|       |=41   A 8

A 7    2=|       |=39   A 9      A 7    3=|       |=40   A 9

A 6    3=|       |=38   A10      A 6    4=|       |=39   A10

A 5    4=|       |=37   A11      A 5    5=|       |=38   A11

A 4    5=|       |=36   A12      A 4    6=|       |=37   A12

A 3    6=|       |=35   A13      A 3    7=|       |=36   A13

A 2    7=|       |=34   A14      A 2    8=|       |=35   A14

A 1    8=|       |=33   A15      A 1    9=|       |=34   A15

A 0    9=|       |=32   A16      A 0   10=|       |=33   A16

!CE   10=|       |=31   !BYTE    !CE   11=|       |=32   !BYTE

GND   11=|       |=30   GND      GND   12=|       |=31   GND

!OE   12=|       |=29   D15      !OE   13=|       |=30   D15

D 0   13=|       |=28   D 7      D 0   14=|       |=29   D 7

D 8   14=|       |=27   D14      D 8   15=|       |=28   D14

D 1   15=|       |=26   D 6      D 1   16=|       |=27   D 6

D 9   16=|       |=25   D13      D 9   17=|       |=26   D13

D 2   17=|       |=24   D 5      D 2   18=|       |=25   D 5

D10   18=|       |=23   D12      D10   19=|       |=24   D12

D 3   19=|       |=22   D 4      D 3   20=|       |=23   D 4

D11   20=|       |=21   VCC      D11   21=|       |=22   VCC

---------                        ---------

Note that the address line numbering is different than that

of the 68000; line A0 of the ROM is really connected to the

68000's A1. This is because of the way the 68000 handles

even/odd byte addressing using !UDS and !LDS.

The !BYTE line is held high. This puts the ROM chip into

"word" mode, where data is accessed 16 bits at a time.

The convenient correspondence between ROM and 68000 data and

address lines (ROM D0 = 68K D0, ROM D1 = 68K D1, etc..., and

ROM A0 = 68K A1,  ROM A1 = 68K A2, etc...) may not exist in

all cartridge designs. This is sometimes difficult to

visualize, but keep in mind that the ROM doesn't care whether

its D0 is really D0, or if its A3 is really A3, etc. As long

as each data line is used as the same D## line for both

programming and reading, and each address line is used as the

same A## for both programming and reading, the ROM will

function properly and the 68000 will see the right

instructions. Think about it...

Why bring this up? If you remove the ROM from the cartridge

PCB and try to read it with a EPROM programmer, there may be

situations where you won't see valid 68000 instructions, or

things won't be in the right place... Assuming your

programmer isn't messed up, this would be because the address

and data lines on the ROM aren't assigned the way you'd

expect.

Why would someone make a cartridge like this? To make your

life difficult, if you're wanting to hack their ROMs, or to

make their life easy, by making the cartridge PCB easier to

design.

The easiest way to avoid this potential problem is to build

an adapter from your programmer's EPROM socket to an edge

card connector into which you plug the cartridge. This is a

bit safer, too, so you don't have to remove the ROM from the

cartridge (or even open it up).

The Memory Map

--------------

$000000 $3fffff Cartridge ROM, when enabled by cartridge

control register and !CART_IN.

$000000 $0007ff Internal OS ROM, when enabled by cartridge

control register and !CART_IN.

$ff0000 $ffffff 64Kbytes scratchpad RAM

Partially decoded; RAM actually appears repeated between

$e00000 through $ffffff.

$a14101 cartridge control register

bit 0 = 0: cartridge disabled, OS ROM enabled

bit 0 = 1: cartridge enabled, OS ROM disabled

$a00000 $a????? Unknown. Audio? Controller I/O?

$c00000 $c????? Video display controller registers

The OS puts the top of its stack at $ffff00, and defines a

"user stack pointer" at $000000 (building down through the

top of RAM memory).

The Z80 processor is used for sound and has 8Kbytes of RAM.

I don't know anything about how the Z80 does its work, as I

haven't looked.

Acknowledgements

----------------

Thanks to:

dt93tn@pt.hk-r.se (Tore Nestenius) for info on older Genesis

systems.

lewism@rpi.edu (Michael Lewis) for info on the Sony CXA1145

and Samsung KM6264 chips.

crs@crs-sys.uucp (Chris Gregors) for posting a version of the

Genesis cartridge port pinouts.

merlyn@digibd.com (Brian Westley) for Game Genie information

and decoder program.

------------------------------------------------------------

2.2 Joystick - 3 button version - Could some please post a 6

button version?

By: nhowland@matt.ksu.ksu.edu (Neal Howland) for information

on the Joystick port.

First some background info:  The chip inside the controller

is a 74HC157.  This is a high-speed cmos quad 2-line to

1-line multiplexer. Basically how this works is there are two

inputs ( A and B ) for every output ( Y ).  There are four

groups like this.  There is one select signal for the whole

chip.  When the select signal is low, the output ( Y ) is the

same as input A.  When the select signal is high, the output

Y is the same as input B.  The pinout for the chip is as

follows:

Pin 1 - SelectPin 16 - Vcc (+5V)

Pin 2 - 1APin 15 - G (? must be low)

Pin 3 - 1BPin 14 - 4A

Pin 4 - 1YPin 13 - 4B

Pin 5 - 2APin 12 - 4Y

Pin 6 - 2BPin 11 - 3A

Pin 7 - 2YPin 10 - 3B

Pin 8 - GndPin 9 - 3Y

All the controls are done with switches.  Up is a switch,

Down is a switch, etc. Now, I will be referring to the output

of these switches later on. What I mean is that the output is

usually high, that is when the switch isn't pressed.

When the button is pushed, the output goes low.  This is

accomplished by connecting the output to +5V through a 10k

resistor. The button is then attached between the output and

ground. It looks like this:

+5V -----/\/\/------+--------- Output

10k       |

|

/        |

Ground -----/  -------+

button

(normally open)

For all of those who could actually decipher the above

schematic, congratulations!

I will now run down what lines from the plug are connected to

what. The line numbers are determined as follows, looking

straight at the plug on the front of the Genesis the numbers

are:

1 2 3 4 5

6 7 8 9

(For those of you who buy a joystick cable from radio shack

the pin #'s to wire colors are as follows: 1-white 2-blue

3-green 4-brown 5-yellow 6-orange 7-red 8-black 9-gray )

anyway, line connections:

Line 1 - Up output      These are the only two direct

Line 2 - Down output             connections

Line 3 - Pin 4 of the chip output 1Y

Line 4 - Pin 7 of the chip output 2Y

Line 5 - This line carries in +5V. It is connected to the

+5V bus line.

Line 6 - Pin 9 of the chip output 3Y

line 7 - Pin 1 of the chip this carries in a select signal

from the Genesis. This is a signal which varies

rapidly and controls which input goes through the

output.

Line 8 - Ground  This is connected to the Ground bus line.

Line 9 - Pin 12 of the chip output 4Y

Now for the chips pin connections:

Pin 1 - Line 7    (select)

Pin 2 - Ground    (1A)  Don't ask me why they do this. Maybe

Pin 3 - Left      (1B)  future expansion

Pin 4 - Line 3    (1Y)

Pin 5 - Ground    (2A)  Again, possibly future expansion

Pin 6 - Right     (2B)

Pin 7 - Line 4    (2Y)

Pin 8 - Ground    (GND)

Pin 9 - Line 6    (3Y)

Pin 10 - Button B (3B)

Pin 11 - Button A (3A)

Pin 12 - Line 9   (4Y)

Pin 13 - Button C (4B)

Pin 14 - Start    (4A)

Pin 15 - Ground   (G)   This must be connected to ground

Pin 16 - +5V      (Vcc) Power source for the chip

Anyway that's all the info needed to build your own joystick.

Now as an added bonus, additional information!

A simple source for a joystick cable is the Radio Shack

joystick extension cable. It is around $5 and is 10 ft. long.

Just snip off the connector that won't plug into the Genesis,

strip the wires back, and use the color pinout list I gave

above.

I went to my local arcade game repair company today and

purchased the supplies I needed.  They were much cheaper than

I expected.  Things you would need to buy from them would be:

an 8-way joystick     this ran me $15

3 buttons             $2.50 apiece

I only bought three buttons because I am going to use a Radio

Shack push button switch for my start button. You can purchase

4 arcade game buttons if you wish.

Interesting ideas for extras on the joystick:

1.  Autofire can be achieved with a simple 555 timer circuit.

For anyone wanting a challenge it should be possible to

build a digital autofire using a divide-by-n counter with the

select signal as clock input.

2.  Slow down mode is just autofire for the start button.

=============================================================

3.0 Companies - these companies produce the hardware that is

required to write games for the Genesis. The original

document came from: Scott Golby <sgolby@st.nepean.uws.edu.au>

Hi Everyone,

Here is the current list of info I have about the Sega.

I have had about 10 replies so far asking to pass my

info on.  If people want I will pass their EMail address on

to everyone else so we can work as a group.  Drop me a line

if you want to be added to the 'group' list.

*****    <----- Five astrict like this means info from

different people

Sega Genesis Developer Resources

Dan Chang April 18, 1993

Sega of America Inc

Consumer Products Division

125 Shoreway Road

San Carlos, California, 94070

Tel : (415) 802-4400

Fax : (415) 802-4458

Products : Detailed SEGA Genesis technical information.  GEMS

SEGA music development package.

Solution : Music and Sound effects package for the Sega. SOA

can provide detailed information on programming the Genesis;

however they will want to know more about your company first.

Western Technologies, Inc.

12057 Jefferson Blvd

Culver City, California, 90230

Tel : (310) 821-7880

Fax : (310) 306-1739

Products : SegaDev Card. PC-Hosted Genesis development

hardware, which includes SEGADEV.EXE, a Windows-hosted

source-level debugger, and SEGALOAD.EXE, a PC_hosted 68000

object code downloader.

Solutions : SEGA Genesis Hardware, WT recommends Sierra

Systems 68000 Assembler.

Sierra Systems

6728 Evergreen Ave

Oakland California 94611

Tel : (510) 339-8200  or (800) 776 4888

Fax : (510) 339 3844

Products : PC-hosted Sierra C cross-compiler, which included

68000 C compiler, assembler, linker.

Motorola, Literature Dist

This entry basically has a list of 68000 reference and

programming books.

Echidna

1101 W Stevens Ave, Suite 232

Santa Ana California 92707

Tel : (714) 545-2662

Fax : (714) 545-3705

Products : tUME - the Universal Map Editor. PC-Hosted tool to

edit tile-maps.

Solutions : Saves time in designing game maps and placing

level information. Downloads maps to Western Techs SegaDev

card.

Cross Products Limited

23 The Calls

Leeds LS2 7EH, England

Tel 0532 429814

Fax 0532 426163   (dial 011 44 532 426163 from USA)

Product : SNASM68K : PC-Hosted SEGA Genesis development

system, which includes a 68000 assembler, linker, debugger.

Parsons Engineering

imhurst Ave

Covina California 91724

Tel (818) 966 5538

Products : PC/Apple IIgs hosted Sluggo Genesis development

hardware.

Advantech

1333 E.9400 South, Suite 160

Sandy Utah, 84092

Tel : (801) 572-5410

Fax : (810) 572-5674

Products : blank SEGA Genesis cartridge PC boards, blank

EPROMS.

*****

Genesis Development System Frequently Asked Questions

Q) What is the Genesis Development System?

A) The GDS is a cross development system that lets you develop

games and other software to run on the Sega Genesis. The

system consists of a 4 inch by 8 inch printed circuit board

that plugs in the cartridge slot of the Genesis, and has a

cable going to the parallel port of the host computer. A

complete software set is included consisting of an

integrated editor/assembler, a stand alone assembler, a

symbolic debugger, a music and sound effect editor, a

utility to break standard 16 color IFF images into tiles to

be downloaded to the Sega, and other files.

Q) Does it come with any source files?

A) You get source to the music editor, source to a Z80 program

to run on the genesis to interpret the music files, source

to a demo program that plays music and has sprites bouncing

around behind the Sega logo, source to the download

utilities. Complete source is provided on communicating

between the host machine and the Genesis.

Q) How much memory does it have on the board?

A) It comes with 1.25 megabytes of memory. 1 meg is intended

for program development and .25 megs is intended for

variable storage for the debugger and scratch memory.

Q) Can you program CD games with it?

A) No, unless you were to write an emulator for the CD rom

that could send data over the normal cable from the host

machine.

Q) Does it include technical specifications for the Genesis?

A) Yes, it comes with descriptions of the memory map, graphics

registers, the Z80's functions, reading the joysticks,

sprites, scrolling, tiles, the DMA copier, the audio chips-

in short everything required to program the genesis. Also

provided is a working demo with source.

Q) Is this proprietary information stolen from Sega?

A) No, the information on the Genesis came from reverse

engineering it, completely independent of Sega. There are

no copyright violations in the purchase or sale of this

development system, nor is there anything illegal about any

use of it.

Q) How does it work?

A) The board has ROMs on it which contain the monitor and

debugger that runs on the Genesis. When the Genesis is

reset, the ROMs take over and wait for commands to come

from the host machine-commands like download bytes, send

bytes back, set up registers, define a symbol, or enter the

interactive debugger. When in the interactive debugger the

host machine acts as a terminal, and characters are sent

over the cable. Full 68000 debugging is possible, including

disassembly, single step, register view and modify,

breakpoints, expression evaluation, memory view, modify,

search and copy, symbol definition, memory compare.  There

is also a z80 disassembler built in.

Q) How does the editor work?

A) From within the Amiga version of the editor through single

keys you can assemble your file, move the cursor to errors,

enter the debugger on the Genesis (editor becomes

terminal), download code and data, download code only (for

faster development). The IBM PC version of the editor

provides none of these features. Instead they are invoked

from commands at the CLI.

Q) What about the assembler?

A) The assembler is a full macro 68000 assembler, and includes

the full 8080 and parts of the Z80 instruction set. It

generates symbol table information that can be downloaded

to the Genesis to allow for symbolic debugging. It is quite

fast-on a standard Amiga about 40,000 lines/minute, on a

68030 at 33 Mhz it runs at about 220,000 lines/minute. On a

66 Mhz DX2 the assembler is about 550,000 lines/minute.

Q) Can I program in C?

A) If you have a C compiler that can generate a binary, you

can download it to the Genesis. This system is geared

towards assembly language development, where it really

shines. In the future if there is sufficient demand a C

compiler will be included.

Q) How do I make artwork? Is there a sprite editor?

A) Originally I planned on including a simple graphic editor,

but I found that no one used it-instead they used Dpaint.

The best way is to use dpaint to create graphics in lo-res

16 color mode, then to use the utility I provide to convert

them to raw data to be sent to the Sega.  Animation can be

done within Dpaint. Unfortunately there is no editor for

manipulating giant figures made out of sprites, but in

principle it would be possible to do. So probably you will

use dpaint or some other painting program to create the

artwork.

A) Can you backup existing cartridges like the game backup

systems?

Q) No-this system is intended for game development. However if

you were able to obtain ROM images, they can be downloaded

just like original code, and they would run on the system,

keeping in mind the 1.25 megabyte memory limit to the

board.  When running a ROM under the system, you can tinker

with memory, modify gameplay, add lives, etc. You can do

all the normal debugging functions, like single step,

breakpoints, etc, to see how the program is working. Making

a cartridge reader requires some hardware expertise, but is

fairly straightforward.

Q) How fast are downloads?

A) Data is transferred a little over 30K/second unless the host

machine is old or slow. That rate is valid for an

accelerated Amiga or a 486, and probably a 386 as well.

There is also a 120K/second transfer capability but there

is no error checking in this mode. Future versions of the

board will have standard rates of 110K/second on a fast PC

and about 100K/second on a fast Amiga.

Q) What host machines is this available on?

A) Currently the complete system is available on the Amiga and

on the IBM PC.

Q) Why should I buy your system and not go through Sega?

A) If you can get Sega to listen to you and you can afford to

pay $$$$$ for their system, then there isn't much advantage

to buying from me. It's been my experience that Sega is

very hard to deal with, and unless you're a big company

they will ignore you. If you buy this system, you don't

need to sign away your future, you don't need to sign any

non-disclosures. You immediately have the ability to create

software for the Genesis. If you create a game, you can

then sell it outright or attempt to publish it yourself-

never having to pay Sega a dime, as they provided you with

nothing and you have no agreement with them. This system

gives you options. Moreover it may be a better system to

develop on that the one Sega provides, but I don't know for

sure as I've never seen theirs-I've just heard rumors.

Q) How long has this system been available?

A) It first became available in April 1991.

Q) How many have already been sold?

A) I've sold around 20 units so far, although I've done no

advertising other than leave a brochure at the 1991 Game

Developer's conference. I haven't really been serious about

marketing it in the last two years. Recently I've posted

some articles on internet and this has resulted in sales of

5 units to three separate groups.

Q) How much does it cost?

A) The cost is $1000.

Q) Will it be ported to other platforms?

A) There are no plans to port the software to any other

platforms.

Q) What about technical support? How about software upgrades?

A) You can send me email and I will answer questions and help

solve your problems. If I'm home you can call me.

Concerning software support: I want you to be happy with

this. If you want some program written, and it seems like a

good idea, chances are I'll do it for free and include it

with the system. Currently if the software seems sparse

it's because no one has complained so I guess I include

enough. There's no point in writing software if it isn't

going to be used. In any case software upgrades are free

unless it's some major upgrade like including a C compiler-

I don't believe in selling something for hundreds of

dollars then nickel and diming you on upgrades. EPROM

upgrades are free if you send back your originals.

Q) Do we need to pay you any royalty on games developed with

the system?

A) No. Once you bought the system you can do anything you want

with it-except copy it and sell it yourself...

Q) Are there future systems in the works? Possibly with more

memory?

A) Eventually a new version of the circuit board will be

finished. This will offer 18 megabits of RAM standard for

the same price as the current board.  Some time after that

there will likely be a special 32 meg option. The 32

meg version will cost about $1150-$1200.

Q) If I buy one of the boards and a newer version of the

hardware becomes available later, can I upgrade?

A) Yes. To upgrade to a future version of the hardware you

would send back the old board along with enough money to

pay for manufacture of the new board. To determine the cost

of manufacture, figure a base price of $30 to cover the

printed circuit board and sockets and connectors, plus $15

for the inexpensive logic chips, plus the following amounts

for memory:

8 megabits -> 16 megabits  = $100 for memory upgrade

8 megabits -> 32 megabits  = $200 for memory upgrade

16 megabits-> 32 megabits  = $100 for memory upgrade (you

re-use old chips)

Note that at this time the 16 and 32 megabit versions are

not completed. The current version of the board uses

obsolete RAM chips that cannot be used on the future

versions. The new 16 and 32 megabit versions will use

smaller, more dense, cheaper RAM chips which can be moved

from one board to another.

Q) How can I contact you?

A) Send me mail, email or call.

818-584-0357

INTERNET: dash@netcom.com

David Ashley

395 Sierra Madre Villa

Pasadena, CA 91107

USA

*****

The person you want to talk to at Sega of America is Stuart

Kosoy, at (415)802-4407.

*****

=============================================================

4.0 Code samples. The following code was graciously given to

me by David Ashley, inventor of the GDS. We all owe him a big

Thanks!

Here's sample code for reading the joystick:

------------

;jsinit must be called to set up the joystick, otherwise

;reading back will have errors. This need only be called

;once.

jsinit:     moveq #$40,d0

move.b      d0,$a10009

move.b      d0,$a1000b

move.b      d0,$a1000d

rts

;The following two routines can be used for reading from the

;joystick ports.

;porta and portb return a byte in d0. 0 bits mean button is

;not down, 1 mean button is down.

;The order is:

;76543210

;SACBRLDU

;meaning Start, A, C, B, Right, Left, Down, Up

porta:      move.b      #$40,$a10003

nop

nop

move.b      $a10003,d1

andi.b      #$3f,d1

move.b      #$00,$a10003

nop

nop

move.b      $a10003,d0

andi.b      #$30,d0

lsl.b #2,d0

or.b  d1,d0

not.b d0

rts

portb:      move.b      #$40,$a10005

nop

nop

move.b      $a10005,d1

andi.b      #$3f,d1

move.b      #$00,$a10005

nop

nop

move.b      $a10005,d0

andi.b      #$30,d0

lsl.b #2,d0

or.b  d1,d0

not.b d0

rts

-----------------------

Graphics code is another matter. Since I sell a development

system, and one of the features is I provide technical info

on the Genesis hardware, it would sort of be a conflict of

interest for me to give things away :^).

-------------------

=============================================================

5.0 Language Switch, from Rec.Games.Video FAQ

How to make a language switch (Genesis/MD).

On a Genesis/MD, there are jumpers labelled JP1, JP2, JP4,

and JP3.  The Genesis has a capacitor on JP1 and a trace on

JP2; the Mega Drive has a capacitor on JP2 and a trace on

JP1.  The bottom ends of JP1 and JP2 are connected together.

So if you cut the trace and the top end of the capacitor, and

install a DPDT switch between them which reconnects them

either unchanged or swapped left to right, you have a

language switch. You'll need some wire, a soldering iron,

solder, and a DPDT switch.

Some machines have an open circuit instead of the capacitor.

Also, I've been told that even if there is a capacitor, you

can throw it out and leave an open circuit. Either way, the

switch is a lot simpler, requiring a SPDT switch and less

wire and solder.

Several people have told me that you could just cut both JP1

and JP2 and put a SPST switch on JP1. This is even simpler,

but I'm not sure it really works, as opposed to putting your

machine in an intermediate state that only sort-of works.

The redesigned Genesis 2 machines don't appear to have either

the capacitor or circuit. Nobody yet knows how to make the

language switch for one, though there are language switch

cartridges you can buy to act as one.

IF YOUR MACHINE HAS NO CAPACITOR (or if you want to cross

your fingers and throw away your capacitor) and is not a

Genesis 2:

Cut JP2.  The trace might be covered with paint and hard to

see.  (If you started with a Mega Drive, JP2 is open and you

have to cut JP1 instead.) If you aren't sure which end I mean

by "bottom", just check the back of the board to see which

end is connected together.

Original state of machine:             After cutting:

JP2 top     JP1 top              JP2 top     JP1 top

|           |                    |           |

|           |                    |           |

|

|

|

|

|           |                    |           |

\         /                      \         /

\_______/                        \_______/

bottom of both                   bottom of both

Add a SPDT switch which can be in one of two positions:

._ _ _ _ _ _ _ _ _ _ _ _

.                        .

.           . _ _ _ _ _ _ _ _ _ _

.           .                .    .

JP2 top     JP1 top               .    .

|           |                  .    .

|           |                 _________

| o   o |

| \     |

|  \    |

\___o___/

|           |                     .

\         /                      .

\_______/ - - - - - - - - - - - -

bottom of both

._ _ _ _ _ _ _ _ _ _ _ _

.                        .

.           . _ _ _ _ _ _ _ _ _ _

.           .                .    .

JP2 top     JP1 top               .    .

|           |                  .    .

|           |                 _________

| o   o |

|     / |

|    /  |

\___o___/

|           |                     .

\         /                      .

\_______/ - - - - - - - - - - - -

bottom of both

-------------------------------------------------------------

IF YOUR MACHINE DOES HAVE THE CAPACITOR:

Cut both sides.  (Note: if you started with a Japanese Mega

Drive the capacitor will be on the side labelled X instead)

Original state of machine:             After cutting:

JP2 top     JP1 top              JP2 top     JP1 top

|           |                    |           |

|           |                    |           |

|           |

|           |

|           |                    |

X |          ###                 X |          ###

|          ###                   |          ###

|          ###                   |          ###

\         /                      \         /

\_______/                        \_______/

bottom of both

add switch which can be in one of two positions:

JP2 top     JP1 top

(Connect 2 to 2

|           |

and 1 to 1)

|           |

2           1                   2  1  1  2

______________

_ _ _ _ _ _ _ _ _ _          | o  o  o  o |

|                   `         |  \  \      |

X |          ###`      `        |   \  \     |

|          ### `      `       \____o__o____/

|          ###  `      `           '  '

\         /     `      `- - - - -'  '

\_______/       ` _ _ _ _ _ _ _ _ '

bottom of both

JP2 top     JP1 top

(Connect 2 to 2

|           |

and 1 to 1)

|           |

2           1                   2  1  1  2

______________

_ _ _ _ _ _ _ _ _ _          | o  o  o  o |

|                   `         |      /  /  |

X |          ###`      `        |     /  /   |

|          ### `      `       \____o__o____/

|          ###  `      `           '  '

\         /     `      `- - - - -'  '

\_______/       ` _ _ _ _ _ _ _ _ '

bottom of both

50/60 hertz switch.

You can build a 50/60 hertz switch on a Genesis/Mega Drive

like a language switch, but using jumpers JP3 and JP4.  The

standard setting is 50 in PAL areas such as Europe, and 60 in

NTSC areas like the US and Japan. In the 60 hertz mode, the

game is faster and the screen taller; however, not all TVs

and monitors in Europe can display this mode.

Some American/Japanese games are protected to keep Europeans

from playing them; this protection checks the 50/60 hertz

setting.  You can usually get around it by installing the

switch and switching when starting the game, then switching

back afterwards.

Many European games are simple ports of American or Japanese

games and are not redesigned for 50 hertz, so work faster and

with "better" screen proportions if played at 60 hertz.

Genesis/Mega Drive dual version (language switch) list:

Bonanza Brothers: Game plays in Japanese.  (Maybe. There

seems to be more than one version floating around.)

Columns: Game plays in Japanese.

*Cyberball: Japanese version has a modem option.

Dragon's Fury: Works only with language set to English.

The original, Devil's Crush MD, works either way.

Dynamite Duke: Harder on the Mega Drive.

Elemental Master: Harder on the Mega Drive.

Fatal Labyrinth: Game plays in Japanese.

Fire Shark: Different title screen with Kanji.

Flicky: Characters have Japanese names and instructions

are in Japanese.

Forgotten Worlds: Game plays in Japanese.

Gaiares: only mentions the Japanese licensee on the title

screen, and has Japanese text; you can also select

Japanese text from the option screen.

Ghostbusters: Game plays in Japanese.

Ghouls and Ghosts: Different title screen with Kanji.  To

see it on a Genesis, select the last music and sound (26

and 56) from the options screen, then press lower left;

A, B, or C; and Start all at the same time.  (I never

tried this, but Gamepro magazine claimed it works.) The

game shows some other Japanese text, and when you die

during a boss you start out earlier.

Insector X: Title screen refers to company as Hot-B, not

Sage's Creation. The MD version shoots more slowly. The

ending text is still English.

*Marvel Land: The Japanese version says "for Mega Drive"

or "for Genesis" but the language stays Japanese. (What

does an English version do?)

Monaco GP: Game plays in Japanese (also an option on the

option screen).

Mystic Defender: This game is actually the anime-based

Kujaku-Ou (Peacock King) 2 game. In Japanese mode, the

opening text is replaced by a graphics screen (never

seen in the US version) with Japanese.  The levels have

names, the main character wears a white robe, the

lightning magic effect is different, and the character

is named Kujaku in the ending (which is still English).

Outrun: The attract mode lacks sound, the startup screen

says "push" (not "press") start button, and "(C) Sega

1986, 1991" is printed in reverse order. The default

options are KM/H and a different button selection (but

can still be changed on the option screen).

Quackshot: Game plays in Japanese.

Raiden Trad: The "licensed to Sega" line is absent on

both title screens, and the second title screen includes

only the Japanese part instead of the non-Japanese part

of the first one.

Revenge of Shinobi: Title changes to Super Shinobi;

credits show at the end.

Rolling Thunder II: The Japanese version only works on a

Japanese setting. The US version works either way (and

isn't bilingual).

Sonic the Hedgehog II: Tails is renamed to "Miles".

Streets of Rage: Title screen changes to Bare Knuckle,

and all text is in Japanese, including the introduction.

The clock resets when you encounter the bosses.

Streets of Rage II: Turns to Bare Knuckle II, and renames

Skate to Sammy --_if_ you change the setting sometime

after turning the machine on (to skip the lockout).

Thunder Force II: Title screen has "MD" on it, and

company name is "Tecnosoft".

Thunder Force III: company's name is spelled "Tecnosoft".

*Thunder Storm FX (CD): Turns to Cobra Command in US mode.

Truxton: Japanese title is Tatsujin.

Twin Hawk: Different title screen with Kanji.

*Wrestle War: The wrestler is blond on a Genesis and

black-haired on a MD.

* Information from testing a Japanese game

=============================================================

I hope to get more information and expand this document.

Please E-Mail me for a copy or to add information to this

list. Again My E-Mail address is

U6c16@Wvnvm.Wvnet.Edu

SnailMail:

Henry Rieke

Attn: Sega FAQ

161 Poplar Dr

Morgantown, Wv 26505-2340

A Very Special Thanks to:

David Ashley <dash@netcom.com>

Special Thanks to :

Scott Golby <sgolby@st.nepean.uws.edu.au>

Rick McTeague <sysrick@starbase.spd.louisville.edu>

Neal Howland <nhowland@matt.ksu.ksu.edu>

#