From PARADIS@htu.tu-graz.ac.at Fri Mar 25 08:41:08 1994

The Bloody SPC-700

-------------------

A try to stumble into the inner secret of a nasty chip.

By Antitrack exclusively for the FAMIDEV development group.

Chapter 1:

----------

FACTS

* The SPC 700 is a very stupid sound chip with about the worst

handling

that you have seen in your lifetime.

* This chip is a co processor. He has a quite large instruction set

(contrary to the Amiga's COPPER, who has a very small one) and 64KB

RAM

memory, of which you can use atleast 32KB.  (or so)

* All program and data that is supposed to be run by this chip must

be'

moved to the SPC's own ram with a small loop that pokes each byte of

your SPC assembler program and (e.g. sample-)data into four memory

locations : $2140 - $2143. They are your only chance to communicate

with

the SPC.

* These four memory locations have different meanings for read and

write;

if you read (LDA) $2140, you get the data from memory loc. 00f4 (or

so)

of the sound chip.

* On power-on, the SPC 700 jumps (much like the main processor) to a

very

small ROM area that resides from $ffc0 to $ffff inside the SPC.

(This chip REALLY follows the black box principle, eh...) This

program

at $ffc0 is waiting to get the data in the right format on his

input ports

at $00f4/5/6/7 , which are $2140/1/2/3 from the 65c816's (e.g.

your's )

point of view.

* Your main program will therefore have to follow the SPC's

conditions and

poke all the program and data for the SPC into 2140/1/2/3 in a

special

order.

* When transmission is completed, you will also have transmitted the

start

address of your SPC code, and the SPC will start to execute your

program

there.

--------------------QUESTIONS.

Q: How do I move my program and data to the SPC then, what format do

I have

to use?

A: First, your SPC data/code has to be moved from ROM to the extra

RAM at

e.g. $7f0000 . Dont ask me why it has to be in RAM, probably it doesnt

but all the existing routines that send data to the SPC do something

like

that.

Your data/code has to be in groups which I will call "chunks". A

valid chunk

looks like that:

first word: number of bytes to transmit to SPC           -+

sec. word : start address where to move data to the SPC   |  one chunk

byte 4-???? : your data/code                             -+

You can have as many chunks as you want to , but the last chunk must

be like

that:

first word : $0000

second word: Start address of your code.

Q: So if you are right, this means: After I transmitted all my code

and

data, and my own SPC code takes over the control, I might encounter

problems

if my SPC program has to communicate with the outer world (the

65c816).

What if the main program wants to change sounds? What if a background

melody

shall always play on two voices, and extra two voices will be used for

sound effects whenever the player sprite e.g. picks up an object?

A: That is sure a point. Your own code will have to look at memory

locations

$00f4/00f5/00f6/00f7 , because they are the only accessible from

outside

at $2140/1/2/3. The easiest way would be: As soon as any of $f4-$f7

change,

jump into the Boot ROM at $ffc0 (?) so the SPC is executing his

receive

routine again. Then you *probably* can send another SPC chunk with new

sound and code to the SPC....

Q: This only helps if a complete new tune is to be played, this

doesnt help

if a melody using two voices shall still remain....

A: Thats true. The best approach is to send own command bytes to the

SPC and

your SPC code has to check out $f4-$f7 constantly and react to it.....

A command byte like $00 could mean: sound off,

$01           : play tune 1

.

.

.

$0f          : play tune $0f

$10           : play jingle (fx) 01

.

.

.

$ff           : jump to $ffc0 (??) the receive

ROM routine

Q: is there another approach?

A: Yes there is. As you probably know, all important addresses of the

SPC 700 reside inside its own RAM's zeropage:

Address         / register          / usage

0000            Volume left

0001            Volume right

0002            Pitch low

0003            Pitch high          (The total 14 bits of pitch

height)

0004            SRCN                Designates source number from 0-

255

0005            ADSR 1

0006            ADSR 2

0007            GAIN            Envelope can be freely designated by

your code

0008            ENVX            Present val of envelope with DSP

rewrites

0009            VALX            Present wave height val

(and so on...)

Your approach would be to move only sample data there, and/or (lots

of) very

small chunks of data with a target address in the zeropage, and a

starting

address of e.g. $ffc0. The small chunks would access zeropage

addresses e.g.

for the volume etc and thus result in tones; if this is done every

frame

you might end up with a music player quite similar to the C64 styled

ones.

Q: So anyway, in what format exactly do I have to move data to the

SPC?

A: I have the following source code for you, but let me explain it a

bit

BEFORE you start to dig into it.

I've already mentioned the general "chunk" format. The loop does the

following:

- move ram destination address to $2142/3 (akku: 16 bit)

- move either #$00 or #$01 into 2141, this depends if you have more

than $0100

bytes of data for the SPC;

- first time (first chunk you transmit): move constant #$cc into 2140

- loop: poke each byte that you want to be transmitted into 2140

(word)

the higher 7-15 bits of your accu-word contain the number of bytes

already

moved (e.g. 00 on the start)

- cmp $2140 with this number of bytes already moved (lower 8 bits of

this

number only!) and wait if its not equal.

- until the loop is over.

- for the next chunk header this is repeated, but not #$cc is moved

into

2140 but "nn" (lobyte of number of bytes moved) +3 or +6 if it was

00 when

+3 was used.

EXAMPLE:

move #$0400 to 2142 /word access

move #$01 to 2141

move #$cc to 2140

move "gg00" to 2140 where "gg" is the first real code/data

byte for

the SPC

wait till 2140 is #$00

move hh01 to 2140 where "hh" is the second byte of code or

data for SPC

wait till 2140 is #$01

move ii02 to 2140 where "ii" is the 3rd byte of data for the

SPC....

wait till 2140 is #$02

lets say "ii" was the last byte. Now we add #$04 (3+carry) to

#$02

(#$02 being the number-1 of how many bytes we moved to the

SPC), we

will push it onto the stack), now :

fetch the next header , poke target RAM address into $2142

(word)

poke 00 or 01 into 2141 depending of how many bytes to send,

poke #$06 into 2140 (06 : number of bytes sent from last chunk-

1 + 3 )

I think I got this scheme pretty much right this time. Now, is PLEASE

someone

going to donate their home-brewed SPC dis/assemblers to me? Oh pretty

please,

I hate silent SNES's !  :)

Source code follows, reassembled from a PAN/Baseline demo "xmas wish

92/93":

----------------------------------------------------------------------

------

; entry to the code starts here

SEP #$30       ; x y a set to 8 bit length

LDA #$FF       ; ff into audio0w (write)

STA $2140

REP #$10       ; x,y: 16 bit length

LDX #$7FFF

l0DB5B      LDA $018000,X  ; move rom music data to ram at $7f0000

STA $7F0000,X

LDA $028000,X  ; move rom music data to ram at $7f0000

STA $7F8000,X

DEX

BPL l0DB5B

LDA #$80       ; screen on , probably not important at all

STA $2100

LDA #$00       ; 00fd/00fe/00ff point to the data that is

now

STA $00FD      ; in ram at $7f0000

LDA #$00

STA $00FE

LDA #$7F

STA $00FF

STZ $4200      ; disable nmi and timer h/v count

SEI            ; disable irq

JSR l0DBCD     ; unknown sub routine, labeled "RESTART"

by PAN/ATX

SEP #$30       ; all regs 8 bit

l0DB8B      LDA $2140      ; wait for reply from sound chip ?

BNE l0DB8B

LDA #$E0       ; audio3w ?

STA $2143

LDA #$FF       ; send data to sound chip ?

STA $2142      ; $ffe0 this could be an address within the

; sound chip ROM between $ffc0 and $ffff

in the

; ROM mask.......

LDA #$01       ; send data to sound chip ?

STA $2141

LDA #$01       ; send data to sound chip ?

STA $2140

l0DBA4      LDA $2140      ; wait for reply from sound chip ?

CMP #$01       ; what a fuck of a protocol .... :(

BNE l0DBA4

l0DBAB      LDA $2140      ; wait again for reply from soundchip ?

CMP #$55

BNE l0DBAB

LDA $0207       ; aha ... move $0207 to sound chip ?

STA $2141       ; probably sound number selector

LDA #$07

STA $2140       ; send data to sound chip

l0DBBD      LDA $2140       ; wait until sound chip accepted data?

CMP #$07

BNE l0DBBD

l0DBC4      LDA $2140       ; wait for reply ?

CMP #$55

BNE l0DBC4

CLI

RTS

l0DBCD      PHP            ; labeled "RESTART" by pan/ATX

JSR l0DBD8     ;

PLP

LDA #$00       ; 00 into audio0w

STA $2140

RTS

l0DBD8      PHP

REP #$30       ; a,x,y 16 bit regs

LDY #$0000     ; needed first time at lda [$fd],y :

pointer to ram

LDA #$BBAA

l0DBE1      CMP $2140      ; wait for sound chip $2140/2141 ?

BNE l0DBE1

SEP #$20      ; akku  8 bit

LDA #$CC

BRA l0DC12    ;  oh well, another mystery  :-)

; jump here if overflow is set  e.g. if more than $0100 data to move

l0DBEC      LDA [$FD],Y   ; get data from ram pointer

INY           ; the accumulator is about to get "xx00"

where

XBA           ;    /"xx" is the byte from [fd],y (first

data byte)

LDA #$00      ;    /and resides into bit 15-7 of accu,

and 00 is

BRA l0DBFF    ;    /#$00 (8bit number of bytes already

sent)

l0DBF4      XBA           ; accu is now "nn??" ?? is old data from

last loop

LDA [$FD],Y   ; accu is now "nnxx" with xx the newest

data byte

INY           ;                                   /for

the SPC!

XBA           ; accu is now "xxnn"

l0DBF9      CMP $2140     ; wait for sound chip to reply with "nn" !!

BNE l0DBF9

INC A         ; increment number of bytes that were

sent...

; accu is now "xxnn" with newest val for

nn:=nn+1

l0DBFF      REP #$20      ; akku 16 bit

STA $2140     ; poke "xxnn" to soundchip. xx is actual

data,

SEP #$20      ; akku 8 bit  ! nn is the 8-bit cutted

number of bytes

DEX                         ! which were already sent!!

BNE l0DBF4    ; as many times as xreg says...

l0DC09      CMP $2140     ; byte "nn" will be replied from the SPC if

data

BNE l0DC09    ; received correctly!

l0DC0E      ADC #$03      ; compare accu with #$fb  ADC WILL ADD #$04

COZ

; CARRY IS ALWAYS SET AFTER THE CMP!!!

ATTENTION!

BEQ l0DC0E    ; if accu was $fb then accu := $03 . (what

for?)

l0DC12      PHA           ; push value accu+$04 to stack (or

beginning: #$cc)

REP #$20      ; accu = 16 bit

LDA [$FD],Y   ; get ram data 2 bytes

INY           ; point to next word

INY

TAX           ; x:=a : number of bytes to transmit

LDA [$FD],Y   ; get ram data

INY

INY

STA $2142     ; audio2w  : possibly the dest. area in the

spc700

SEP #$20      ; accu 8 bit

CPX #$0100    ; set carry if first ram data was >= 0100

lda #$00      ;

ROL           ;

STA $2141     ; if ram data >= 0100, poke "1" into reg 1

otherw 0

ADC #$7F      ; SET OVERFLOW FLAG IF X>=$0100 !!!! (nice

trick!)

PLA

STA $2140     ; $cc in the first case , nn+4 on all later

cases

l0DC32      CMP $2140    ; wait for snd chip reply

BNE l0DC32

BVS l0DBEC   ; if there were more than $0100 data for the

spc's RAM

; move them where they R supposed to belong

to!

PLP

RTS

PLA

STA $2140    ; same shit, never been jumped into

l0DC3F      CMP $2140

BNE l0DC3F

BVS l0DBF9

PLP

RTS

; also lets look at 7f0000: the first few bytes at 7f0000 are:

7f0000: b7 0e 00 04 20 cd cf bd e8 00 5d af c8 f0 d0 fb 5d d5 00 01

d5 00 02

b7 0e should be number of bytes to transmit, 0400 the destination

inside the

spc....

at this point I really need an SPC dis/assembler.....  :(((

Okay well my first source was incompetent, sure thing. But I think I

could

solve a lot of questions meanwhile.