From jgordon@animator.slip.umd.edu Sat Mar  5 16:41:05 1994

hi....please look on the famidev site that accompanies this mailing list,

ftp to busop.cit.wayne.edu, then "cd pub/famidev/misc"

there you will find stool.exe, mtool.exe, ccx.exe, and several other

utilities for dealing with the smc and other console copier file formats...

i think that subjects such as "how do i split/join/convert this file" <do>

belong on this mailing list, but <<ONLY>> when relating to Super Famicon

Development....hence "famidev"....for game questions i would think that

the numberous pirate ftp's, irc, etc would be the place to ask....

personally i'm not as interested in copying games as i am in coding for

the super nintendo....

so on that note, here is all i know right <and> wrong about sprites...much

info in this document is correct, much info is wrong, much info is

confusing...to the point when this document was written, i had no snes

documentation....all info was gleaned from some disassembled demo code,

some source code, and a lot from a friend on irc <you know who you are :):)>

please don't send me corrections, please don't post to <me> and tell me

what is wrong....please <do> write a better doc using this info as a

catalyst.....(Yoshi this means you)....please <do> post corrections,

pointing out misinformation, but in a friendly manner, not an "I can read

my copy of the manual, and you are wrong buddy, i'm c00l and you're not d00d"

manner....

i'm not gonna correct/maintain this at all...i have sprite stuff,

<basically> working, and people have expressed interest in seeing stuff

to help them get started in sprites....

so enough said (probably more than enough :)

-jeremy

ps. feel free to ask any specific questions, i'll try to use my limited

sprite knowledge to answer them :)

--------------------cut me here----------------------------

SPRITE DOC --- JEREMY GORDON (JGORDON@ANIMATOR.SLIP.UMD.EDU)

------------------------------------------------------------

if you haven't already obtained "yoshi doc", get it and read it before you

read this doc....

Part I - the bitplane representation of a 16 color 8x8 pixel tile

-----------------------------------------------------------------

sprites are made of tiles, in particular 4-bitplane tiles, 4-bitplane

tiles, means that the tiles are made of 4-bit color values, so this means

that the tiles can have a maximum of 16 colors.

in many graphics formats, this type of data would be stored as follows

(assuming a 8 pixel by 8 pixel tile)

$1 $0 $2 $8 $2 $4 $5

$2 $0 $6 $1 $f $e $1

$a $2 $2 $2 $6 $7 $0

$1 $0 $2 $8 $2 $4 $5

$1 $0 $2 $8 $2 $4 $5

$1 $0 $2 $8 $2 $4 $5

$1 $0 $2 $8 $2 $4 $5

$1 $0 $2 $8 $2 $4 $5

where each hex number represents a color, so pixel (0,0) would be color

number "1", and pixel (4,2) would be color "6"....this is <not> the case

on the super nintendo....for reasons having to do with the implementation

of the graphics engine, the super nintendo represents its image data in

the "bitplane" format, assuming and 8 pixel by 8 pixel tile with 16

colors, this data would have four bitplanes (0-3) of monochrome, binary

image data:

0 0 0 0 0 0 0 0

0 0 0 0 0 0 0 0 1

0 0 0 0 0 0 0 0 1 2

0 0 0 0 0 0 0 0 1 2 3

0 0 0 0 0 0 0 0 1 2 3

0 0 0 0 0 0 0 0 1 2 3

0 0 0 0 0 0 0 0 1 2 3

0 0 0 0 0 0 0 0 1 2 3

1 1 1 1 1 1 1 1 2 3

2 2 2 2 2 2 2 2 3

3 3 3 3 3 3 3 3

four monochrome 8x8 pixel images stacked on top of each other. if you

wanted pixel (4,2) to have the color "6" you would have to put a "1" in

bitplane one, and a "1" in bitplane two, with the bitplane zero and three

having "0"'s.  this is because the binary representation of "6" is "0110".

ok, so it is obviously possible to store each monochrome bitplane in 8

bytes, each byte representing a row in the bitplane. so a single tile

takes up 32 bytes (8 bytes per row * 8 rows * 4 bitplanes) of memory.

Part II - the way tiles are stored in vram for sprite data

---------------------------------------------------------

in vram you store a tile 16 bits at a time as follows:

<-------2 bytes at a time------>

<--1 byte wide-><--1 byte wide->

[plane 0, row 0][plane 1, row 0]

[plane 0, row 1][plane 1, row 1]

[plane 0, row 2][plane 1, row 2]

..

..

..

[plane 0, row 7][plane 1, row 7]

[plane 2, row 0][plane 2, row 0]

[plane 2, row 1][plane 2, row 1]

[plane 2, row 2][plane 2, row 2]

..

..

..

[plane 2, row 7][plane 2, row 7]

the super nintendo can have two different sizes of sprite on the screen

at one time, you can choose from the following combinations:

[value]      [sprite size 0][sprite size 1]

000	      8x8 pixel	    16x16 pixel

001	      8x8 pixel	    32x32 pixel

010	      8x8 pixel	    64x64 pixel

011          16x16 pixel    32x32 pixel

100          16x16 pixel    64x64 pixel

101	     32x32 pixel    64x64 pixel

it is set in the upper three bits of address $2101....so to use 8x8 pixel

sprites, and 32x32 pixel sprites, you would load $2101 with the value

"001xxxxx" (where x is don't care)

the lower five bits of address $2101 are also very important, these bits

tell the super nintendo where in vram your sprites are located. the

lowest three bits are the "name base", and the fourth and fifth bits are the

"name". the "name" bits specify the upper 4k word of the sprite address,

and the "name base" specfies the offset. so if put you tile data in vram

$0000 you would set these bits all to zero.

suppose you want to have four 32 pixel by 32 pixel sprites; each would be

composed of 16 tiles, each tile is numbered, tiles $0-$3f

[sprite 0]

0  1  2  3

4  5  6  7

8  9  a  b

c  d  e  f

..

..

[sprite 3]

30 31 32 33

34 35 36 37

38 39 3a 3b

3c 3d 3e 3f

in vram, these tiles, $0 through tile $3f would be store in an interlaced

fashion as follows:

$0  $1  $2  $3  $10 $11 $12 $13 $20 $21 $22 $23 $30 $31 $32 $33

$4  $5  $6  $7  $14 $15 $16 $17 $24 $25 $26 $27 $34 $35 $36 $37

do you see the pattern? you must store the first row (four tiles) of each

sprite, and then the second row, then the third, etc....

if you were dealing with 8x8 sprites, you would have to store the first

row of 16 sprites, then the second row of the 16 sprites, etc....

if you were dealing with 16x16 sprites, you have to store the first row

of 8 sprites, then the second, then the third, etc...

with 64x64, yep, you guessed it, the first row of two sprites, then the

second row, etc....

Part III  - setting up the OAM table for your sprites

-----------------------------------------------------

for each sprite in the sprite table (maximum of 128 sprites, numbered 0-127)

you must have four bytes of information, the first byte is the low 8 bits of

the horizontal position of the sprite, the second byte is the vertical

position of the sprite, the third byte is the "address" of the sprite...

it is not the actual vram address, it is the tile number, that is to say,

the physical vram address of the sprite can be obtained by the following:

multiply the sprite "address" by 32 (because each tile is 32 bytes) and add it

to the starting vram address that you set in the $2101 register.

this tile number points to the first tile in the sprite...the snes expects the

rest of tiles to follow in the order described in the previous section...

the next byte containes the 9th bit of the tile "address" and a few attributes

bit 0   = 9th bit

bit 1-3 = palette number

bit 4,5 = playfield priority

bit 6   = horizontal flip

bit 7   = horizonal flip

the palette number is not a 4 bit number, so obviously, you can only choose

between 8 of the 16 palettes....if you set these bits to all 0, you will be

selecting the eigth palette (palette 7), if you set them to "001" it is the

ninth palette etc...it was trial and error, and some <serious> frustration

before i figured this one out :)

to set these bytes in the OAM table, you must first setup the OAM "address"

on the 16 bit register $2102 (and $2103). again this is not a real address,

you use the 7 lowest bits of this address to select which sprite you

would like to set the data for (sprite 0 to sprite 127)

then you can write the four bytes discussed above to the register $2104, its

like the color register, auto incrementing...

so....what are all the other bits for??? (the remaining 9) well, i know about

only two others, the highest bit (bit 15) is a sprite priority bit, it is

basically the bit you set to "turn on" the sprite, and keep it being redrawn

on the screen...its a little more than this, but thats all i know about its

behavior.  so when you setup your table, (and periodically throughout the

sprites' display lifecycle) you must set this bit high....(for any sprite being

displayed)

there is another small table (32 bytes) that you must load into the OAM, these

consist of two bits for every sprite, one of the bits being the 9th horizontal

position bit, and the other is the size select bit (remember we can pick from

two different size sprites on the screen at once) the first bit is the

most significant horizontal location, and the second is the size toggle

bit...

to write this table the OAM, you must set the 9th bit of the "address" in $2103

to one....then write the bytes, again it is an auto incrementing register

make sure that you have enabled the sprites to be on a particular plane (see

the yoshi doc, on $212C....and make sure that you have set your palette

correctly (remember sprite palette 0 is the 8th palette!!!)

if you have any other questions i'm reachable at jgordon@animator.slip.umd.edu

i check my e-mail several times a day....feel free....and i'm on the famidev

list, and irc with the nick "jgordon"....i'm basically jeremy gordon

everywhere....

good luck....

-jeremy gordon